3GPP TSG-RAN WG4 # 106bis-e	R4-2304918

Electronic Meeting, April 17 – April 26, 2023

Agenda item:	6.8.7.1

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN UE demodulation requirements

Document for:	Discussion

- Introduction

In RAN4#106 meeting, the WF for IoT-NTN UE demodulation requirements was agreed [1]. In this contribution, we provide our views to the previously agreed TS36.102 skeleton and present simulation results based on the simulation assumptions in [1].

- Discussion

TS36.102 skeleton for IoT-NTN UE demodulation

In RAN4#106, the TS36.102 skeleton for IoT-NTN UE demodulation was agreed in [1] and only Annex A to Annex D are introduced. It is noted that the NTN-TDL channel models apply for both NR-NTN and IoT-NTN. However, the parameters used to define requirements are different for NR-NTN and IoT-NTN. Therefore, we propose to add “Annex E: propagation conditions” in TS36.102.

| Clause number | Clause name |
| --- | --- |
| 8 | Demodulation Performance requirements |
| 8.1 | General |
| 8.2 | Demodulation performance requirements for UE category M1 |
| 8.2.1 | FDD and half-duplex FDD |
| 8.2.1.1 | PDSCH demodulation requirements |
| 8.3 | Demodulation performance requirements for UE category NB1 and NB2 |
| 8.3.1 | Half-duplex FDD |
| 8.3.1.1 | NPDSCH demodulation requirements |
| 8.3.1.1.1 | Minimum Requirements for Standalone |
| Annex A (normative): | Measurement channels |
| Annex B (informative) | Change history |
| Annex C | Downlink physical channels |
| Annex D | Environmental conditions |
| Annex E | Propagation conditions |

Proposal 1: Add “Annex E: propagation conditions” in TS36.102.

Simulation results for NB-IoT NPDSCH requirements

For NTN NB-IoT NPDSCH requirements, RAN4 agreed to consider the legacy TS36.101 NB-IoT standalone assumptions/settings but with some modifications, e.g., use NTN propagation channel and modify PDSCH repetition for Test 2 from 256 to 128. We present simulation results in below figures for the agreed NTN propagation channels, NTN-TDLC5-200 and NTN-TDLA100-10. Also, we provide ideal and impairment SNR to achieve 70% of maximum throughput in Table 1.

| Figure 1: Simulation result for Test 1 | Figure 2: Simulation result for Test 2 |
| --- | --- |

Table 1. Ideal and impairment SNR achieving 70% of maximum throughput

| Test number | CBW | Carrier type | MCS | NPDSCH repetition | Propagation condition | Antenna configuration | Ideal SNR(dB)@ 70% of max TP | Impairment SNR(dB)@ 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 200kHz | Anchor | QPSK 1/2 (Reuse R.NB6.FDD) | 32 | NTN-TDLC5-200 | 1x1 | -7.2 | -5.2 |
| 2 | FDD 200kHz | Non-anchor | QPSK 1/3 (Reuse R.NB6-1.FDD) | 128 | NTN-TDLA100-10 | 1x1 | -12.9 | -10.9 |

- Conclusion

In this contribution, we propose to add “Annex E: propagation conditions” in TS36.102 and present simulation results based on the simulation assumptions in [1].

Proposal 1: Add “Annex E: propagation conditions” in TS36.102.

- Reference

- R4-2302932, “WF on IoT-NTN UE demodulation and CQI reporting requirements”, MediaTek