3GPP TSG-RAN WG4 Meeting # 106-bis-e	R4-2305162

Online, April 17 – April 26, 2023

Agenda Item: 5.8.3.2

Source: MediaTek Inc.

Title: Discussion on L1 measurements for multi-Rx UEs

Document for: Discussion

# Introduction

According to WF [1] and discussion summary [2], RAN4 had some discussion but most issues were discussed without conclusion yet. In this meeting, this WI is divided into six agenda items to be discussed: (1) general issues, (2) L1 measurement, (3) RLM/BFD/CBD, (4) scheduling/measurement restriction, (5) dual TCI state switching and (6) receive timing difference. The discussion in this paper focus on the “L1 measurement”, e.g., L1-RSRP/L1-SINR measurement.

# Discussion

The following discussion will cover N factor, P factor and open issue for L1 measurements.

- N factor

- P factor

- L1-SINR

## 2.1 N factor

Before we discuss whether N factor can be reduced or not, we would like to share our understanding on UE measurement behavior first. In the following, the legacy R15/R16 UE is discussed first, and then R18 multi-Rx UE is further studied.

R15/R16 UE

For a R15/R16 UE, it is assumed that more than one antenna module can be equipped but only one antenna module is active at a time. To meet the measurement delay requirement (N=8), UE may or may not use L3 measurement result to down-select antenna module first. So, there are two possible designs as below.

- Design 1: use L3 measurement to down select antenna modules

- Design 2: not use L3 measurement to down select antenna modules

For design 1, as step 1 in Figure 1, UE takes previous L3 measurement results into account. Therefore, in step 2, UE can further perform L1 measurement only on the antenna module which is down selected by L3 measurement. In this case, the number of fine beams on each antenna module can be up to 8 to meet the L1 measurement delay requirement (N=8).

Figure 1. one possible UE design: performing L1 measurement based on previous L3 measurement info

For design 2, as Figure 2, UE does not take previous L3 measurement result into account. So, UE uses fine beam to perform L1 measurement on two antenna modules sequentially. In this case, the number of fine beams on each antenna module is 4 and total number of beams is 8. The reason why 4 fine beams is assumed on each antenna module is that UE should complete L1 measurement on both two antenna modules sequentially within a limited time (N=8).

Figure 2. one possible UE design: performing L1 measurement without previous L3 measurement info

R18 UE

In R18, two active antenna modules at a time are allowed. So, we would like to further study what is the difference for the design 1 and design 2 in this R18 WI.

For design 1, as step 1 in Figure 3, UE takes previous L3 measurement results into account. In step 2, the key point is the number of beams on each antenna module is still 8 if UE wants to maintain the same spherical coverage on each antenna module. Therefore, to our understanding, for design 1 UE, N should still be 8. Otherwise, the measurement delay is insufficient for UE to finish beam sweeping.

Figure 3. one possible UE design (two antenna modules can be activated at a time): performing L1 measurement based on previous L3 measurement info

For design 2, as Figure 4, UE does not take previous L3 measurement results into account. UE may activate up to two antenna modules at a time for L1 measurement. In this case, UE can perform L1 measurement on both antenna modules at a time. Therefore, N can be reduced.

Figure 4. one possible UE design (two antenna modules can be activated at a time): performing L1 measurement without previous L3 measurement info

So, based on above analysis, N cannot be reduced for all UEs because it depends on UE implementation.

Observation 1: N cannot be reduced for all UEs because it depends on UE implementation.

Proposal 1: Reducing the value of N in L1 measurement (L1-RSRP/BFD/CBD/RLM) delay requirement should be up to UE’s capability.

## 2.2 P factor

In the last meeting, L1 and L3 measurement cannot be performed simultaneously based on below agreement in RAN4 #106 WF [1] . So, the Psharing_factor cannot be enhanced.

<Agreement >: Issue 1-1-6: Simultaneous L3 measurements and L1 measurements - Simultaneous L3 and L1 measurements is not supported in this release for multi-Rx. - Note: The agreement has no impact on scheduling restriction discussion for L3 measurements. Feasibility and benefit of scheduling restriction relaxation for L3 measurements is FFS.

In the WID [3], there is no consideration for high mobility. So, we do not see the need and gain why UE is required to perform L3 measurement more frequently. Besides, in order to achieve simultaneous L3 and L1 measurement, joint consideration between L3 and L1 measurement in UE implementation is needed. It will hugely increase UE complexity because UE needs to control different measurements on different antenna modules at a time. As a result, the following proposal is suggested.

Proposal 2: In L1 measurement (L1-RSRP/BFD/CBD/RLM) delay requirement, P factor should not be enhanced due to simultaneous L3 and L1 measurement is not supported in R18.

## 2.3 L1-SINR

In last meeting, some companies discuss L1-SINR handling as below issue [1].

Issue 1-9: L1-SINR Handling - Proposals - Option 1: Handle L1-SINR in the same say as L1-RSRP(same requirements framework) - Option 2: Handle L1-SINR separately - Recommended WF - Option 1 If Option 2 is preferred, please provide more details on how to handle L1-SINR

RAN4 agreed that Rel-17 group-based reporting is used as a prerequisite to define requirement for simultaneous reception. And the details could refer to the signaling structure [5] in TS 38.331 and definition [6] in TS 38.306 as below.

mTRP-GroupBasedL1-RSRP-r17 SEQUENCE { maxNumBeamGroups-r17 INTEGER (1..4), maxNumRS-WithinSlot-r17 ENUMERATED {n2,n3,n4,n8,n16,n32,n64}, maxNumRS-AcrossSlot-r17 ENUMERATED {n8, n16, n32, n64, n128} } Definitions for parameters; Per; M; FDD TDD DIFF; FR1-FR2 DIFF. mTRP-GroupBasedL1-RSRP-r17 Indicates the support of group based L1-RSRP reporting enhancements. This feature also includes following parameters: - maxNumBeamGroups-r17 indicates the maximum number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets. - maxNumRS-WithinSlot-r17 indicates the maximum number of SSB and CSIRS resources for measurement in both CMR sets within a slot across all CCs. - maxNumRS-AcrossSlot-r17 indicates the maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs. maxNumRS-WithinSlot-r17 and maxNumRS-AcrossSlot-r17 are also counted in maxTotalResourcesForOneFreqRange-r16 and maxTotalResourcesForAcrossFreqRanges-r16; Band; No; N/A; N/A.

From above, we could observe Rel-17 group-based reporting is only for L1-RSRP, and L1-SINR is not supported, so the following proposal is suggested.

Proposal 3: Not discuss this issue because L1-SINR is not supported by Rel-17 group-based reporting.

# Summary

In this paper, the discussion of R18 multi-Rx chains WI is provided. We have the following proposal:

Observation 1: N cannot be reduced for all UEs because it depends on UE implementation.

Proposal 1: Reducing the value of N in L1 measurement (L1-RSRP/BFD/CBD/RLM) delay requirement should be up to UE’s capability.

Proposal 2: In L1 measurement (L1-RSRP/BFD/CBD/RLM) delay requirement, P factor should not be enhanced due to simultaneous L3 and L1 measurement is not supported in R18.

Proposal 3: Not discuss this issue because L1-SINR is not supported by Rel-17 group-based reporting.

# Reference

- R4-2303317, WF on NR FR2 multi-Rx chain DL reception – Part 2, Vivo, RAN4 #106

- R4-2302768, Topic summary for [106][210] FR2_multiRx_part2, Qualcomm, RAN#106

- RP-223527, Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Incorporated, RAN #98e

- R4-2303302, WF on NR FR2 multi-Rx chain DL reception – Part 1, Vivo, RAN4 #106

- NR Radio Resource Control (RRC); Protocol specification (3GPP TS 38.331 version 17.2.0)

- NR User Equipment (UE) radio access capabilities (3GPP TS 38.306 version 17.2.0)