3GPP TSG-RAN WG4 Meeting #106bis-e                                                        R4-2305204

Online, April 17th – 26th, 2023

Source:	Samsung

Title:	Discussion on implementation feasibility study of SBFD: FR2 BS Aspects

Agenda item:			5.20.2.2.2

Document for:	Discussion

# 1. Introduction

Rel-18 Study Item is approved on Study on evolution of NR duplex operation with the target to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation. According to latest SID in [1], in this RAN1 led SI tasks for RAN4 scope are explicitly stated as below:

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In RAN4#104-bis-e, WF on SBFD feasibility study and RF impacts from BS aspects was agreed [4], which includes the further WF based upon the agreement on August meeting [2][3]. In RAN4#105, further agreements and way forwards have been achieved and captured in [6] and reply LS to RAN1 for interference modelling and subband configuration has been approved in [8].  In RAN4#106, WFs on SBFD BS feasibility study on the self-interference and co-channel inter-sector interference are approved in [9] and [10] respectively, and one reply LS to RAN1 on interference modelling  has been approved in [11]. Accordingly, in this contribution, we would like to further provide our views on the feasibility and RF impact of SBFD from FR2 BS aspects.

# 2 SBFD Self-Interference Modeling

Based upon the RAN4 discussion over RAN4#104-e and RAN4#104-bis-e, the RSIC capability is broken down into four aspects: (1) spatial isolation; (2) frequency isolation; (3) beam nulling/isolation and (4) digital IC. And based upon the inputs from companies, the ranges for values of (1)-(4) are summarized in table 1 of reply LS [2]; however, the detailed ranges are the supersets of results provided from source companies which require further feasibility analysis. During RAN4#105, a more detailed RSIC analysis framework has been approved [6], and further agreements are achieved in the approved WF [9] and response LS to RAN1 [11].

## 2.1 Further Discussion on RSIC Capability for FR1 BS

Based on our hardware PoC and further analysis of the component RSIC capability, the FR1 RSIC budget calculation is further provided in Table 1 which is based on the agreed RSIC analysis framework.

Observation 1: Samsung’s input for RSIC budget calculation for FR2-1 BS is provided in Table-1.

Table-1: FR2-1 RSIC budget calculation Summary

| FR2-1 |  |  |  | Samsung |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |
|  |  | Frequency isolation techniques used |  | Without DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 87 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -95 dBm Note: provided by ①-②-③-⑨ dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | Not applicable |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | No impact |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -67 dBm Note: provided by ①-③-④-⑤dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 24 dBc |
|  |  | Frequency isolation techniques |  | Filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -161dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -91 dBm Note: provided by ①-③-④-⑤-⑥dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 10 dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 134.5 dBc |
| Noise floor ⑩dBm |  |  |  | -83 dBm/100MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -89 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 119 dBc |
| SBFD configuration |  |  |  | DU (100MHz-100MHz) |
| Guardband assumption (if exist) |  |  |  | 5PRB |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |
| Others |  |  |  |  |

Proposal 1: According to SIC budget calculation in Table-1, it’s feasible to ensure 1dB de-sensitivity based on achievable spatial isolation, frequency isolation, beam nulling and digital IC applied, for FR2-1 BS.

# 3 Co-channel Inter-Subband gNB-gNB CLI Modeling

## 3.1 Existing RAN4 Agreement

Based on the approved WF [6] as below, there are two key issues to be further discussed for co-channel co-site inter-sector gNB-gNB CLI evaluation: (1) the analysis framework, and (2) the achievable antenna isolation in the case of co-site inter-sector gNB-gNB CLI. Based the further discussion in RAN4#106, agreement on the range of isolation and analysis framework has been approved in WF [10]. We have summarized the existing agreement in our FR1 counterpart paper.

## 3.2 Further discussion on FR2-1

### 3.2.1 Discussion on analysis framework

In our accompanying paper for FR1 BS feasibility study, for co-channel co-site inter-sector inter-gNB CLI, we propose that company can report XdB desense (relative to RX noise floor) in additional to the self-interference in the implementation feasibility study, which contains the interference either from both co-site neighboring sectors or from one single sector. We see no reason the same proposal can’t be applicable for FR2.

Proposal 2: For co-channel co-site inter-sector CLI, company can report XdB desense (or YdB relative to RX noise floor) in additional to the self-interference in the implementation feasibility study, for which company can report it contains the interference from single or both co-site neighboring sectors. (Note: The same as the proposal for FR1)

- There is no necessity to define a criterion in terms of desense on co-channel co-site inter-sector CLI for implementation feasibility study.

### 3.2.2 Co-channel co-site inter-sector gNB-gNB CLI

By using the detailed analysis framework provided in our paper [12], we provided the analysis for co-channel co-site inter-sector gNB-gNB CLI

Observation 2: Samsung’s input for co-channel co-site inter-sector CLI for FR2 BS is provided in Table-2, in which the interference from one co-site sector can be suppressed to the level much lower than noise floor.

Table 2: Analysis framework for co-channel co-site inter-sector gNB-gNB CLI

| FR2-1 |  |  |  | Samsung |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |
|  |  | Frequency isolation techniques used |  | Without DPD |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | 108 dBc |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Based on 88dB for typical spatial isolation, and additional 20dB contributed by different beamforming directions of sectors |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -106dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | N/A |
|  |  | RF IC techniques and other tech. (before LNA) |  | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -78dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 24 dBc |
|  |  | Frequency isolation techniques |  | Filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -194dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -102 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | Not used in this case |
| Overall interference suppression capability dB (Note 1) |  |  |  | 130.5 dBc |
| Noise floor ⑩dBm |  |  |  | -83 dBm/100MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  |  |
| Required RSIC budget (①-⑪dBc) |  |  |  |  |
| SBFD configuration |  |  |  | DU (100MHz-100MHz) |
| Guardband assumption (if exist) |  |  |  | 5PRB |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

### 3.2.3 Antennal Isolation for Co-channel Co-site Inter-sector gNB-gNB CLI

Similar to the analysis contained in the contribution for FR1 BS feasibility, the following observations can be made:

Observation 3: Considering the distinct beamforming directions for different sectors, RAN4 can further study the additional spatial isolation value contributed from the suppression given by beamforming sidelobe, e.g., whether or not [10]dB is feasible for FR2 BS implementation. (Note: The same as the proposal for FR1)

Observation 4: Digital IC is still technically feasible to cancel the residual co-channel co-site inter-sector interference. (Note: The same as the proposal for FR1)

# 4 Adjacent-channel gNB-gNB CLI Modeling

based on the analysis in our accompanying paper on the interference modelling and reply LS under AI 5.20.2, we conclude the following proposal. In short, we expect the spatial isolation shall be larger for adjacent-channel CLI compared with co-channel case, which can be translated to 20-30dB higher spatial isolation to be added to the spatial isolation for co-channel inter-sector CLI.

Proposal 3: For the spatial isolation of adjacent-channel inter-sector CLI, the following values have been proposed for macro BS in RAN4:

    - FR2: 100-123dB with 113dB being typical value.

# 5. Conclusion

In this contribution, we provided our viewpoints on the feasibility and RF impact of SBFD from BS aspects, accordingly the following observations and proposals are obtained:

RSIC Analysis Framework:

Observation 1: Samsung’s input for RSIC budget calculation for FR2-1 BS is provided in Table-1.

Table-1: FR2-1 RSIC budget calculation Summary

| FR2-1 |  |  |  | Samsung |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |
|  |  | Frequency isolation techniques used |  | Without DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 87 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -95 dBm Note: provided by ①-②-③-⑨ dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | Not applicable |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | No impact |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -67 dBm Note: provided by ①-③-④-⑤dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 24 dBc |
|  |  | Frequency isolation techniques |  | Filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -161dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -91 dBm Note: provided by ①-③-④-⑤-⑥dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 10 dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 134.5 dBc |
| Noise floor ⑩dBm |  |  |  | -83 dBm/100MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -89 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 119 dBc |
| SBFD configuration |  |  |  | DU (100MHz-100MHz) |
| Guardband assumption (if exist) |  |  |  | 5PRB |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |
| Others |  |  |  |  |

Proposal 1: According to SIC budget calculation in Table-1, it’s feasible to ensure 1dB de-sensitivity based on achievable spatial isolation, frequency isolation, beam nulling and digital IC applied, for FR2-1 BS.

Co-channel Inter-Subband gNB-gNB CLI Modeling:

Proposal 2: For co-channel co-site inter-sector CLI, company can report XdB desense (or YdB relative to RX noise floor) in additional to the self-interference in the implementation feasibility study, for which company can report it contains the interference from single or both co-site neighboring sectors. (Note: The same as the proposal for FR1)

- There is no necessity to define a criterion in terms of desense on co-channel co-site inter-sector CLI for implementation feasibility study.

- Observation 2: Samsung’s input for co-channel co-site inter-sector CLI for FR2 BS is provided in Table-2, in which the interference from one co-site sector can be suppressed to the level much lower than noise floor.

- Table 2: Analysis framework for co-channel co-site inter-sector gNB-gNB CLI

| FR2-1 |  |  |  | Samsung |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |
|  |  | Frequency isolation techniques used |  | Without DPD |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαSI-spatial = ③ dBc |  | 108 dBc |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Based on 88dB for typical spatial isolation, and additional 20dB contributed by different beamforming directions of sectors |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -106dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | N/A |
|  |  | RF IC techniques and other tech. (before LNA) |  | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -78dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 24 dBc |
|  |  | Frequency isolation techniques |  | Filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -20dBm |
|  |  |  | Rx IM3 contribution (dBm) | -194dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -102 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | Not used in this case |
| Overall interference suppression capability dB (Note 1) |  |  |  | 130.5 dBc |
| Noise floor ⑩dBm |  |  |  | -83 dBm/100MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  |  |
| Required RSIC budget (①-⑪dBc) |  |  |  |  |
| SBFD configuration |  |  |  | DU (100MHz-100MHz) |
| Guardband assumption (if exist) |  |  |  | 5PRB |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |
| Others |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |

Observation 3: Considering the distinct beamforming directions for different sectors, RAN4 can further study the additional spatial isolation value contributed from the suppression given by beamforming sidelobe, e.g., whether or not [10]dB is feasible for FR2 BS implementation. (Note: The same as the proposal for FR1)

Observation 4: Digital IC is still technically feasible to cancel the residual co-channel co-site inter-sector interference. (Note: The same as the proposal for FR1)

Adjacent-channel gNB-gNB CLI Modeling:

Proposal 3: For the spatial isolation of adjacent-channel inter-sector CLI, the following values have been proposed for macro BS in RAN4:

    - FR2: 100-123dB with 113dB being typical value.

# 6. Reference

[1] RP-222110, “Revised SID: Study on evolution of NR duplex operation”, CMCC, RAN#97.

[2] R4-2214376, “Reply LS on interference modelling for duplex evolution”, RAN4, Samsung, CMCC, Qualcomm, RAN4#104-e.

[3] R4-2214377, “WF on feasibility study from RF perspective”, Samsung, RAN4#104-e.

[4] R4-2217464, “WF on SBFD feasibility study and RF impact: BS aspect”, Samsung, RAN4#104-Bis-e.

[5] R4-2217513, “WF on SBFD feasibility study and RF impact: UE aspect”, Qualcomm, RAN4#104-Bis-e.

[6] R4-2220244, “WF for the feasibility from BS aspect”, Samsung, RAN4#105.

[7] R4-2220245, “WF for the feasibility from UE aspect”, Qualcomm, RAN4#105.

[8] R4-2220243, “LS response to RAN1 for interference modelling and Sub-band configuration”, RAN4, Samsung, RAN4#105.

[9] R4-2302883, “WF for Self-interference analysis from BS aspect for SBFD operation”, Samsung, RAN4#106.

[10] R4-2302922, “WF for Co-channel inter-sector interference analysis from BS aspect for SBFD operation”, Ericsson, RAN4#106.

[11] R4-2302885, “Reply LS on interference modelling”, RAN4, CATT, RAN4#106.

[12] R4-2302968, “Discussion on feasibility and RF impact of SBFD: BS Aspects”, Samsung, RAN4#106