3GPP TSG-RAN WG4 Meeting #106bis-e                                                        R4-2305212

Online, April 17th – 26th, 2023

Source:	Samsung

Title:	Discussion on UL timing adjustment solutions for FR2 HST

Agenda item:			5.13.4.3

Document for:	Discussion

# 1. Introduction

In RAN#95e meeting, the Rel-18 RAN4-led work item on enhanced NR support for high speed train scenario in FR2 has been approved as [1]. As identified in Rel-17 FR2 HST work item, there is a large propagation delay difference from neighbouring RRHs to UE. For example in the uni-directional RRH deployment, it is identified that the signal propagation delay difference can be as much as 2.5us, which is much larger than CP length with 120kHz subcarrier spacing. In Rel-17 work item, the larger autonomous timing adjust step Tq is specified for FR2 HST UE, and the RAN4-based solutions for the uplink timing issue are focused, while other solutions, including involving other RAN working groups, are not fully studied. As the objectives of Rel-18 enhanced NR support for FR2 HST, to study on reference tunnel deployment scenario for FR2 HST is expected to be included in this WI, which is provided in WID [2] as follows:

- Specify UL timing adjustment solution, including explicit NW signalling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2].

In last three RAN4 meeting (RAN4#104Bis-e, RAN4#105 and RAN4#106), the WFs on this issue have been approved as [4] [5] and [6], with the following relevant WF and agreements obtained:

## WF obtained in R4-2217254 ## General view on UL timing adjustment solution Way Forward: - RAN4 continue to discuss UL timing adjustment solution, including explicit NW signalling assistance in Rel-18, based upon Option 3 and 4 captured in WF R4-2120416. - Both RRC and MAC-CE based solutions are FFS. - FFS whether to embedded spatial similarity (QCL-like relation for beams across RRHs) information by ordering the SSB index (to RRH mapping) signaled in RRC solution ## WF obtained in R4-2220396 ## General view on UL timing adjustment solution Way forward: The issue requires further study: - Clarify the improvement or the performance gain compared with existing R17 solution. - Details on network signalling (based upon Option 3 and 4 captured in WF R4-2120416): - Option 1: MAC-CE based solutions to inform UE of the TCI state switch is across RRH - Option 2: Embedded spatial similarity (QCL-like relation for beams across RRHs) information by ordering the SSB index (to RRH mapping) signalled in RRC solution - Other options are not precluded - Proponents of network signaling are encouraged to provide design details in next RAN4 meeting. ## UL transmit timing adjustment at UL beam switch Way forward: The issue requires further study: - Whether requirements on UL transmit timing adjustment at UL spatial relation switch are needed - Whether PC6 UE, capable of simultaneous multi-panel reception, can support UL transmit timing to two RRHs ## WF obtained in R4-2303240 Way forwards on UL timing adjustment solution are provided as: Topic-1: Network signalling assistance Agreement - Do not consider RRC based solutions to inform UE of the TCI state switch across RRH. Way forward: - FFS: Introduce MAC-CE based solutions to inform UE of the TCI state switch across RRH - FFS: RAN4 informs RAN2 that the following additional information shall be added for TCI State Indication for UE-specific PDCCH MAC CE, for Rel-18 FR2 HST scenario: - [1] bit indication for whether or not UE shall follow the one shot large UL timing adjustment specified in clause 7.1.2.3 in TS38.133 when switching to the TCI State ID contained in the MAC-CE Topic-2: UL transmit timing adjustment at UL beam switch Way forward: - FFS: Whether requirements on UL transmit timing adjustment at UL spatial relation switch are needed. - FFS: Whether PC6 UE, capable of simultaneous multi-panel reception, can support UL transmit timing to two RRHs Topic-3: Impact of large propagation delay jump on timeAlignemntTimer Way forward: - FFS: Potential impacts of large jump in propagation delay on UE MAC timeAlignmentTimer - Option 1: In HST FR2 scenarios, UE MAC timeAlignmentTimer should be stopped after inter-RRH TCI state switch - Option 2: Not to introduce timeAlignemntTimer enhancements at UL timing adjustment. - Other options are not precluded.

In this contribution, we would like to further share our views on UL timing adjustment solutions for FR2 HST.

# 2. Discussion

When UE is moving across neighboring RRHs, to maintain the wireless link, the network needs to switch the working RRH from one to another, where the excessive propagation delay difference could be experienced at the UE side, as a particular technical problem not experienced by non-HST systems.  To solve this problem, we introduced a new one-shot large timing adjustment mechanism in this Release-17 work (as an optional feature), which is capable of enabling UE to perform the cross-RRH switch with much reduced data package losses.

Figure.1 illustration of one-shot timing adjustment mechanism for FR2 HST UE.

## 2.1 Rel-18 UL timing adjustment solution

As tasked by WID for Rel-18, RAN4 shall specify UL timing adjustment solution, including explicit NW signalling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE. Based on agreement in RAN4#106 as below, MAC-CE based soltuion shall be focused.

Agreement - Do not consider RRC based solutions to inform UE of the TCI state switch across RRH.

Furthermore, based on Based on the contents for Option 3 and Option 4 captured in WF R4-2120416, the difference of two options comes from whether or not UE large DL timing change detection is needed in the specified solution. From my understanding, the benefit of introducing this Rel-18 enhancement comes from that UE can be informed by NW the upcoming beam switching is inter-RRH or intra-RRH one, while no need for UE to judge the condition by using DL timing information. Therefore, option 4 could be more preferable since it is more aligned with the objective in Rel-18 WID.

Proposal 1: For the MAC-CE based network signaling assistance, the condition that UE measurement on DL timing difference is larger than certain threshold (as specified in Rel-17) shall not be included as the applicable condition for Rel-18 UL timing adjustment solution.

## 2.2 Network signaling for UL timing adjustment solution

As agreed in [6], RAN4 shall further study ”introducing MAC-CE based solutions to inform UE of the TCI state switch across RRH”.

Based on our understanding, for MAC-CE based solution, UE only need to know whether or not the UL timing adjustment solution shall be applied when UE is switching to the new TCI state, and no need to understand the detailed spatial relationship between the new and old TCI state. RAN4 shall further discuss the solution based on the MAC-CE for PDCCH TCI indication, i.e., TCI State Indication for UE-specific PDCCH MAC CE. The reason comes from (1) in practical FR2 deployment, MAC-CE based PDCCH TCI switching is widely used while TCI state for the UE-specific PDSCH will follow the PDCCH TCI; and (2) compared with MAC-CE TCI states activation +DCI based PDSCH TCI state indication, the MAC-CE based PDCCH TCI indication is more suitable for HST scenario which is more demanding on the procedure delay.

Proposal 2: For the MAC-CE based network signaling assistance, RAN4 shall discuss the necessary new information which needed to be added to existing MAC-CE for PDCCH state indication, i.e., TCI State Indication for UE-specific PDCCH MAC CE.

By considering the current MAC CE design, there is no reserved bit left for TCI State Indication for UE-specific PDCCH MAC CE:

Figure 6.1.3.15-1: TCI State Indication for UE-specific PDCCH MAC CE (from TS38.321)

Therefore, RAN2 shall be informed and involved as early as possible for the new MAC-CE design.

Proposal 3: RAN4 shall inform RAN2 that the following additional information shall be added for TCI State Indication for UE-specific PDCCH MAC CE, for Rel-18 FR2 HST scenario:

- One bit indication for whether or not UE shall follow the new UL timing solution when switching to the TCI State ID in MAC-CE.

# 3. Conclusion

In this contribution, we provided our viewpoints on our views on UL timing adjustment solutions for FR2 HST, accordingly the following observations are obtained:

Proposal 1: For the MAC-CE based network signaling assistance, the condition that UE measurement on DL timing difference is larger than certain threshold (as specified in Rel-17) shall not be included as the applicable condition for Rel-18 UL timing adjustment solution.

Proposal 2: For the MAC-CE based network signaling assistance, RAN4 shall discuss the necessary new information which needed to be added to existing MAC-CE for PDCCH state indication, i.e., TCI State Indication for UE-specific PDCCH MAC CE.

Proposal 3: RAN4 shall inform RAN2 that the following additional information shall be added for TCI State Indication for UE-specific PDCCH MAC CE, for Rel-18 FR2 HST scenario:

- One bit indication for whether or not UE shall follow the new UL timing solution when switching to the TCI State ID in MAC-CE.

# 5. Reference

[1] RP-220985, “New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[2]	RP-222272, “Revised WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[3]	TR 38.854 v0.4.0, NR support for high speed train scenario in frequency range 2 (FR2).

[4] R4-2217254, “WF on tunnel deployment and UL timing adjustment for FR2 HST enhancement”, Samsung.

[5] R4-2220396, “WF on NR FR2 HST Tunnel deployment and UL timing adjustment”, Nokia.

[6] R4-2303240, “WF on NR FR2 HST UL timing adjustment solutions”, Qualcomm.