3GPP TSG-RAN WG4 Meeting # 106bis-e									R4-2305257

Online, April 17 – April 26, 2023

Title: 	Discussion on CA_n28-n105

Source: 	Xiaomi

Agenda item:	5.4.2.2

Document for:	Approval

- Introduction

In RAN#99 meeting, a new SI on sub-1GHz band combinations including 600MHz band was approved [1]. RAN4 just completed the similar SI on 700/800/900MHz band combinations and a follow-up WI was also approved targeting at specifying RF requirements. This contribution will provide our views on CA_n28-n105.

- Discussion

The SI on sub-1GHz is similar as the completed Rel-18 SI on 700/800/900MHz. The difference is that 600MHz band is added in the scope. In the SI on 700/800/900MHz, RAN4 achieved a good progress on low-band band combinations with the conclusions captured in TR 38.872. From methodology perspective, the similar procedure and method can be reused as much as possible for this SI.

For sub-1GHz, two RF architectures can be captured for further study, i.e., two antenna architecture and three antenna architecture. The antenna number is also assumed as the total number of antennas to support Main UL/DL and diversity DL for all bands. The RF requirements for either two antenna or three antenna architecture should be evaluated in the SI phase. RAN4 is expected to specify UE RF requirements in the WI phase.

Proposal 1: Study both 2 antenna architecture and 3 antenna architecture for CA_n28-n105 in SI phase and specify RF requirements in the follow-up WI phase.

UE architecture

Figure 1 demonstrates potential UE architecture to support CA_n28-n105 with 3 antenna. The topology for 3 antenna architecture is listed as below:

1) n28UL+n28DL duplexer on antenna 1

	2) n105UL+n105DL duplexer on antenna 2

3) n28DL+n105DL duplexer on antenna 3

Figure 1: Potential UE architecture to support CA_n28-n105 with 3 antenna

Figure 2a demonstrates potential UE architecture to support CA_n28-n105 with 2 antenna (quadplexer and duplexer). The topology for 2 antenna architecture a is listed as below.

-	1) n28UL+n28DL+n105UL+n105DL quadplexer on antenna 1

-	2) n28DL+n105DL duplexer on antenna 2

Figure 2b demonstrates potential UE architecture to support CA_n28-n105 with 2 antenna (two triplexers). The topology for 2 antenna architecture b is listed as below.

-	1) n28UL+n28DL+n105DL triplexer on antenna 1

-	2) n105UL+n105DL+n28DL triplexer on antenna 2

Figure 2a: Potential UE architecture to support CA_n28-n105 with 2 antenna

Figure 2b: Potential UE architecture to support CA_n28-n105 with 2 antenna

Proposal 2: Three antenna architecture in Figure 1 and two antenna architecture in Figure 2a and 2b can be used for further study.

UE coexistence

Table 1: Operating bands for CA_n28-n105

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n105 | 663 MHz – 703 MHz | 612 MHz – 652 MHz | FDD |

Table 2: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 | 3515 | 3740 |
| n105 | 663 | 703 | 612 | 652 | 1326 | 1406 | 1989 | 2109 | 2652 | 2812 | 3315 | 3515 |

Table 3: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3112 | 3790 | 3815 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on the analysis above, there is no harmonic issue and harmonic mixing issue for CA_n28-n105.

Observation 1: There is no harmonic issue and harmonic mixing issue for CA_n28-n105.

Table 4: Band n28 and Band n105 UL IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 663 | 703 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 85 | 0 | 1366 | 1451 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 703 | 833 | 578 | 703 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2069 | 2199 | 2029 | 2154 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1406 | 1581 | 1241 | 1406 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2772 | 2947 | 2692 | 2857 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 0 | 170 | 2732 | 2902 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2109 | 1904 | 2329 | 2109 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 703 | 493 | 918 | 703 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3355 | 3560 | 3475 | 3695 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3395 | 3605 | 3435 | 3650 |

Based on the analysis above, there is no IMD issue for CA_n28-n105

Observation 2: There is no IMD issue for CA_n28-n105.

- Conclusion

This contribution present our initial views on CA_n28-n105. The following proposals are derived:

Proposal 1: Study both 2 antenna architecture and 3 antenna architecture for CA_n28-n105 in SI phase and specify RF requirements in the follow-up WI phase.

Proposal 2: Three antenna architecture in Figure 1 and two antenna architecture in Figure 2a and 2b can be used for further study.

Observation 1: There is no harmonic issue and harmonic mixing issue for CA_n28-n105.

Observation 2: There is no IMD issue for CA_n28-n105.

- Reference

- RP-230789, Study on enhancement for sub-1GHz NR band combinations, RAN#99