3GPP TSG-RAN WG4 Meeting #106-bis-e                              R4-2305393

Online, April 17 – April 26, 2023

Source: 	Murata Manufacturing Co Ltd.

Title: 	PC2 FDD bands RSD

Agenda Item:	4.22.2 High power UE for FR1 for FDD single band(s) with PC2 	[HPUE_NR_FR1_FDD_R18]

Document for:	Approval

Introduction

In the RAN4#106 meeting, the Reference Sensitivity Degradation (RSD) of newly added PC2 FDD bands in the WID [1] has been discussed. However, the agreement was not achieved. Especially for wide channel bandwidth there are large differ in the proposals proposed by each company [2]. We re-evaluated the PA noise and review our estimation for RSD for Low bands. Additionally, we also considered the RSD of n2, n25. These results are shown in this contribution.

Discussion

Analysis conditions

The two kinds of FE architecture shown in the Figure 1 are analysis baseline. For 2Tx/2Rx architecture, the mutual influence between pTx and dTx shall be considered.

Figure 1. FE architecture for PC2 FDD

RxNoise measurement

PA calibration:

- For PC3：PA is calibrated to deliver - 30 dBc ACLR at 1dB MPR using full allocation 20MHz QPSK DFT-s-OFDM SCS15kHz.

- For PC2：PA is calibrated to deliver - 31 dBc ACLR at 1dB MPR using full allocation 20MHz QPSK DFT-s-OFDM SCS 15kHz.

Waveform:

- QPSK DFT-s-OFDM

- Image and Local Oscillator rejection is set to -28 dBc

- Counter-IM3 set to -60 dBc

- Counter-IM5 set to -70 dBc

UL configuration:

UL configuration is set according to [3].

Table 1 UL configuration for RxNoise measurement

|  |  | UL/DL BW, UL configuration |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS [kHz] | 5 [MHz] | 10 [MHz] | 15 [MHz] | 20 [MHz] | 25 [MHz] | 30 [MHz] | 35 [MHz] | 40 [MHz] | 45 [MHz] |
| n2 | 15 | (0,25) | (2,50) | (29,50) | (56,50) | (83,50) | (112,48) | (148,40) | (176,40) |  |
| n5 | 15 | (0,25) | (27,25) | (59,20) | (86,20) | (86,20)* |  |  |  |  |
| n8 | 15 | (0,25) | (27,25) | (59,20) | (86,20) |  |  | (86,20)* |  |  |
| n25 | 15 | (0,25) | (2,50) | (29,50) | (56,50) | (83,50) | (112,48) | (148,40) | (176,40) | (176,40)* |
| n26 | 15 | (0,25) | (27,25) | (54,25) | (81,25) | (81,25)* | (81,25)* |  |  |  |
| n28 | 15 | (0,25) | (27,25) | (54,25) | (81,25) | (108,25) | (135,25) |  |  |  |
| n71 | 15 | (0,25) | (0,25) | (0,20) | (0,20) | (0,20)* | (0,20)* | (0,20)* |  |  |
| *：DL only, UL BW is 20MHz for n5, n8, n26, n71. UL BW is 40MHz for n25. |  |  |  |  |  |  |  |  |  |  |  |

RSD analysis

Assumptions

Table 2 basic assumptions

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx power | 26 dBm | PC2 PA * 1Tx, at ANT1 |
|  | 23 dBm | PC3 PA * 2Tx, at ANT1&ANT2 |
| Post PA loss | 4 dB |  |
| ANT_ Iso. | 10 dB | ANT1⇔ANT2 |
| MRC | ANT1&ANT2 | Correlated |

RSD calculation results

The RSD calculation results are shown in Table 3 and Table 4.

To account the reverse IMD between pTx and dTx, additional 1dB RxNoise contribution is added for 2Tx/2Rx architecture.

Table 3 Reference sensitivity degradation for 1Tx PC2

| Operating Band | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) |
| n2 | 1.3 | 0.9 | 0.8 | 0.7 | 0.6 | 2.8 | 2.6 | 3.2 | - | - |
| n5 | 0.7 | 0.6 | 0.4 | 2.9 | 3.0 | - | - | - | - | - |
| n8 | 0.4 | 0.3 | 0.3 | 2.4 | - | - | 2.9 | - | - | - |
| n25 | 1.0 | 0.7 | 0.6 | 0.6 | 0.5 | 2.8 | 2.6 | 3.2 | 3.5 | - |
| n26 | 1.0 | 0.5 | 0.5 | 2.9 | 3.1 | 3.1 | - | - | - | - |
| n28 | 0.9 | 0.7 | 0.9 | 1.3 | 2.2 | 3.2 | - | - | - | - |
| n71 | 0.7 | 0.8 | 0.6 | 2.1 | 2.1 | 2.0 | 2.8 | - | - | - |

Table 4 Reference sensitivity degradation for 2Tx PC2

| Operating Band | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) |
| n2 | 0.9 | 0.7 | 0.7 | 0.9 | 0.9 | 4.8 | 5.9 | 6.1 | - | - |
| n5 | 1.7 | 1.9 | 2.1 | 5.8 | 6.3 | - | - | - | - | - |
| n8 | 1.8 | 1.8 | 2.2 | 5.6 | - | - | 6.8 | - | - | - |
| n25 | 0.7 | 0.6 | 0.7 | 0.9 | 0.8 | 4.8 | 5.9 | 6.1 | 6.9 | - |
| n26 | 1.4 | 1.7 | 2.2 | 5.8 | 6.3 | 6.6 | - | - | - | - |
| n28 | 1.7 | 2.0 | 2.7 | 3.8 | 6.0 | 9.2 | - | - | - | - |
| n71 | 1.6 | 1.8 | 2.0 | 5.6 | 6.0 | 6.5 | 6.8 | - | - | - |

Proposal:

The RSD of n2, n5, n8, n25, n26, n28, n71 for PC2 FDD are proposed highlighted as Table 3 and Table 4.

Conclusion

In this contribution, we presented the analysis condition and RSD calculation results for PC2 FDD band.

Proposal:

The RSD of n2, n5, n8, n25, n26, n28, n71 for PC2 FDD are proposed highlighted as Table 3 and Table 4.

Table 3 Reference sensitivity degradation for 1Tx PC2

| Operating Band | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) |
| n2 | 1.3 | 0.9 | 0.8 | 0.7 | 0.6 | 2.8 | 2.6 | 3.2 | - | - |
| n5 | 0.7 | 0.6 | 0.4 | 2.9 | 3.0 | - | - | - | - | - |
| n8 | 0.4 | 0.3 | 0.3 | 2.4 | - | - | 2.9 | - | - | - |
| n25 | 1.0 | 0.7 | 0.6 | 0.6 | 0.5 | 2.8 | 2.6 | 3.2 | 3.5 | - |
| n26 | 1.0 | 0.5 | 0.5 | 2.9 | 3.1 | 3.1 | - | - | - | - |
| n28 | 0.9 | 0.7 | 0.9 | 1.3 | 2.2 | 3.2 | - | - | - | - |
| n71 | 0.7 | 0.8 | 0.6 | 2.1 | 2.1 | 2.0 | 2.8 | - | - | - |

Table 4 Reference sensitivity degradation for 2Tx PC2

| Operating Band | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) |
| n2 | 0.9 | 0.7 | 0.7 | 0.9 | 0.9 | 4.8 | 5.9 | 6.1 | - | - |
| n5 | 1.7 | 1.9 | 2.1 | 5.8 | 6.3 | - | - | - | - | - |
| n8 | 1.8 | 1.8 | 2.2 | 5.6 | - | - | 6.8 | - | - | - |
| n25 | 0.7 | 0.6 | 0.7 | 0.9 | 0.8 | 4.8 | 5.9 | 6.1 | 6.9 | - |
| n26 | 1.4 | 1.7 | 2.2 | 5.8 | 6.3 | 6.6 | - | - | - | - |
| n28 | 1.7 | 2.0 | 2.7 | 3.8 | 6.0 | 9.2 | - | - | - | - |
| n71 | 1.6 | 1.8 | 2.0 | 5.6 | 6.0 | 6.5 | 6.8 | - | - | - |

References

[1] RP-222625 “New WID: High power UE for FR1 for FDD single band(s) with power class 2”, China Unicom, 3GPP TSG-RAN Meeting #97-e, Online, September 12th – 16th, 2022

[2] R4-2303458 “WF on FDD PC2 HPUE”, China Unicom, Skyworks, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, 27 February –03 March, 2023

[3] 3GPP TS 38.101-1 V17.7.0