3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305470

# Online, April 17 – April 26, 2023

Title: 	Simulation results on advanced receiver for MU-MIMO

Source: 	Huawei, HiSilicon

Agenda item:	5.16.1.2

Document for:	Discussion

Background

In RAN4 #106 meeting, a WF [1] was agreed and interested companies can bring simulation results based on initial simulation assumptions to study the performance gain of advanced receiver compared to baseline receiver. In this paper, we provide our related simulation results.

Simulation results

The simulation assumptions are summarized in Table 2-1:

Table 2-1: Simulation assumptions

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | 13 | Rank 1+1: QPSK Rank 2+2: QPSK and 64QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | Scrambling ID |  | Same scrambling ID for both UEs |  |
| MIMO configuration |  |  | For Rank 1+1: 2T2R Medium For Rank 2+2: 4T4R Low |  |
| Number of CDM groups without data |  |  | 1 for paired UE allocated in same CDM groups and 2 for paired UE allocated in different CDM groups |  |
| HARQ process number |  |  | 4 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - Option 1: Select the precoding matrix to ensure orthogonality with target UE - Option 2: Select the precoding matrix randomly ensuring the selected precoding matrix shall not be identical to the precoding matrix of target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLC300-100,TDLA30-10 |  |
| Receiver type |  |  | MMSE-IRC,E-MMSE-IRC, R-ML | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |

Rank 1+1

Normalized TP-SNR curve is captured in Figure 2-2

Figure 2-2: Normalized TP-SNR curve for Rank 1+1

Rank 2+2

Normalized TP-SNR curve is captured in Figure 2-3

Figure 2-3: Normalized TP-SNR curve for Rank 2+2

Summary of simulation results are captured in Table 2-2:

Table 2-2: Summary of simulation results

| Rank allocation | Propagation conditions | Serving UE:MCS13 (16QAM) |  | E-MMSE-IRC(Gain) | R-ML(Gain) |
| --- | --- | --- | --- | --- | --- |
| Rank 1+1 | TDLC300-100 | Interference UE: QPSK | Random PMI | 1.0 | 11.0 |
|  |  |  | Orthogonal | 0.8 | 9.2 |
| Rank 2+2 | TDLC300-100 | Interference UE: QPSK | Random PMI | 4.0 | 8.6 |
|  |  |  | Orthogonal | 1.2 | 3.8 |
|  |  | Interference UE: 64QAM | Random PMI | 4.0 | 4.9 |
|  |  |  | Orthogonal | 1.2 | 1.6 |
|  | TDLA30-10 | Interference UE: QPSK | Random PMI | 0.0 | 4.2 |
|  |  |  | Orthogonal | 0.0 | 2.8 |
|  |  | Interference UE: 64QAM | Random PMI | 0.0 | 0.5 |
|  |  |  | Orthogonal | 0.0 | 0.4 |

We can make following conclusions based on the observations from above simulation results:

- Observation 1: For R-ML receiver, compared to modulation order of serving UE, the modulation order of interference UE smaller, the performance gain is higher

- Observation 2: R-ML receiver has significant gain for low Rank allocation (Rank 1+1) with medium antenna correlation and high Rank allocation (Rank 2+2) with low antenna correlation.

- Observation 3: R-ML receiver performs better than E-MMSE-IRC for all cases

- Observation 4: E-MMSE-IRC has no performance gain over MMSE-IRC for TDLA30-10, even for TDLC300-100, the gain is limited.

- Observation 5: Compared to TDLA30-10,  R-ML has more performance gain over MMSE-IRC for TDLC300-100

- Observation 6: Random PMI increases the performance gain significantly compared to orthogonal PMI.

Based on following observations, we propose the following:

Proposal 1: RAN4 to focus on R-ML receiver with following conditions:

- TDLC300-100

- The modulation order of co-scheduled UE is no larger than serving UE

- Rank 1+1 with medium antenna correlation and Rank 2+2 with low antenna correlation

- Random PMI selection

Conclusion

In this paper we provide our simulation results for advanced receiver for MU-MIMO. The observations and proposals are:

- Observation 1: For R-ML receiver, compared to modulation order of serving UE, the modulation order of interference UE smaller, the performance gain is higher

- Observation 2: R-ML receiver has significant gain for low Rank allocation (Rank 1+1) with medium antenna correlation and high Rank allocation (Rank 2+2) with low antenna correlation.

- Observation 3: R-ML receiver performs better than E-MMSE-IRC for all cases

- Observation 4: E-MMSE-IRC has no performance gain over MMSE-IRC for TDLA30-10, even for TDLC300-100, the gain is limited.

- Observation 5: Compared to TDLA30-10,  R-ML has more performance gain over MMSE-IRC for TDLC300-100

- Observation 6: Random PMI increases the performance gain significantly compared to orthogonal PMI.

Proposal 1: RAN4 to focus on R-ML receiver with following conditions:

- TDLC300-100

- The modulation order of co-scheduled UE is no larger than serving UE

- Rank 1+1 with medium antenna correlation and Rank 2+2 with low antenna correlation

- Random PMI selection

Reference

- R4-2302929 WF on advanced receiver for MU-MIMO scenario. China Telecom.