3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305616

e-meeting, April 17 – April 26, 2023

Title:	On the RF requirement of multi DL reception

Source:	OPPO

Agenda item:	5.8.2.2

Document for:	Approval

- Introduction

The UE RF requirement of multi DL reception was discussed in the past several RAN4 meetings. Both the requirement concept and the 3D scan grids were made great progress in RAN4 #106 meeting.

This contribution shares our consideration on the UE RF requirement.

- Discussion

The UE RF requirement concept is agreed to use Option 3 as the baseline, the WF is captured as below [1].

The Option 3 is a kind of spherical coverage metric and the output on every measurement grid point is the state of UE functionality rather than EIS. And the qualified UE RF multi DL reception performance requires the coverage over the whole sphere exceeds M%. Besides the requirement concept, the measurement grid for multi DL reception is also decided in the last RAN4 meeting as below [1].

Based on the DL direction simulation of the additional test probe on the sphere, the optimized sphere sampling approach is utilizing the constant-step size grids, which is illustrated as below with 15-degree step size for example.

The test points with the constant-step size grids are not uniformly distributed on the sphere. So the same quantity of test points probably covers quite different sizes of the spherical surface. For example, five test points distributed around the equator region covers larger area than the same quantity of test points located near the poles region.

Therefore, the ratio of the qualified test points to all the test points based on the constant-step size grids can not correctly stand for the performance of spherical coverage. In principle, the performance of spherical coverage is evaluated by the ratio of the performance-qualified area to the whole spherical surface. To meeting this principle requirement, the test points should be scaled according to the θ angle of the test points. And the proposed weights for the test point are Sine or Clenshaw-Curtis Quanrature.

Proposal: For the constant-step size grids, when consider spherical coverage performance, the test points should be scaled according to the θ angle of the test points. And the proposed weights for the test point are Sine or Clenshaw-Curtis Quanrature.

- Conclusions

This paper presents our consideration on the UE RF requirement, i.e. spherical coverage performance with the constant-step size grids. And provide the following proposal.

Proposal: For the constant-step size grids, when consider spherical coverage performance, the test points should be scaled according to the θ angle of the test points. And the proposed weights for the test point are Sine or Clenshaw-Curtis Quanrature.

- References

[1] R4-2302923, WF for FR2 OTA test enhancement SI, Qualcomm Incorporated