3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305672

Electronic meeting, Apr 17 – 26, 2023

Agenda item:	5.23.3.4

Source: 	Qualcomm Incorporated

Title: 	Simulation results for 1Rx RedCap UEs without frequency hopping

Document for:	Information

# Introduction

RAN4 agreed on assumptions for a first round of simulations of PRS measurement accuracy for 1Rx RedCap UEs without frequency hopping [1]. This paper includes preliminary results for the following measurements:

- UE Rx-Tx time difference

- PRS-RSRP

- RSTD

# Simulation results for UE Rx-Tx time difference

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 | 118 | 121 | 124 |
|  |  |  |  |  |  | 2 | 118 | 118 | 121 |
|  | 52 |  | 128 | 4 | 1 | 1 | 57 | 58 | 63 |
|  |  |  |  |  |  | 2 | 57 | 57 | 59 |
|  | 104 |  | 64 | 4 | 1 | 1 | 28 | 31 | 32 |
|  |  |  |  |  |  | 2 | 28 | 31 | 31 |
|  | 24 | 30 | 128 | 4 | 4 | 1 | 56 | 58 | 62 |
|  |  |  |  |  |  | 2 | 56 | 58 | 60 |
|  | 48 |  | 64 | 4 | 1 | 1 | 28 | 30 | 34 |
|  |  |  |  |  |  | 2 | 28 | 30 | 32 |

Table 1: UE Rx-Tx simulation results for FR1 in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR2 | 24 | 60 | 64 | 4 | 4 | 1 | 29 | 30 | 31 |
|  |  |  |  |  |  | 2 | 29 | 30 | 30 |
|  | 64 |  | 32 | 4 | 1 | 1 | 14 | 15 | 16 |
|  |  |  |  |  |  | 2 | 14 | 15 | 15 |
|  | 132 |  | 16 | 4 | 1 |  |  |  |  |
|  | 32 | 120 | 32 | 4 | 4 | 1 | 15 | 15 | 15 |
|  |  |  |  |  |  | 2 | 15 | 15 | 15 |
|  | 64 |  | 16 | 4 | 1 | 1 | 8 | 8 | 8 |
|  |  |  |  |  |  | 2 | 8 | 8 | 8 |

Table 2: UE Rx-Tx simulation results for FR2 in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 | 187 | 189 | 735 |
|  |  |  |  |  |  | 2 | 162 | 163 | 167 |
|  | 52 |  | 128 | 4 | 1 | 1 | 143 | 149 | 828 |
|  |  |  |  |  |  | 2 | 106 | 106 | 118 |
|  | 104 |  | 64 | 4 | 1 | 1 | 104 | 103 | 134 |
|  |  |  |  |  |  | 2 | 64 | 66 | 68 |
|  | 24 | 30 | 128 | 4 | 4 | 1 | 149 | 159 | 482 |
|  |  |  |  |  |  | 2 | 104 | 105 | 107 |
|  | 48 |  | 64 | 4 | 1 | 1 | 117 | 121 | 628 |
|  |  |  |  |  |  | 2 | 68 | 68 | 74 |

Table 3: UE Rx-Tx simulation results for FR1 in TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [90%] (Tc) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR2 | 24 | 60 | 64 | 4 | 4 | 1 |  |  |  |
|  |  |  |  |  |  | 2 |  |  |  |
|  | 64 |  | 32 | 4 | 1 | 1 | 87 | 89 | 141 |
|  |  |  |  |  |  | 2 | 84 | 85 | 87 |
|  | 132 |  | 16 | 4 | 1 |  |  |  |  |
|  | 32 | 120 | 32 | 4 | 4 | 1 |  |  |  |
|  |  |  |  |  |  | 2 |  |  |  |
|  | 64 |  | 16 | 4 | 1 | 1 | 81 | 83 | 816 |
|  |  |  |  |  |  | 2 | 79 | 82 | 83 |

Table 4: UE Rx-Tx simulation results for FR2 in TDL-C.

# Simulation results for PRS-RSRP

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 | 0.6 | 0.7 | 1.6 |
|  |  |  |  |  |  | 2 | 0.5 | 0.6 | 1.2 |
|  | 52 |  | 128 | 4 | 1 | 1 | 0.6 | 0.9 | 2.2 |
|  |  |  |  |  |  | 2 | 0.5 | 0.6 | 1.6 |
|  | 104 |  | 64 | 4 | 1 | 1 | 0.5 | 0.7 | 1.8 |
|  |  |  |  |  |  | 2 | 0.3 | 0.5 | 1.2 |
|  | 24 | 30 | 128 | 4 | 4 | 1 | 0.5 | 0.8 | 2.3 |
|  |  |  |  |  |  | 2 | 0.4 | 0.5 | 1.7 |
|  | 48 |  | 64 | 4 | 1 | 1 | 0.7 | 0.9 | 2.3 |
|  |  |  |  |  |  | 2 | 0.5 | 0.7 | 1.6 |

Table 5: PRS-RSRP simulation results for FR1 in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR2 | 24 | 60 | 64 | 4 | 4 | 1 | 0.5 | 0.8 | 2.6 |
|  |  |  |  |  |  | 2 | 0.4 | 0.6 | 1.8 |
|  | 64 |  | 32 | 4 | 1 | 1 | 0.9 | 1.5 | 4.4 |
|  |  |  |  |  |  | 2 | 0.6 | 1.0 | 3.1 |
|  | 132 |  | 16 | 4 | 1 |  |  |  |  |
|  | 32 | 120 | 32 | 4 | 4 | 1 | 0.6 | 1.1 | 3.5 |
|  |  |  |  |  |  | 2 | 0.4 | 0.7 | 2.5 |
|  | 64 |  | 16 | 4 | 1 | 1 | 1.1 | 1.9 | 5.4 |
|  |  |  |  |  |  | 2 | 0.7 | 1.2 | 3.8 |

Table 6: PRS-RSRP simulation results for FR2 in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 | 6.5 | 6.4 | 5.7 |
|  |  |  |  |  |  | 2 | 0.5 | 0.7 | 1.7 |
|  | 52 |  | 128 | 4 | 1 | 1 | 5.2 | 4.9 | 4.4 |
|  |  |  |  |  |  | 2 | 0.6 | 0.8 | 2.2 |
|  | 104 |  | 64 | 4 | 1 | 1 | 4.2 | 3.9 | 3.7 |
|  |  |  |  |  |  | 2 | 0.4 | 0.6 | 1.7 |
|  | 24 | 30 | 128 | 4 | 4 | 1 | 5.5 | 5.1 | 4.9 |
|  |  |  |  |  |  | 2 | 0.5 | 0.8 | 2.4 |
|  | 48 |  | 64 | 4 | 1 | 1 | 4.7 | 4.5 | 4.2 |
|  |  |  |  |  |  | 2 | 0.6 | 0.8 | 2.1 |

Table 7: PRS-RSRP simulation results for FR1 in TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINR = -3 dB | SINR = -6 dB | SINR = -13 dB |
| FR2 | 24 | 60 | 64 | 4 | 4 | 1 |  |  |  |
|  |  |  |  |  |  | 2 |  |  |  |
|  | 64 |  | 32 | 4 | 1 | 1 | 2.5 | 2.7 | 5.5 |
|  |  |  |  |  |  | 2 | 0.7 | 1.2 | 3.8 |
|  | 132 |  | 16 | 4 | 1 |  |  |  |  |
|  | 32 | 120 | 32 | 4 | 4 | 1 |  |  |  |
|  |  |  |  |  |  | 2 |  |  |  |
|  | 64 |  | 16 | 4 | 1 | 1 | 2.5 | 2.9 | 6.5 |
|  |  |  |  |  |  | 2 | 0.9 | 1.5 | 4.6 |

Table 8: PRS-RSRP simulation results for FR2 in TDL-C.

# Simulation results for RSTD

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 | ±155 |
|  |  |  |  |  |  | 2 | ±146 |
|  | 52 |  | 128 | 4 | 1 | 1 | ±66 |
|  |  |  |  |  |  | 2 | ±66 |
|  | 104 |  | 64 | 4 | 1 | 1 | ±38 |
|  |  |  |  |  |  | 2 | ±38 |
|  | 24 | 30 | 128 | 4 | 4 | 1 | ±68 |
|  |  |  |  |  |  | 2 | ±68 |
|  | 48 |  | 64 | 4 | 1 | 1 | ±40 |
|  |  |  |  |  |  | 2 | ±39 |

Table 9: RSTD simulation results for FR1 in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR2 | 24 | 60 | 64 | 4 | 4 | 1 | ±38 |
|  |  |  |  |  |  | 2 | ±38 |
|  | 64 |  | 32 | 4 | 1 | 1 | ±17 |
|  |  |  |  |  |  | 2 | ±17 |
|  | 132 |  | 16 | 4 | 1 |  |  |
|  | 32 | 120 | 32 | 4 | 4 | 1 | ±18 |
|  |  |  |  |  |  | 2 | ±18 |
|  | 64 |  | 16 | 4 | 1 | 1 | ±9 |
|  |  |  |  |  |  | 2 | ±9 |

Table 10: RSTD simulation results for FR2 in AWGN.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR1 | 24 | 15 | 256 | 4 | 4 | 1 | ±366 |
|  |  |  |  |  |  | 2 | ±161 |
|  | 52 |  | 128 | 4 | 1 | 1 | ±345 |
|  |  |  |  |  |  | 2 | ±103 |
|  | 104 |  | 64 | 4 | 1 | 1 | ±94 |
|  |  |  |  |  |  | 2 | ±51 |
|  | 24 | 30 | 128 | 4 | 4 | 1 | ±227 |
|  |  |  |  |  |  | 2 | ±100 |
|  | 48 |  | 64 | 4 | 1 | 1 | ±196 |
|  |  |  |  |  |  | 2 | ±52 |

Table 11: RSTD simulation results for FR1 in TDL-A.

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Num Rx | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR2 | 24 | 60 | 64 | 4 | 4 | 1 |  |
|  |  |  |  |  |  | 2 |  |
|  | 64 |  | 32 | 4 | 1 | 1 | ±72 |
|  |  |  |  |  |  | 2 | ±62 |
|  | 132 |  | 16 | 4 | 1 | 1 |  |
|  |  |  |  |  |  | 2 |  |
|  | 32 | 120 | 32 | 4 | 4 | 1 |  |
|  |  |  |  |  |  | 2 |  |
|  | 64 |  | 16 | 4 | 1 | 1 | ±62 |
|  |  |  |  |  |  | 2 | ±56 |

Table 12: RSTD simulation results for FR2 in TDL-C.

# References

[1] R4-2303318, Simulation assumptions for 1Rx RedCap UE PRS measurements, RAN4#106