3GPP TSG-RAN WG4 Meeting #107 	R4-2307023

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on 8Rx general demodulation aspects

Agenda item:	8.5.3.1.1

Document for:	Discussion

Introduction

This document extends the discussion on the 8Rx UE demodulation and CSI requirements introduced in RAN4 #105 summarized in [1], discussions were continued at RAN4 #106 and RAN4 #106-bis-e where the way forward was summarized in [3] and [4].

Some of the issues regarding general items for 8Rx are detailed below, which is an extract from [3].

Issue 1-1: Whether to consider CA requirements - Start the discussion on CA scenario for 8Rx after RF session clearly agrees to define any CA band combinations for 8Rx. Issue 1-2 whether to define PDCCH requirements - Don't define PDCCH requirements Issue 1-3: Whether to define PDSCH requirements with CSI-RS overlapped with PDSCH - Don’t define PDSCH requirements with CSI-RS overlapped with PDSCH

This paper presents Nokia’s views on the open issues related to the 8Rx UE SDR demodulation.

Discussion

From RAN4#106-bis-e, the remaining open issues which require agreement are as follows:

Issue 1-4: Applicability rules for PDSCH Issue 1-5: Applicability rules for PDCCH Issue 1-6: Applicability rules for CSI test

Issue 1-4: Applicability rules for PDSCH

During RAN4#106-bis-e the following table were presented in the WF and summary as an option for applicability rule for PDSCH.

Cases; 2Rx test in section 5.2.2 in TS 38.101-4; 4Rx test in section 5.2.3 in TS 38.101-4; 8RX test; Tests skipped. 8Rx UE supporting both 2Rx and 4Rx band; ×; √; √; - Basic Rank2 tests: Test 2-1 and 2-2 in Table 5.2.3.1.1-4 and Table 5.2.3.2.1-4. - Basic Rank4 tests: Test 4-1 in Table 5.2.3.1.1-6 and Table 5.2.3.2.1-6.. 8Rx UE supporting 4Rx band but not supporting 2Rx band; ×; √; √; - Basic Rank2 tests: Test 2-1 and 2-2 in Table 5.2.3.1.1-4 and Table 5.2.3.2.1-4. - Basic Rank4 tests: Test 4-1 in Table 5.2.3.1.1-6 and Table 5.2.3.2.1-6.. 8Rx UE supporting 2Rx band but not supporting 4Rx band; √; ×; √; - Basic Rank2 tests: Test 2-1 and 2-2 in Table 5.2.2.1.1-4 and Table 5.2.2.2.1-4. 8Rx UE not supporting both 2Rx band and 4Rx band; ×; √; √; - Basic Rank2 tests: Test 2-1 and 2-2 in Table 5.2.3.1.1-4 and Table 5.2.3.2.1-4. - Basic Rank4 tests: Test 4-1 in Table 5.2.3.1.1-6 and Table 5.2.3.2.1-6. - Enhanced Receiver Type 1 test: Test 5-1 in Table 5.2.3.1.1-7 and Table 5.2.3.2.1-7 - MMSE-IRC with inter cell interference: All cases in section 5.2.3.1.15 and 5.2.3.2.16 - MMSE-IRC with intra cell inter user interference: All cases in section 5.2.3.1.16 and 5.2.3.2.17 - CRS-IM with scenario 1: All cases in section 5.2.3.1.17 and 5.2.3.2.18 - CRS-IM with scenario 2: All cases in section 5.2.3.1.18 and 5.2.3.2.19. Use PDSCH test applicability rules as follows: - Case1&Case2: For 8RX capable UEs support only 4Rx bands or both 2RX and 4RX bands, single carrier test cases specified in 5.2.3.1 and 5.2.3.2 with 4Rx except for Test 2-1 and 2-2 in Table 5.2.3.1.1-4 and Table 5.2.3.2.1-4 (Basic Rank 2 test) and Test 4-1 in Table 5.2.3.1.1-6 and Table 5.2.3.2.1-6 (Basic Rank 4 test) are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 4Rx should be applied. - Case3: For 8RX capable UEs support only 2Rx bands, single carrier test cases specified in 5.2.2.1 and 5.2.2.2 with 2Rx except for Test 2-1 and 2-2 in Table 5.2.2.1.1-4 and Table 5.2.2.2.1-4 (Basic Rank 2 test) are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied. - Case4: For 8RX capable UEs without support of any 4Rx and 2Rx bands, single carrier tests specified in 5.2.3.1 and 5.2.3.2 with 4Rx except for Test 2-1 and 2-2 in Table 5.2.3.1.1-4 and Table 5.2.3.2.1-4 (Basic Rank 2 test), Test 4-1 in Table 5.2.3.1.1-6 and Table 5.2.3.2.1-6 (Basic Rank 4 test), Test 5-1 in Table 5.2.3.1.1-7 and Table 5.2.3.2.1-7(Enhanced Receiver Type 1 test), cases in section 5.2.3.1.15 and 5.2.3.2.16 (MMSE-IRC with inter cell interference), cases in section 5.2.3.1.16 and 5.2.3.2.17(MMSE-IRC with intra cell inter user interference), cases in section 5.2.3.1.17 and 5.2.3.2.18(CRS-IM with scenario 1), cases in section 5.2.3.1.18 and 5.2.3.2.19 (CRS-IM with scenario 2) are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. The SNR requirements should be applied with 1.5 dB less than the number specified for 4Rx tests..

The above proposals broadly enable UEs that support bands other than 8Rx (notably 4Rx and 2Rx) to bypass the applicability rules for 8Rx regarding PDSCH and utilize the existing PDSCH applicability rules for 4Rx and 2Rx, as agreed in previous meetings on this work item we agree with this, however, propose slightly different wording.

We propose RAN4 shall define applicability rules for PDSCH based upon UE declared capability.

Issue 1-5: Applicability rules for PDCCH

During RAN4#106-bis-e the following table were presented in the WF and summary as an option for applicability rule for PDCCH.

Cases; 2Rx test in section 5.3.2 in TS 38.101-4; 4Rx test in section 5.3.3 in TS 38.101-4. 8Rx UE supporting both 2Rx and 4Rx band; ×; √. 8Rx UE supporting 4Rx band but not supporting 2Rx band; ×; √. 8Rx UE supporting 2Rx band but not supporting 4Rx band; √; ×. 8Rx UE not supporting both 2Rx band and 4Rx band; ×; √. Use PDCCH test applicability rules as follows: - Case1&Case2: For 8RX capable UEs support only 4Rx bands or both 2RX and 4RX bands, all single carrier test cases specified in 5.3.3.1 and 5.3.3.2 with 4Rx are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 4Rx should be applied. - Case3: For 8RX capable UEs support only 2Rx bands, all single carrier test cases specified in 5.3.2.1 and 5.3.2.2 with 2Rx are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Same requirements specified with 2Rx should be applied. - Case4: For 8RX capable UEs without support of any 4Rx and 2Rx bands, all single carrier test cases specified in 5.3.3.1 and 5.3.3.2 with 4Rx are tested on any of the 8Rx supported RF bands by duplicating the fading channel from each Tx antenna and add independent noise for each Rx antenna. Same requirements specified with 4Rx should be applied..

The above proposals broadly enable UEs that support bands other than 8Rx (notably 4Rx and 2Rx) to bypass the applicability rules for 8Rx regarding PDCCH and utilize the existing PDCCH applicability rules for 4Rx and 2Rx, as agreed in previous meetings on this work item we agree with this, however, propose slightly different wording.

This updated wording is included in the summary in section 2.5 of this contribution.

We propose RAN4 shall define applicability rules for PDCCH based upon UE declared capability

Issue 1-6: Applicability rules for CSI test

During RAN4#106-bis-e the following table were presented in the WF and summary as an option for applicability rule for PDCCH.

8Rx capable UE can skip all legacy 2Rx and 4Rx CSI tests.

We note that within the work split for 8Rx that the applicability rules for CSI test are not covered, and this would require a change to CSI reporting applicability of requirements clauses in TS 38.101-4.

We propose that CSI reporting applicability rules are included in the work split.

PBCH Applicability Rule

Table 5.1.1.2-1 of TS 38.101-4 [5] includes PBCH applicability rules for the 4Rx and 2Rx cases, as well as including the PDSCH and PDCCH applicability rules. Should this table be expanded to include PDSCH and PDDCH as agreed in the WF then we believe we should also introduce PBCH applicability rules for 8Rx.

There is currently no proposals for an 8Rx PBCH Applicability Rule

We propose that a PBCH Applicability Rule for 8Rx UEs shall be introduced which includes representative 2Rx and 4Rx applicability tests depending on UE capability.

TP of Applicability Rules

RAN4 to adapt the following as an update of Table 5.1.1.2-1 Requirements Applicability within TS 38.101-4.

Table 5.1.1.2-1: Requirements applicability (updated from [5])

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports only 2RX | PDSCH | All tests in Clause 5.2.2 |
|  | PDCCH | All tests in Clause 5.3.2 |
|  | PBCH | All tests in Clause 5.4.2 |
| UE supports only 4RX or both 2RX and 4RX | PDSCH | All tests in Clause 5.2.3 (Note 2) |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 1) |
| UE supports 8Rx, 4Rx and 2Rx | PDSCH | All tests in Clause 5.2.3 or 5.2.4(Note 2) |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 4) |
| UE supports only 8Rx and 4Rx | PDSCH | All tests in Clause 5.2.3 or 5.2.4(Note 2) |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 4) |
| UE supports only 8Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 or 5.2.4(Note 2) |
|  | PDCCH | All tests in Clause 5.3.2 |
|  | PBCH | All tests in Clause 5.4.2 |
| UE supports only 8Rx | PDSCH | All tests in Clause 5.2.3 or 5.2.4(Note 2) |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2, Note 3) |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 4) |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration. Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: For UEs supporting only 8Rx, tests will apply by duplicating the fading channel from each Tx antenna with the application of independent noise for each Rx demodulation branch. Note 4: Requirements for PBCH with 8Rx is up to UE declaration. |  |  |

Conclusion

This contribution provides our views on general items relating to 8RX UE demodulation. The proposals are summarized as follows:

Proposal 1: We propose RAN4 shall define applicability rules for PDSCH based upon UE declared capability.

Proposal 2: We propose RAN4 shall define applicability rules for PDCCH based upon UE declared capability

Proposal 3: We propose that CSI reporting applicability rules are included in the work split.

Observation 1: There is currently no proposals for an 8Rx PBCH Applicability Rule

Proposal 4: We propose that a PBCH Applicability Rule for 8Rx UEs shall be introduced which includes representative 2Rx and 4Rx applicability tests depending on UE capability.

Proposal 5: RAN4 to adapt the following as an update of Table 5.1.1.2-1 Requirements Applicability within TS 38.101-4.

Table 5.1.1.2-1: Requirements applicability (updated from [5])

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports only 2RX | PDSCH | All tests in Clause 5.2.2 |
|  | PDCCH | All tests in Clause 5.3.2 |
|  | PBCH | All tests in Clause 5.4.2 |
| UE supports only 4RX or both 2RX and 4RX | PDSCH | All tests in Clause 5.2.3 (Note 2) |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 1) |
| UE supports 8Rx, 4Rx and 2Rx | PDSCH | All tests in Clause 5.2.3 or 5.2.4(Note 2) |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 4) |
| UE supports only 8Rx and 4Rx | PDSCH | All tests in Clause 5.2.3 or 5.2.4(Note 2) |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 4) |
| UE supports only 8Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 or 5.2.4(Note 2) |
|  | PDCCH | All tests in Clause 5.3.2 |
|  | PBCH | All tests in Clause 5.4.2 |
| UE supports only 8Rx | PDSCH | All tests in Clause 5.2.3 or 5.2.4(Note 2) |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2, Note 3) |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 4) |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: For UEs supporting only 8Rx, tests will apply by duplicating the fading channel from each Tx antenna with the application of independent noise for each Rx demodulation branch. Note 4: Requirements for PBCH with 8Rx is up to UE declaration |  |  |

References

- RP-221496 “Revised WID Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”

- R4-2220152, “Topic summary for [105][326] RF_FR1_enh2_Demod”, Huawei, HiSilicon, RAN WG4 #105, Toulouse, Nov. 2022

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106-bis-e, Online, April 17 – April 26, 2023

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106, Athens, Greece, 27 February - 03 March, 2023

- TS 38.101-4 v17.8.0, Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 17), March 2023