3GPP TSG WG4 Meeting # 107	R4-2307230

Incheon, Korea, 22 – 26 May, 2023

Agenda Item:	8.2.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.877: Clause 4.2 and Clause 5

Document for:	Approval

# 1.	Introduction

The revised SID on NR BS RF requirement evolution was approved at TSG RAN#97-e [1] in which one of the objectives is to study the following aspects relating to FR2 multi-band BS:

    - Investigate whether a generic solution for all combinations within FR2-1 is possible and/or a solution for all or a part of the frequency range should be targeted

      - Frequency range 24-29 GHz which includes n257/n258/n261

      - Frequency range 37-48 GHz which includes n260/n259/n262

The recent version of TR38.877 [2] was agreed in RAN4#106bis-e meeting. However, we found that:

- In Clause 4.2.1, several feasibility aspects of FR2 multiband RIB are discussed (i.e., antenna array, PA and component placements), whereas the scope of Clause 4 is to present the scope of the SI. Moreover, some points in the feasibility discussion seem overlapped with the study in Clause 5.

- In Clause 5.1, multi-band FR2-1 BS was referred to, but the corresponding definition is for multi-band RIB.

This contribution provides a text proposal to:

- relocate/merge some discussions in Clause 4.2.1 to the relevant subclauses in Clause 5.

- correct the wrong statement in Clause 5.1

# 2.	Text proposal

<Start of change>

# 4	General

# 4.1	General

### 4.2.1	Architecture options

Figure 4.2.1-1 depicts potential architectures based on the implementation options available for multi-band support in FR2-1 BS. The main components included are the PA, antenna array, and diplexers (wherever needed). Note that there are no active RF components before the RF input in any of the options in the figure.

Figure 4.2.1-1:  High-level architecture options to support multi-band operation in FR2-1

From the options captured in the figure, Option #2 and Option #4 are the target scenarios of this study item, while no study is needed for Option #5. For Option #1 and Option #3, if the PA is capable of being operated in multiple bands at the same time, then it supports multi-band operation. However, if the PA can only be configured and operated in a single-band, then it is considered a single-band RIB.

Option #1 and Option #3 are feasible with current technology if the ranges covered by each path have a fractional bandwidth up to 19.5%. Per agreements in sub-Clause 5.2.1.4, Option #2 and Option #4 should also be feasible for a fractional bandwidth up to 19.5%. Note that even within the same frequency group, the performance attained would yield a lower Tx power compared to a single-band design. However, given that this requirement is a manufacturer declaration, it would not impact the requirement applicability.

# 5	Feasibility study

# 5.1	General

According to the definition in section 6.1, a multi-band FR2-1 RIB is a RIB that transmits in two or more FR2-1 bands using common active components. In this section, the feasibility of an FR2-1 multi-band solution is considered. The feasibility is examined taking into account technology challenges and emerging solutions. It is possible that not all parts of the BS are multi-band but that some advantage can be gained by implementing some parts as multi-band. For example one potential advantage of a multiband PA might be that power can be distributed between the supported bands offering more flexibility to optimize the power available for each, this requires only a multi-band PA.

The evaluation considers whether the proposed set of FR2-1 requirements is likely to be achievable with present or future implementation possibilities. The study does not aim to conclude on whether a multi-band FR2 solution is more optimal or effective (considering complexity, power, weight etc.) than other possibilities, for example mounting individual radio panels for each band within a BS enclosure.

When discussing the feasibility of wideband components/systems the multi-band bandwidth as a percentage of the carrier frequency should be considered.

Note that this is generally called fractional BW but in [2]the term fractional BW is already defined to mean something specific so is best avoided to prevent confusion.

Fractional bandwidth (FBW) is defined for a single operating band as shown below:

Fractional bandwidth (FBW) is defined in [2] as FBW=200∙FFBWhigh-FFBWlowFFBWhigh+FFBWlow%,

where FFBWhigh is highest supported frequency within supported operating band, for which fractional bandwidth support was declared, and FFBWlow is lowest supported frequency within supported operating band, for which fractional bandwidth support was declared.

Multiple operating bands are discussed in this study so the existing definition of fractional bandwidth is not suitable, the term percentage bandwidth is therefore used and defined as follows:

Percentage bandwidth (PBW) is defined in this study as PBW=200∙FPBWhigh-FPBWlowFPBWhigh+FPBWlow%,

where FPBWhigh is highest supported frequency, for which percentage bandwidth support was declared, and FPBWlow is lowest supported frequency, for which percentage bandwidth support was declared.

Table 5.1-1 shows the percentage bandwidth for the FR2-1 bands and various potential multi-band groups.

Table 5.1-1:Percentage band width of FR2-1 operating band (s)

| Frequency group, NR operating band pairing (lowest, highest) | Frequency range | Percentage bandwidth |
| --- | --- | --- |
| 26 GHz (n258) | 24250 MHz – 27500 MHz | 12.6% |
| 28 GHz (n257) | 26500 MHz – 29500 MHz | 10.7% |
| 28 GHz (n261) | 27500 MHz – 28350 MHz | 3% |
| 39 GHz (n260) | 37000 MHz – 40000 MHz | 7.8% |
| 40 GHz (n259) | 39500 MHz – 43500 MHz | 9.6% |
| 48 GHz (n262) | 47200 MHz – 48200 MHz | 2.1% |
| 24-29 GHz (n257/n258/n261) | 24250 MHz –29500 MHz | 19.5% |
| 37-48 GHz (n260/n259/n262) | 37000 MHz – 48200 MHz | 26.3% |
| 26+28 GHz (n258, n261) | 24250 MHz – 29500 MHz | 19.5% |
| 28+39 GHz (n257/n261, n260) | 26500 MHz – 40000 MHz | 40.6% |
| 26+40 GHz (n258, n259/n262) | 24250 MHz – 48200 MHz | 66.1% |
| 28+40 GHz (n257/n261, n259/n262) | 26500 MHz – 48200 MHz | 58.1% |

# 5.3	Wideband antenna

5.3.3	Interleaved array structures

If the required multi-band antenna bandwidth is too great to be handled by a single antenna array (for the required steering range) it is also possible to use interleaved array structures such as stacked patches. Using stacked patches allows the different bands to feed different elements which are tuned for the specific band. This resolves some of the physical restrictions on the element size. However, if the arrays are stacked then restrictions still exist on the element separation in the array.

Stacked patches can be designed with either a single dual band input or with separate input ports. As the bands are separated (for example 26GHz and 40GHz), it is possible to implement a diplexer to separate the bands for each of the antenna arrays, if necessary.

Figure 5.3.3-1: Multi-band PA with stacked single band arrays (single and dual input ports)

A number of papers have been identified [19], [20], [21], and [22] which have demonstrated the feasibility of operational stacked patch antennas in the range 26 to 40 GHz

Dual band performance can be achieved by using multiple patches stacked on top of each other. For example, in [20] a simple stack with a high band patch and a low band patch with a single input port was shown. Using these patches, a 2x2 array was demonstrated with acceptable levels of input match, boresight gain isolation and scanning angle. Although of course such small arrays are more suitable for a handset than a BS.

In [22] a larger 8x4 array was demonstrated along with separate single band beamformers. While this is still small for BS purposes, it demonstrates the scalability of such structures and that larger BS-size arrays may be technically feasible to implement.

From the above discussions for different antenna array structures, either a multi-band antenna array (broadband or with multi-resonances) or separate antenna arrays with lower percentage bandwidth can be used in FR2 multiband RIB/BS. In comparison to separate antenna arrays, a consolidated multi-band design is more compact and reduces costs, but trades performance aspects that can be optimized for in separate designs. On the other hand, having separate designs enables dedicated optimizations, yielding better performance. However, this comes at the cost of a significantly larger circuit area being used for two designs and additional integration losses from lines and transitions.

# 5.4	Other

In this section, a number of other considerations for FR2-1 wideband BS from the feasibility point of view, that are not considered in the preceding sections are mentioned.

Digital considerations

To support a large bandwidth, it is necessary to provide a high sampling rate for ADC and DAC. For individual Frequency Band Groups (e.g. 24.5-29.5GHz, n257+n258), the bandwidth to be supported may be more than 5GHz, and to cover the full FR2-1 the bandwidth might be up to 24GHz. If linearization would be seen later to be feasible for multiband BS, then DAC/ADC would need to support for the feedback and correction, which poses another challenge to DAC/ADC.

Potentially only the in band spectrum could be generated by separate converters [xx] which would reduce the requirements on individual converters.

In an implementation, the architecture, RF performance and power consumption of the analogue/digital interface would be key considerations. The ADC and DAC complexity and power consumption could be reduced by reducing the sampling resolution, but this would impact TX factors such as EVM and emissions and RX factors such as dynamic range and RX EVM. For this reason, care would need to be taken that requirements on e.g. EVM, receiver dynamic range and demodulation performance would be achievable.

In addition to the DAC and ADC, the large bandwidth and sampling rates would lead to very high volumes of data to be moved within the radio architecture. Data transport and interface architectures would need to support the very high data volume. Reducing the data volume (e.g. by reducing the sampling resolution) would lead to similar considerations as for ADC and DAC on meeting requirements such as EVM, emissions, RX dynamic range and demodulation.

In addition to the interface bandwidth, the digital transport latency may also impact radio near algorithms  (such as DPD, CFR) and could impact the performance of the transmitter and receiver. It is not clear whether the need to support a much larger interface bandwidth could impact the interface latency.

Digital filtering may be needed for meeting selectivity and blocking requirements depending on the sensitivity and architecture. The large sampling rates and bandwidth would increase the amount of computational power needed for digital filtering, and potentially the achievable steepness of the filters. This could impact the feasibility of meeting TX EVM and RX selectivity, blocking and demodulation requirements.

Analogue considerations

The possibilities for analogue filtering within an FR2-1 AAS array are extremely limited and are described in [xx]. The filter in a typical single-band FR2 BS may not be placed before the antenna. However, it could be the case of multi-band, due to the multi-frequency signal going through the PA if the PA is highly nonlinearity, or due to the architecture splitting multi-band signals. And thus filter before the antenna may be needed.

Choosing where to place filter components is an important part of a multi-band solution as it impacts the overall performance. For instance, if a filter would need to be placed before the antenna, then there are other issues in practice with insertion loss and power unbalancing due to non-identical filter for each branch.

The regulatory requirements in the inter-RF gap and the linearity of the transmitter will set the boundary for the analogue filtering. The feasibility of analogue filtering may impact the ability to meet some regulatory emissions requirements and out of band blocking, and if the filter has ripple also the EVM may be impacted.

Depending on the array architecture, it may be necessary to split different frequency components of the multi-band signal and route them to different antenna elements, e.g., in architectures using diplexer. The splitting and additional routing can have implications for TX power loss and RX sensitivity, and the placement of components needs to be carefully considered. For instance, placing the diplexer before the PA is preferred from a design loss and power dissipation perspective. This is because the insertion loss of the diplexer occurs at a lower power level and can be accommodated by a simple increase in driver gain. Conversely, if the diplexer is placed after the PA, the insertion loss of the diplexer directly reduces the power available to the antenna array and leads to more thermal dissipation.

On the other hand, it is worth noting that if the power loss is significantly different/uneven between antenna branches, then beamforming may be spoiled, and calibration may be required which may bring complexity to the architecture. This may impact the feasibility of the multi-band solution, although since TX power and RX sensitivity are subject to declarations it may not impact the requirements definition. As shown in Clause 4.2.1 Option #4, splitting and additional routing can be avoided by using a multi-band antenna, such as a single input stacked patch design.

Dedicated PAs for the desired frequency ranges may be adopted from a design loss and power dissipation perspective, as the narrower band designs will have better efficiency and the thermal dissipation will be spread out over a larger area.

<End of change>

# References

[1]	RP-222501, “SID revision: NR BS RF requirement evolution”, Huawei, HiSilicon.

[2]	R4-2307753, “TR 38.877 v0.4.0”, Huawei, HiSilicon.