3GPP TSG-RAN WG4 Meeting # 107	R4-2307264

# Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.22.3

Source: 	Qualcomm Incorporated

Title: 	On AI/ML RAN4 Testing Framework: Interoperability and testability aspect

Document for:	Approval

# Introduction

We present our view on the scope of AI/ML work item in this contribution.

# Discussion

## General RAN4 Testing Framework

RAN4 defines requirements and tests on the UE and network devices to ensure that the system can operate as expected by the designed procedures in the standard. Moreover, it provides the principles and basic sets of tests for different entities, e.g., OEM, operator, to develop testing procedures to verify the devices and interoperability of the connected system. AI/ML brings additional challenges to the existing requirements and testing framework in wireless communication systems: the AI/ML model may not have well-defined structure and output message interpretation, and there are many model management procedures to accommodate different environments and along the product life cycle. New frameworks or methodologies have to be studied to define proper requirements and tests for AI/ML model.

Test configuration

Data generator

AI/ML model

Model control/ signaling

AI/ML model

Performance monitoring

DUT

TE

Data collection

We summarize the list of issues captured in WF[1] in the following figure and items:

The above diagram illustrates the issues to be studied in [1]. The testing loop first configures DUT according to the agreed test setup, and the data generator in the TE transmit signals to the DUT. DUT then process the signals by its AI/ML model, and provides the report or transmit the signals produced by the AI/ML model. When two sided model is considered, TE has a corresponding reference AI/ML model to process the received signals at TE. Then TE uses the metric defined in the test to validate AI/ML model in the DUT. Several types of tests/procedures are captured in [1] for future studies:

- Inference tests: under a static configuration, TE verifies the performance of AI/ML model based on the output it produces with possible post processing by TE’s reference AI/ML model, if two sided model is considered.

- Model management related tests: DUT executes the actions triggered by model control or associated signaling, and TE verifies the reaction on the DUT AI/ML model against the defined requirement.

- Performance monitoring related tests: TE generates the signal based on the agreed test setup, and DUT produces report based on AI/ML model output and/or received signal/information from TE. TE then verifies the report against the requirement.

- Data collection: RAN4 can collect (emulated) data from DUT AI/ML model output and/or data generator in TE.

We discuss the open issues regarding the feasibility of the above tests/procedures for UE side testing in the following:

- Performance monitoring

Performance monitoring may require UE to collect statistics based on its AI/ML model output and signals from the network, estimate the performance accordingly, and report the requested quantities to the network. RAN1 is still discussing the performance monitoring procedures, and therefore we provide analysis and observations for the potential requirement discussion based on the KPIs captured in [1] without assumptions on specific procedure. Take CSI feedback enhancement as an example. CSI feedback mechanism aims at providing accurate channel state information, explicitly or implicitly, to the gNB and improve DL throughput by choosing the correct precoding matrix and MCS at gNB. Therefore, throughput is a reasonable metric to monitor for AI/ML model performance, and throughput has been captured as the KPI for RAN4 study in CSI use cases in [1]. Measurement on throughput and some related quantities, e.g., BLER, are straightforward and known to both the UE and network. However, many other factors contribute to throughput besides CSI, e.g., propagation condition/signal quality change, presence of interference. Therefore, measuring AI/ML model performance based on throughput becomes challenging due to these additional factors, and determine margins to account for all the factors so that we can define requirements on the performance measurement may not be feasible even with a procedure specified by RAN1. On the other hand, if we want to consider the direct metrics, which are based solely on the AI/ML model, captured in [1], e.g., SGCS/NMSE or accuracy of CSI prediction, the availability of the appropriate quantity to compare with the model output becomes an issue. Whether there exists an appropriate quantity to compare with the model output which can isolate model performance from other factors, and how to communicate it to UE or how UE can derive the quantity, are critical issues to be resolve before discussing the requirement on monitoring the metrics. The potential resolution for the issues mentioned above requires RAN1 procedure specification, and RAN4 can wait until RAN1 has enough progress to continue the related requirement discussion.

- Model/functionality management

If the model/functionality management procedure follows legacy 3GPP signaling framework, i.e., UE executes the actions after receiving the configuration signaling from the network, the requirement discussion fallbacks to the legacy signaling procedure requirements, which depends on the signaling procedure design. However, when considering the actions involves UE decision, there is implicit performance aspect on decision correctness in the requirement discussion, and the feasibility of UE based decision as part of the model management procedure depends on the availability of required information on UE side, which is subject to RAN1 decision.

- Data generation and collection

Traditionally, RAN4 tests are executed by TE emulating the network and environment by generating signals based on abstract models, e.g, TDL or CDL channels. However, AI/ML models are mostly targeting complicated and dynamic scenarios, which may go beyond the capability of TE data generations with reasonable model and complexity. Field data as a source for data generator in the test becomes an alternative to resolve the dynamic and complicated environment emulation issue and is also captured as an option in [1]. However, there are many open issues to resolve before concluding that field data is applicable to RAN4 tests. If RAN4 defines a fixed dataset captured in standard, the test becomes deterministic, and overfitting the model to the dataset and lack of generalization may deteriorate the value of the test. For example, UE can train the model based on the entire dataset in the standard, and this UE can perform well in the test. However, once the environment is not perfectly aligned with the dataset, we have no idea how robust the model is against misalignment to the dataset. One the other hand, if RAN4 doesn’t specify the field dataset for testing and UE vendors don’t have access to the dataset, UE can’t properly train the model to capture the characteristics of the environment represented by the dataset. Moreover, model performance varies across different dataset collected from different environments even if the model is properly trained against the dataset. How RAN4 specifies a unified requirement that can accommodate different dataset used by different TEs is an open issue requiring a proper resolution.

- Prediction

It is important to ensure the accuracy of the prediction/estimation from AI/ML model, and therefore the performance/testing requirement on accuracy is an essential topic in the AI/ML model requirements. In legacy measurement accuracy requirement, RAN4 specifies error margins together with a sets of side conditions, and the specification requires UE to achieve accuracy within the error margin when the side conditions are satisfied. The side conditions can be explicitly specified as SINR/RSRP conditions, or implicitly specified in measurement periods which determines how many measurement samples can be collects. However, there is a new aspect being introduced to the accuracy requirement framework when we consider prediction accuracy: the prediction is on a set of quantities not directly observed by the UE, and UE derives those quantities by a set of quantities observed by the UE. In fact, channel estimation and CSI reporting are the procedures with properties similar to prediction based on AI/ML model. The set of quantities observed and to predict in channel estimation and CSI reporting procedures are well-defined in the specification: the location, periodicity, density of the reference signals and the reporting period. On the other hand, the relationship between the observed set and to predict set may not be explicitly specified in some use cases, e.g, in beam prediction, beam width, number of beams in the observed/to predict set, beam separation etc. may depend on network implementation and vary across scenarios and vendors. Therefore, to define a fixed requirement, in addition to the legacy side conditions based on SINR or RSRP, we may have to include the configurations determining the relationship between observed and to predict sets as part of the side conditions, otherwise the margin needs to be very large to accommodate different types of deployment. However, side conditions on observed and to predict sets specified limit the applicable scenarios. Therefore, whether it is feasible to define a meaningful requirement with a reasonable margin and side conditions covering a large enough sets of common deployment scenarios becomes an issue required careful study.

Note that we don’t have general performance requirements on channel estimation or CSI reporting defined by a margin and side conditions; instead, we define tests with specified environment that are directly verifying CSI reporting performance, e.g., CSI reporting tests, or indirectly verifying channel estimation performance, e.g. demod and CSI reporting tests. However, it is challenging to design tests on prediction accuracy with specific testing conditions in which the achievable accuracy levels in the tests are good indicators of common deployment scenarios, due to the same reason as side condition specification explained above.

Test feasibility is another issue we observed for prediction. We use beam prediction as an example to explain the issue. When we consider beam prediction (on spatial or temporal domain), the prediction accuracy in absolute sense, e.g, predicted RSRP accuracy, or in relative sense, e.g., relative RSRP or best beam ranking, is a reasonable metric to be considered in performance monitoring, and it is also captured in [1]. However, it is challenging to acquire the ground truth to evaluate the prediction accuracy. We provide an example below to explain the challenge of acquiring the ground truth. Denote the RS set corresponding to the beams being predicted as RS set A, and the set being measured, to predict RS set A, as RS set B. A straightforward way to acquire ground truth and validate the prediction is by requesting beam prediction corresponding to RS set A at time T2, and sending RS set A at time T3 to validate the prediction at T2. However, the propagation conditions are not static, and therefore both the prediction accuracy and propagation condition change can contribute to the difference between prediction at T2 and the measurement at T3. Therefore, even in this simplified scenario, it’s not straightforward to derive the desired metric that can isolate the model accuracy from other environment factors, and is challenging to define a requirement on performance monitoring that can accommodate the uncertainty introduced by environment changes outside the AI/ML model.

RS set A, B

RS set B

Report 1 with measurement of A, B

Report 2 with measurement of B, prediction of A

RS set A, B

Report 3 with measurement of B, validation of A

RS set A predicted for T2 is not necessary the same as RS set A measured in T3, i.e., RS set A(T3) may not be a valid ground truth for RS set A(T2)

T1

T2

T3

- Test equipment support

RAN4 tests are specified mostly under static and simplified conditions so that the equipment can emulate the environment efficiently. However, AI/ML models perform better than the legacy wireless communication algorithms mostly in a more complicated and dynamic environment which may not easily be captured by simple abstract models. For example, positioning use case targets indoor scenarios which have abundant reflection paths from different directions; beam prediction use case is most beneficial to the scenario with a large number of beams pointing to different directions. However, the state of art test equipment design relies on simplified tap or cluster based propagation models, and the number of beams that can be emulated in the test chamber are very limited.

- Two-sided model

The following principles for studying the framework for two-sided model test are captured in the WF[1]

  - Common assumptions for proposals of the reference decoder/encoder (and the paired ones) for tester

  - Definition and derivation procedure of intermediate KPI for decoder evaluation and selection

  - Data collection/generation for decoder/encoder evaluation, and the common assumptions/environment needed for data collection/generation

  - How to minimize the impact of possible variations/differences in the reference decoder/encoder design/implementation on UE/ gNB performance verification

  - The impact of reference decoder/ encoder for testing complexity to UE/gNB performance verification, and the advantage/disadvantage analysis of high/low complexity decoders.

  - FFS whether any reference for the encoder/ decoder needs to be considered given that the encoder/decoder performance is to be tested

Based on the above agreement, we explain our view in the next section.

## Reference Decoder for UE test for CSI compression test

We first propose a candidate set of common assumptions for two-sided CSI compression test:

Proposal 1: Consider the following as a candidate set of common assumptions for two-sided CSI compression test that should be agreed in RAN4 when defining the test in *WI* phase

- Propagation condition

- CSI report and format in the encoder/decoder pair

  - Assumptions for quantization/dequantization scheme/format in the encoder/decoder

  - Payload size and format of the latent message

  - Output CSI format in the decoder

- Common CSI test configurations: reporting periodicity, antenna configuration, CSI-RS periodicity

Next, we discuss the following options selected from the ones captured in the WF [1]:

    - Option 1: reference decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained

    - Option 2: reference decoder is provided by the vendor of the decoder so that the encoder and decoder are jointly designed and trained

    - Option 3: The reference decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed.

    - Option 4: The reference decoder(s) are partially specified and captured in RAN4 spec.

With option 1, UE vendor can jointly design and train reference decoder and encoder, and it has the following benefits and potential issues:

Observation 1: The following are observed for the reference decoder provided by the vendor of the encoder under test

- Joint encoder/decoder design by the UE vendor can alleviate the impact of model mismatch and any potential performance limitation due to a decoder, and therefore better capture the native performance of the encoder.

- Leaving the reference decoder design and training to UE vendor avoids the limitation on UE encoder design implicitly imposed by the reference and allows UE vendors to update the model when new AI/ML model with better performance is developed in the future.

- Reduce RAN4 loading on specifying a reference decoder.

- Encoder is jointly designed/trained with decoder, and may or may not work for the decoders in the field which may have a model structure mismatch and/or may not be jointly trained. How much performance degradation due to such mismatch and lack of joint training depends on further study of model mismatch and training method impact. And note that collaboration procedures between network and UE vendors can reduce this impact.

- Possible lengthy RAN4 discussion due to large performance span from alignment results

Proposal 2: Given the advantages listed above, RAN4 can select the reference decoder provided by the vendor of the encoder under test.

We also observe the following issues for option 2:

Observation 2: The following are observed for the reference decoder provided by the decoder vendors

- Can test with real infra vendor decoder, no additional RAN4 encoder not use in practice and the test can reflect the performance in the field.

- It’s not clear which network vendor provides the decoder to the TE. If the spec requires UE to pass the same test for all the models provided by different network vendors, the test burden is very large and not feasible for UE vendors.

- It is not clear how to define a requirement for models provided by different network vendors. The achievable performance may vary from models to models, and if there are a few models perform worse than other models, UE may fail the test due to a model provided by network vendor of which UE vendor doesn’t have control. Then this test can’t verify UE performance.

Proposal 2: The reference decoder provided by the decoder vendors is not preferred due to the above issues.

For option 3, we have the following observations:

Observation 3: The following are observed for the fully specified reference decoder

- Simpler testing procedure since TE can directly implement the decoder, otherwise the decoder format has to be defined so that TE can load external decoders

- Possible lengthy RAN4 discussion to agree on one (or more) fully specified reference decoder

- Encoder is designed/trained with full decoder knowledge, and may or may not work for the decoders in the field which may have a model structure mismatch. How much performance degradation due to such mismatch depends on further study of model mismatch impact. And note that collaboration procedures between network and UE vendors can reduce this impact.

- If the ideal fully specified reference decoder design that works for all types of encoder is not feasible, and network vendors do not consider the fully specified reference decoder as part of their implementation, there are potential issues:

  - UE has to implement an additional encoder only for the RAN4 test, or the UE encoder implementation flexibility is limited by the RAN4 test

We propose the following principles for decoder derivation:

Proposal 3: We propose the following principles to design the decoder derivation procedure for the fully specified reference decoder:

- In WI phase, RAN4 to agree on complexity limitation based on feasibility of TE implementation and common complexity range considered by network vendors for decoder deployment.

- RAN4’s choice of reference decoder should avoid its impact on the implementation choices, including complexity, back-bone model etc, of UE encoders operating in the field with different propagation conditions and configurations than RAN4 test design.

- The reference decoder choice should correctly reflect the UE performance in the test RAN4 to study if the following principle is feasible: UE with an encoder that performs well when paired with an intended decoder shall pass the test when paired with the reference decoder; UE with an encoder that performs poorly even when paired with an intended decoder shall fail the test when paired with the reference decoder.

We observe the following issues for option 4:

Observation 4: : The following are observed for the partially specified decoder:

- When the reference decoder is partially specified, the unspecified part is left to TE vendor implementation, and a similar problem is observed as in option 2: The achievable performance may vary from TE to TE depending on the different design, and if there are a few TE models perform worse than other TE models, UE may fail the test due to a model provided by TE of which UE vendor doesn’t have control. Then this test can’t verify UE performance.

- Given that different TEs may have different reference decoders, UE may have to implement multiple encoders when test against TE models, which increases UE vendor’s burden.

Proposal 4: The partially specified reference decoder is not preferred due to the above issues.

# Conclusion

Proposal 1: Consider the following as a candidate set of common assumptions for two-sided CSI compression test that should be agreed in RAN4 when defining the test in *WI* phase

- Propagation condition

- CSI report and format in the encoder/decoder pair

  - Assumptions for quantization/dequantization scheme/format in the encoder/decoder

  - Payload size and format of the latent message

  - Output CSI format in the decoder

- Common CSI test configurations: reporting periodicity, antenna configuration, CSI-RS periodicity

Observation 1: The following are observed for the reference decoder provided by the vendor of the encoder under test

- Joint encoder/decoder design by the UE vendor can alleviate the impact of model mismatch and any potential performance limitation due to a decoder, and therefore better capture the native performance of the encoder.

- Leaving the reference decoder design and training to UE vendor avoids the limitation on UE encoder design implicitly imposed by the reference and allows UE vendors to update the model when new AI/ML model with better performance is developed in the future.

- Reduce RAN4 loading on specifying a reference decoder.

- Encoder is jointly designed/trained with decoder, and may or may not work for the decoders in the field which may have a model structure mismatch and/or may not be jointly trained. How much performance degradation due to such mismatch and lack of joint training depends on further study of model mismatch and training method impact. And note that collaboration procedures between network and UE vendors can reduce this impact.

- Possible lengthy RAN4 discussion due to large performance span from alignment results

Proposal 2: Given the advantages listed above, RAN4 can select the reference decoder provided by the vendor of the encoder under test.

Observation 2: The following are observed for the reference decoder provided by the decoder vendors

- Can test with real infra vendor decoder, no additional RAN4 encoder not use in practice and the test can reflect the performance in the field.

- It’s not clear which network vendor provides the decoder to the TE. If the spec requires UE to pass the same test for all the models provided by different network vendors, the test burden is very large and not feasible for UE vendors.

- It is not clear how to define a requirement for models provided by different network vendors. The achievable performance may vary from models to models, and if there are a few models perform worse than other models, UE may fail the test due to a model provided by network vendor of which UE vendor doesn’t have control. Then this test can’t verify UE performance.

Proposal 2: The reference decoder provided by the decoder vendors is not preferred due to the above issues.

Observation 3: The following are observed for the fully specified reference decoder

- Simpler testing procedure since TE can directly implement the decoder, otherwise the decoder format has to be defined so that TE can load external decoders

- Possible lengthy RAN4 discussion to agree on one (or more) fully specified reference decoder

- Encoder is designed/trained with full decoder knowledge, and may or may not work for the decoders in the field which may have a model structure mismatch. How much performance degradation due to such mismatch depends on further study of model mismatch impact. And note that collaboration procedures between network and UE vendors can reduce this impact.

- If the ideal fully specified reference decoder design that works for all types of encoder is not feasible, and network vendors do not consider the fully specified reference decoder as part of their implementation, there are potential issues:

  - UE has to implement an additional encoder only for the RAN4 test, or the UE encoder implementation flexibility is limited by the RAN4 test

Proposal 3: We propose the following principles to design the decoder derivation procedure for the fully specified reference decoder:

- In WI phase, RAN4 to agree on complexity limitation based on feasibility of TE implementation and common complexity range considered by network vendors for decoder deployment.

- RAN4’s choice of reference decoder should avoid its impact on the implementation choices, including complexity, back-bone model etc, of UE encoders operating in the field with different propagation conditions and configurations than RAN4 test design.

- The reference decoder choice should correctly reflect the UE performance in the test RAN4 to study if the following principle is feasible: UE with an encoder that performs well when paired with an intended decoder shall pass the test when paired with the reference decoder; UE with an encoder that performs poorly even when paired with an intended decoder shall fail the test when paired with the reference decoder.

Observation 4: The following are observed for the partially specified decoder:

- When the reference decoder is partially specified, the unspecified part is left to TE vendor implementation, and a similar problem is observed as in option 2: The achievable performance may vary from TE to TE depending on the different design, and if there are a few TE models perform worse than other TE models, UE may fail the test due to a model provided by TE of which UE vendor doesn’t have control. Then this test can’t verify UE performance.

- Given that different TEs may have different reference decoders, UE may have to implement multiple encoders when test against TE models, which increases UE vendor’s burden.

Proposal 4: The partially specified reference decoder is not preferred due to the above issues.

# Reference

[1] R4-2306299