3GPP TSG-RAN4 Meeting #107	                                                            R4-2307413

Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on RRM requirements of LPHAP

Source:	CATT

Agenda Item:	8.23.3.3

Document for:	Discussion

1 Introduction

In last meeting, RAN4 has discussed general RRM impact of LPHAP with conclusions captured in [1]. RAN1#112bis-e meeting has also discussed the work item and the agreements were captured in [2]. In this paper, we further discuss the potential RRM requirements for LPHAP based on RAN1 progress.

2 Discussion

Based on the following objective in the WID, the main enhancement for LPHAP use case is to extend eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement and to define the requirements for eDRX cycle beyond 10.24s. Although it is possible to define the requirements for eDRX <=10.24s, we think it is better to prioritize the requirements for beyond 10.24s to follow the intention of the WID.

Proposal 1: Not to define requirements for eDRX <= 10.24s.

For the requirements when eDRX is larger than 10.24s, since it is discussing in Rel-18 Redcap, we think we can wait for the conclusions and take them as the baseline for positioning.

Proposal 2: For the requirements when eDRX is larger than 10.24s, take the requirements for eDRX cycle > 10.24s in Rel-18 eRedCap as baseline.

In last meeting, it was agreed that reduced number of PRS samples, parallelPRS-MeasRRC-Inactive-r17 capability and lower Rx beam sweeping in FR2 are supported for PRS measurement with eDRX in RRC_INACTIVE and the other possible adaptions are FFS. For the number of samples, we think both reduced samples and baseline 4 samples can be considered in the requirements. But considering the very long eDRX cycle, we are also fine to prioritize reduced samples.

And for the measurement requirements, either PRS periodicity or DRX cycle should be used for the measurement interval. Considering the eDRX cycle is much longer, we think we can use PRS periodicity as baseline to define available measurement interval. In this case, the Tavailable needs to be updated to exclude the DRX cycle.

For the SRS validation area, based on RAN2 agreement, the configuration contains a list of cells in which it is valid. We understand the determination on whether UE is within validation area can be left to UE implementation and what we should consider is the impact on measurement period requirements when UE reselects out of the positioning validity area during SRS transmission.

Proposal 3: RAN4 to prioritize the measurement requirements with reduced samples.

Proposal 4: Tavailable in the requirements needs to be updated to exclude the DRX cycle when eDRX cycle is much larger than positioning interval.

Proposal 5: RAN4 not to define cell reselection requirement for the determination of validation area.

For the measurement in RRC_IDLE mode, it was agreed to reuse the requirements in RRC_INACTIVE. We think the Rel-17 feature including reduced samples, reduced Rx beams and TEG should be all included and the accuracy requirements can also be reused.

Based on existing reporting requirements, the extra time to transition to RRC_CONNECTED is not included. We think this can also apply to RRC_IDLE.

Proposal 6: Reuse values from Rel. 17 for NRx,TEG,I, Nsamples and NRxBeam,i in positioning measurement delay requirements for RRC_IDLE state.

Proposal 7: Accuracy requirements in RRC_INACTIVE state also apply to positioning measurements in RRC_IDLE.  .

Proposal 8: The reporting delay for PRS measurements performed in RRC_IDLE doesn’t need to account for extra delay to transition to RRC_CONNECTED state.

3 Summary

In this paper, we discuss the RRM aspects on LPHAP and the following proposals are given:

Proposal 1: Not to define requirements for eDRX <= 10.24s.

Proposal 2: For the requirements when eDRX is larger than 10.24s, take the requirements for eDRX cycle > 10.24s in Rel-18 eRedCap as baseline.

Proposal 3: RAN4 to prioritize the measurement requirements with reduced samples.

Proposal 4: Tavailable in the requirements needs to be updated to exclude the DRX cycle when eDRX cycle is much larger than positioning interval.

Proposal 5: RAN4 not to define cell reselection requirement for the determination of validation area.

Proposal 6: Reuse values from Rel. 17 for NRx,TEG,I, Nsamples and NRxBeam,i in positioning measurement delay requirements for RRC_IDLE state.

Proposal 7: Accuracy requirements in RRC_INACTIVE state also apply to positioning measurements in RRC_IDLE.  .

Proposal 8: The reporting delay for PRS measurements performed in RRC_IDLE doesn’t need to account for extra delay to transition to RRC_CONNECTED state.

4 References

- R4-2306353 WF on RRM requirements for LPHAP, Huawei, RAN4#106bis-e

- Draft Report of 3GPP TSG RAN WG1 #112bis-e v0.1.0

Annex RAN1/2 agreements on LPHAP

RAN1#112bis-e

Agreement

For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, sequenceID in SRS for positioning configuration is commonly configured across cells within the validity area.

Working assumption

For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, support the following options:

  - Option 1: Pathloss RS is absent in the configuration.

  - Option 2: Pathloss RS is provided in the configuration.

Working assumption

For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support that spatial relation information can be absent or present in the configuration.

Agreement

- For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state:

  - When the spatial relation information is absent in the configuration, the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources;

  - When the spatial relation information is provided in the configuration, it is applicable across the cells within the SRS positioning validity area. Further study the configuration details.

    - FFS: spatial relation information validity criteria, and whether/how to determine UE fallback behavior if validity criteria for spatial relation of the configured RS is not met.

Agreement

For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is absent in the configuration, the UE determines the pathloss RS using a RS resource obtained from the SS/PBCH block of the camping cell that the UE uses to obtain MIB as the pathloss RS.

Agreement

For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is provided in the configuration, support one or multiple of the following alternatives on how to configure pathloss RS (down-selection to be made in RAN1#113 meeting):

- Alt. 2-1: Reuse the configuration of pathloss RS in Rel-17;

  - Reuse the validity criteria of OPLC for SRS transmissions by RRC_INACTIVE UEs in Rel-17, i.e. when the pathloss RS cannot be accurately measured. When the validity criteria of OPLC fail, pathloss is calculated based on the RS resources obtained from SS/PBCH block of the new camping cell that the UE uses to obtain MIB.

- Alt. 2-4: Configure a list of candidate pathloss RSs or candidate cells per SRS for positioning resource set. The UE transmits SRS resources in a SRS resource set using a pathloss RS determined from the candidate pathloss RSs based on its DL measurements on the candidate pathloss RSs;

  - FFS: whether/how to define validity criteria of OLPC and UE fallback behavior if validity criteria fail.

- Alt. 2-5: Reuse the configuration of pathloss RS in Rel-17. The UE only transmits SRS resources in a SRS resource set associated with pathloss RS that can be accurately measured.

  - FFS: whether/how to define validity criteria of OLPC and UE fallback behavior if validity criteria fail.

- Alt. 2-6: Reuse the configuration of pathloss RS in Rel-17;

  - A cell-agnostic DL RS can be the pathloss RS in the validity area and the cell-agnostic DL RS itself.

- Note: UE power consumption should be considered

RAN2#121bis-e

Agreement:

Wait for RedCap progress on extending eDRX cycle (from RAN2 perspective).

Agreement:

The SRS validity area configuration contains a list of cells in which it is valid.  FFS validity timer or if we would depend only on explicit release by the network.

Agreements:

RRCRelease can be used to provide SRS configuration with validity area for use in RRC_INACTIVE.

Agreement:

SRS configuration request can be indicated via Msg3/MsgA transmission.  FFS if the request is in the RRC message or an accompanying MAC CE.

Agreement:

Send a reply LS to SA2 to indicate that from the perspective of RAN2, “low power or high accuracy” positioning is out of the Rel-18 WI scope.