3GPP TSG-RAN WG4 Meeting # 107	R4-2307758

# Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	TP to TR 38.858: Self-interference analysis for FR2 BS

Agenda Item:	8.20.2.2.2

Document for:	Approval

- Introduction

SBFD feasibility study and RF impact from BS aspects were discussed in last meeting and WF for the feasibility from BS aspect was approved in [1].

Agreement:

- The TR section “10.2 Feasibility of FR1 Wide Area BS aspects” shall be further broken-down to harmonize the common understating from RAN4 if possible and summarize the input from companies.

- The same TR section break-down as FR1 WA BS can be applied to FR2-1 BS.

In this contribution, we provide some text proposal for the section 10.5 Self-interference analysis based on our input in [2] and [3].

- Text Proposal

### 10.5.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability and analysis results.

### 10.5.1.1	Summary table for self-interference analysis

Editor's note: This section captures the summary table which is based on self-interference analysis framework.

| FR2 |  |  |  | Huawei |
| --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | 35 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 |
|  |  | Frequency isolation techniques used |  | DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 85~95 |
|  |  | Spatial isolation techniques used |  | Spatial separation between TX/RX panel, with absorbing material and choke structure. |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Less than 0.5 dB loss |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -94~-104 |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | N/A |
|  |  | RF IC techniques and other tech. (before LNA) |  | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -60 ~ -70 |
|  |  | Blocker Suppression at RX |  | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  |  | N/A |  |
|  |  |  |  | Frequency isolation techniques |  |  |  |  |
|  |  |  |  | RX IMD |  | Rx IIP3 capability (dBm) |  |  |
|  |  |  |  |  |  | Rx IM3 contribution (dBm) | negligible |  |
|  |  |  |  | Other RX |  | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | negligible |  |
|  |  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  |  |  | negligible |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 10 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Less than 0.5 dB loss |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | - |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 129 ~ 139 |
| Noise floor ⑩dBm |  |  |  | -88 dBm/40 MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -94 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 129 |
| SBFD configuration |  |  |  | DUD [80,40,80] |
| Guardband assumption (if exist) |  |  |  | Existing SU |
| bandwidth over which suppression is achieved |  |  |  | Several GHz |

### 10.5.1.2	Feasibility study on self-interference

Editor's note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

### 10.5.1.2.1	Huawei

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 10.5.1.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

As shown in the 10.5.1.1, it can be found that the blocking level to RX pannel is weak and should be even lower at each LNA input. And the power at the Hence the IM3 is not a limited factors. Due to the same reason that the blocking level is relatively weak, the other RX impacts due to blocker in Tx sub-band can also be negligible. Hence we think that the evaluation on Self-Interference leakage in gNB RX subband would be sufficient for FR2.

### 10.5.1.2.2	[Company Name]

### 10.5.1.2.3	[Company Name]

### 10.5.1.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on self-interference based on RAN4 agreement.

- Reference

[1] R4-2306005, “WF on implementation feasibility of SBFD”, Samsung

[2] R4-2301426,	“Feasibility study from BS aspect”, Huawei, HiSilicon

[3] R4-2305303, “Discussion on the feasibility of FR2 BS aspects”, Huawei, HiSilicon