3GPP TSG-RAN WG4 Meeting #107	R4-2307933

# Incheon, KR, May 22 – May 26, 2023

Title:	On measurement grid for multi-RX DL 2AoA spherical coverage

Source:	Samsung

Agenda item:	8.3.2

Document for:	Discussion

1.	Introduction

Test method for multi-RX DL RF on 2AoA spherical coverage are converging and test procedure for multiple DCI UE and single DCI UE is almost clear except for several issues depending on RF session conclusion. Given test method is becoming stable, it is time to start the work on preliminary measurement uncertainty, as indicated in the SID [1]

- Develop the related preliminary uncertainty assessments for the test methodologies - FR2 test methods defined in TR 38.810 and TR 38.884 should be used as the baseline. - The tests shall take the test system reuse, test system complexity and test time into account to keep the whole test costs within a reasonable level.

Measurement grid is one of the most critical prerequisite for preliminary uncertainty assessment. In this contribution, we initiate the measurement grid discussion.

2. 	Discussion

In Multi-RX DL core WI, simulation campaign is ongoing to derive the multi-RX DL 2AoA spherical coverage requirement, where very fine measurement grid is used since core requirement does not contain measurement uncertainty [2]:

The following procedure can be used as a guideline for simulation: 1. For one UE implementation 2. For one UE orientation 3. Run EM simulation to obtain per-beam antenna gain patterns - Constant step size is suggested <= 5° - Performance difference between V/H element can be considered - Normalize antenna gain to align with the gain drop between peak EIS and spherical coverage in current spec (option 1 in 2.6) - Other calibration method that described in 2.6 also can be used. 4. For one angular separation 5. For one test grid point in 3D scan - Select beam based on RSRP (or SINR) 5.1 Calculate SINR of AoA+ and AoA- respectively - SINR = P_signal/(Noise + P_interf) Where the P_signal is the power of wanted signal and the P_interf is the power of interference, Noise(dBm)= -174 +10*log10(CBW) +NF, CBW is channel bandwidth, NF =10 - If SINR>=-1, PASS, otherwise, FAIL 5.3a OR combining the results of AoA+ and AoA- 5.3b No logic combination of the results of AoA+ and AoA-, but treat them as two separate points (e.g., arithmetic mean) - Other methods for +/- offset data are not precluded - Companies are encouraging to provide analysis on the pros and cons for each “combination” method 5.4 Add weighting (sin θ or Clenshaw-Curtis Quadrature) 6. Repeat for other test grid point 7. Calculate the spherical coverage percentage 8. Repeat for other angular separation 9. Repeat for other UE orientations 10. Repeat for other UE implementations

From test perspective in OTA test SI, practical measurement grid should be considered with a reasonable measurement uncertainty due to finite measurement grid points.

In legacy 1AoA EIS spherical coverage, measurement grid analysis is provided in TR38.810 and 266 grid points for constant step size grid (corresponding to 15deg step size) are recommended based on a reasonable standard deviation in dB.

G.3.3.2.3 Conclusions According to results in clause G.3.3.2.1 and G.3.3.2.2, the following conclusions can be made: - The EIS spherical coverage measurement to be performed without having to have the beam peak placed on a grid point, e.g., for coarse grids of beam peak searches. - In order to make a reasonable trade-off with measurement uncertainties, it is recommended to use for spherical coverage grids with at least 200 unique measurement points, i.e., - constant density grid with at least 200 grid points: STD of 0.1dB and Mean Error: DL power step size - constant step size grid with at least 266 grid points: STD of 0.12dB and Mean Error: DL power step size

For multi-RX DL 2AoA spherical coverage, the requirement metric has been changed from sensitivity measurement to throughput measurement based on go no-go approach, and the final spherical coverage test result in not in dBm unit but in percentage.

Observation 1:	multi-RX DL 2AoA spherical coverage test result in not in dBm unit but in percentage.

In legacy 1AoA EIS spherical coverage, measurement grid analysis is done by simulating the deviation based on numerous random UE orientation, however, UE orientation itself is part of core-requirement for multi-RX 2AoA case.

Observation 2:	UE orientation itself is part of Multi-RX core-requirement and hence deviation analysis based on numerous random orientation may be not doable.

It seems necessary to further discuss how the measurement grid analysis could be done for multi-RX DL 2AoA spherical coverage. Given random orientation is not feasible, maybe one way to move forward is to compare the simulation results difference between fine grids and coarse grids for companies who is involved in core requirement simulation campaign. And the difference between fine grids and coarse grids from companies could provide a rough level how measurement error due to measurement grid could be. Fine grids are suggested with <=2deg step size, and course grids are suggested to be 15deg step size. Both sin θ weighting and Clenshaw-Curtis weighting should be included in the analysis.

In summary, we have following proposals:

Proposal 1:	RAN4 further discuss how the measurement grid analysis could be done for multi-RX DL 2AoA spherical coverage.

Proposal 2:	companies are encouraged to compare the simulation results difference between fine grids and coarse grids. Fine grids are suggested with <=2deg step size, and course grids are suggested to be 15deg step size.

Proposal 3:	Both sin θ weighting and Clenshaw-Curtis weighting should be included in the measurement grid analysis.

3. 	Conclusion

Observation 1:	multi-RX DL 2AoA spherical coverage test result in not in dBm unit but in percentage.

Observation 2:	UE orientation itself is part of Multi-RX core-requirement and hence deviation analysis based on numerous random orientation may be not doable.

Proposal 1:	RAN4 further discuss how the measurement grid analysis could be done for multi-RX DL 2AoA spherical coverage.

Proposal 2:	companies are encouraged to compare the simulation results difference between fine grids and coarse grids. Fine grids are suggested with <=2deg step size, and course grids are suggested to be 15deg step size.

Proposal 3:	Both sin θ weighting and Clenshaw-Curtis weighting should be included in the measurement grid analysis.

4. 	References

- RP-230814    “Revised SID: Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements”, Qualcomm

- R4-2306604    “WF on FR2-1 UERF requirements for FR2 multi-Rx”, vivo, Apple

- TR38.810