3GPP TSG-RAN4 Meeting #107	R4-2307973

# Incheon, KR, May 22nd - 26th, 2023                                                      Revision of R4-23xxxx

Source:	China Telecom

Title:	TP for TR 38.899 to introduce HPUE SUL configuration CA_n78C_n80A-n84A

Agenda item:	7.20.2

Document for:	Approval

1 Background

This contribution provides text proposal on the NR HPUE SUL configuration CA_n78C_n80A-n84A.

2 Text Proposal

---Start of changes---

## 5.X	CA_n78_n80-n84

### 5.X.1	Configurations for SUL

Table 5.X.1-1: Supported bandwidths per SUL band combination with inter-band CA (two SUL cells)

| SUL band combination with CA | UL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n78C_n80A-n84A | SUL_n78A-n80A SUL_n78A-n84A CA_n78C2 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 of TS 38.101-1 | 0 |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 2: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.X.2	Maximum output power

For UL configuration CA_n78C, the requirement in clause 6.2A.1.1 from 38.101-1 is applicable.

For other SUL configurations, the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

### 5.X.3	Spurious emission band UE co-existence

There is only single UL band in uplink so the requirement for each band in clause 6.5.3.2 from 38.101-1 is applicable.

### 5.X.4	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 is applicable.

Since the 2nd harmonic interference of SUL band n80 may fall into the Rx band n78, the MSD due to harmonic interference should be taken care of. The REFSENS requirements can refer to the MSD due to harmonic interference between SUL band n80 and n78 which has been specified in Table 7.3C.2-2, if harmonic interference need to be considered.

### 5.X.5	∆TIB and ∆RIB values

For CA_n78_n80-n84, the TIB,c and RIB,c values are given in the tables below referring to CA_n1-n3-n78.

Table 5.X.6-1: ΔTIB,c due to SUL (two cells)

| SUL Band combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n78_n80-n84 | 0.8 | 0.6 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands. |  |  |  |

Table 5.X.6-2: ΔRIB,c due to SUL (two cells)

| SUL Band combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n78_n80-n84 | 0.5 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands. |  |  |  |

---End of changes---

Reference

- RP-230309 “Revised WID: Rel-18 High power UE (power class 1,5 and 2) for a single FR1 NR TDD band in UL of NR inter-band CA/DC combinations with/without NR SUL (supplementary uplink) with y bands downlink (y=2,3,4,5,6) and x bands uplink (x=1,2)”, China Telecom, RAN#99