3GPP TSG-RAN WG4 Meeting #107	R4-2308047

Incheon, ROK, 22 – 26 May, 2023

Source:	ZTE Corporation

Title:	Discussion on collisions between gaps and priority rules

Agenda item:	8.26.2.2

Document for:	Approval

Introduction

At RAN 95 meeting the revised WI “Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR” [1] was approved.  In the last meeting, we discussed the collisions between gaps and priority rules, and the outcomes were captured in [2]. Based on the outcomes, the following issues need to be further discussed.

- MUSIM gap priority configuration

- On collision between different MUSIM gaps

- On collision between MUSIM and legacy gaps

- In this paper we will continue to discuss the related issues and provide our views on the above issues.

Discussion

For MUSIM procedure[3], SIM A works on NW A and SIM B works on NW B. In general, UE is in RRC_CONNECTED state and in RRC_IDLE/_INACTIVE state on NWA and NW B, respectively. UE needs to request the certain MUSIM gaps from NW A in order to monitor the NW B actives, such as paging monitoring, measurements and system information reading, etc.

## Figure 1: MUSIM procedures

UE can request the proper MUSIM gaps from NW A and UE should provide the UAI to NW A and NW A may obtain the terminal request so as to provide the required configurations for MUSIM gaps.

MUSIM gap priority configuration

2.1.1  Priority rules for MUSIM gaps

At RAN4#106bis-e meeting, RAN4 has reached the agreements towards the priority rules for MUSIM gaps as below:

Issue 2-1-5: Priority setting for aperiodic MUSIM gaps Agreements from GTW - Option 1 (QC, Nokia, vivo, Charter, Xiaomi) - The priority level of aperiodic MUSIM gap can be configured by NW A - If the priority level is not configured by NW A then the aperiodic MUSIM gap by default has the highest priority level - The aperiodic MUSIM gap priority level can be optionally requested by UE from NW A - Option 2 (MTK, ZTE, Huawei, Apple, Ericsson, OPPO): - Aperiodic MUSIM gap by default has the highest priority level. - The gap priority level is not explicitly configured by the NW Agreements: No Candidate options: Recommendations: try to down select between option 1 and 2

According to the agreements in #106bis-e meeting, we deeply consider the  priority for MUSIM gaps. As we can see in the agreements we reached, companies admitted that the different priorities for each MUSIM gap. RAN2 has discussed that the MUSIM gaps contain 4 kinds of gaps including 3 periodic gaps and 1 aperiodic gap. As for the periodic gaps, they may have different purposes, such as:

- SSB detection

- Measurement

- Paging

As for the aperiodic gap, most companies agreed that the aperiodic gap should always have higher priority than the other NW-A’s legacy gaps in the last meeting. We deem that aperiodic gap doesn’t need to be configured by NW-A or UE with the high priority than other legacy gaps since it is the one-shot gap that only has one occasion that is more urgent than the periodic gap and NW-A’s legacy gaps and in RAN4 105#meeting, we considered whether the priority for MUSIM gap should be determined by NW-A or UE itself, the two choices were analyzed:

If priority is determined by UE: UE will report the requisite priority when requesting MUSIM gap based on the figure 1 and NW-A can receipt or reject such request. If NW-A  choose to configure the aperiodic gap, it is improper to enable it to own the lower priority since we mentioned before which AP gap only has one occasion and if the AP gap configures with the lower priority, the priority handling rule will apply for it and AP gap will be dropped.

If priority is determined by NW-A: UAI (i.g.MGRP, offset, MGL etc. ) is a critical component for NW-A configuring priority since RAN2 disagrees with that NW-A knows about the purpose or usage for MUSIM gap.

Above all, we prefer option 2 and totally agree that aperiodic gap should not be configured with the higher priority via NW-A or UE since it needs to have the default higher priority than other NW-A’s legacy gaps and periodic gaps in MUSIM gaps. For option 1, there is actually no unreasonable point here. If the first bullet is used with the overall rule, the only strange thing is that if the AP gap priority is low, it will be dropped when encountering a drop, so the AP gap will never be found for a period of time, and its uniqueness has not been fully utilized. Therefore, doing down selection in options 1 and 2 is more biased towards option 2, maintaining its uniqueness (one shot gap) and retaining AP gap (default higher priority).

Observation 1:  It is improper to enable AP gap to own the lower priority since it only has one occasion and if the AP gap configures with the lower priority, the priority handling rule will apply for it and AP gap will be dropped.

- Proposal 1: The Aperiodic gap need to own the default higher priority  than other NW-A’s legacy gap and periodic MUSIM gaps.

On collision between different MUSIM gaps

Issue 2-2-1: Definition of the collision between different MUSIM gaps - Proposals - Option 1: The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap when priority rules are used to handle the collision between MUSIM gaps (Qualcomm Apple MTK xiaomi Ericsson CMCC ZTE oppo vivo) - Option 2: Postpone the discussion till issue 2-2-2 is stable (Huawei Qualcomm ZTE) - Option 3: A collision between MUSIM gaps means a physical overlap in time domain between two MUSIM gaps and RAN4 does not define ‘proximity’ for collisions between MUSIM gaps. (Nokia) Recommended WF Continue discussion

In 104bis_e# meeting, RAN4 has agreed to reuse proximity condition for defining the collision between MUSIM gap and legacy MGs (only Type-1 and Type-2 MG). In our view, same definition can still be applied here. So we consider that the option 1 is fine to us.  We have two cases:

- The UE may use two gaps to measure different MOs at the same frequency layer, it is likely to keep both gaps.

- The UE may use two gaps to measure different MOs at the different frequency layers, or the conditions for keeping both gaps which are collided are not satisfied, under this scenario the priority handling rule may be used, that is, keeping higher priority gap and the lower one should be dropped.

- Based on the two cases, the definition for collision between different MUSIM gaps should be clarified so as to  adopt the proper methods to handle it.

- Proposal 2:  The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap when priority rules are used to handle the collision between MUSIM gaps.

Issue 2-2-2: Solutions for collision between different MUSIM gaps - Proposals - Option 1: Priority based solution is used for collision between different MUSIM gaps when these collided MUSIM gaps are assigned with different priority levels (vivo Apple Qualcomm) - Option 2: Priority based solution is used for collision between different MUSIM gaps (MTK xiaomi oppo Charter) - Option 2-1: The priority handling rule shall be used if it collides with the periodic gaps (except the paging gap) (ZTE) - Option 3: Keep solution (keep all collided MUSIM gap) is used when MUSIM gaps have same priority (Apple Qualcomm oppo vivo) - Option 3a: The overlapping MUSIM gap occasions under scenario of option 3 shall be merged. (Apple) - Option 3b: whether multiple MUSIM gaps can be assigned the same priority level or not subject to UE capability (Qualcomm vivo) - Option 3c: Up two periodic MUSIM gaps can be configured with the same priority and inform such the configuration to RAN2 (oppo) - Option 4: MUSIM gap ‘keep rule’ will be applied in some certain scenarios, such as Paging monitoring and AGC (Ericsson) - Option 4-1: The kept/merged solution is used if the second gap in collision is paging gap (ZTE) - Option 5: Keep solution (keep all collided MUSIM gap) is used when different MUSIM gaps collide (Huawei) - Option 6: For keep solution, UE shall under defined conditions not drop a colliding MUSIM gap of lower priority, provided the UE perform all actions related to the colliding MUSIM gaps of higher priority or priorities. RAN4 shall define these conditions. (Nokia) Agreements: No Recommendations: Consider and down-select from the following options at the next meeting: - Option 1: Priority based solution is used for collision handling between different MUSIM gaps - Option 2: Keep solution (keep all collided MUSIM gap) is used for collision handling between different MUSIM gaps - Option 3a: Use both Priority based solution and Keep solution for collision handling between different MUSIM gaps - Priority based solution is used when collided MUSIM gaps have different priority levels - Conditions when Keep solution is used are FFS. - Option 3b: Use both Priority based solution and Keep solution for collision handling between different MUSIM gaps - Conditions when priority based solution is used and conditions when Keep solution is used are FFS.

When the collision occurs between the different MUSIM gaps, some companies proposed that the priority handling rule should be used, some companies considered that kept/merged solution should be used for collision between different MUSIM gaps. From my perspective, each has its own merits and demerits. As we discussed before, aperiodic gap shall have the default higher priority and paging gap is the key MUSIM gap. So based on above, we think the different method should be adopted in specific cases, that is, the two periodic MUSIM gaps collide except the paging gap,and one of gaps is aperiodic gap collides with other periodic MUSIM gap except paging gap if we adopt the priority rule and keeping both MUSIM gaps are collision between MUSIM gap for SSB detection so as to guarantee the paging syn and MUSIM gap for paging reception, we propose the SSB before paging and paging gap could be kept when different MUSIM gaps collide. So the remaining issue is how to handle the collision between aperiodic gap and paging gap. Some companies proposed that UE will request the MUSIM gaps with a reasonable purpose based on gap scheduling design. It’s impossible to request an aperiodic gap which is colliding with paging gap. I deem it is an effective way to avoid the collision between them.

- Proposal 3:

- The aperiodic gap which has higher priority than other periodic gaps, the priority handling rule shall be used if it collides with the periodic gaps (except the paging gap) .

- The paging gap should not be dropped, the kept/merged solution is used if the second gap is paging gap.

- Otherwise, the priority handling rule will be used among MUSIM gaps.

On collision between MUSIM gaps and legacy gaps

Issue 2-3-2: Solutions for collision between MUSIM gap and Type-1 MG or gap configured without priority - Proposals - P1: When a MUSIM gap collides with a legacy MG, requirements shall not apply if any one of the collided gaps is not assigned a priority. (vivo Apple xiaomi oppo) - Against: (Nokia) - P2: No requirements apply if the two gaps are configured with same MGRP (Huawei Ericsson) - Against: (Nokia) - P3: RAN4 to prioritize the gap with longer MGRP for the following MUSIM collision scenarios: Any of the collision gaps is Type-1 MG; NW-A doesn’t configure a priority associated with any of the collision gaps. (Ericsson Huawei ZTE MTK) - P4: MUSIM gaps are assumed to have higher priority than a Type-1 MG when either MUSIM gaps or Type-1 MG (or both) are not assigned priorities by the network. (Qualcomm) - Against: (Huawei xiaomi) - P5: Introduce priority for Type-1 MG when MUSIM gaps are configured when also having Type-1 measurement gaps allocated. (Nokia) - Against: (Qualcomm Huawei xiaomi ) - P6: The sharing rule solution could be considered. (xiaomi) - Against: (Qualcomm Huawei) Agreements: No Candidate options: Recommendations: Suggest to focus on P1, P2 and P3 since they have relative more supporters and less objections.

Several companies argued that the priority of MUSIM gap should always be higher than other MGs. On the contrary, several companies thought that the fix-based priority has the inflexibility to UE. As for this discussion, we need to consider the practical cases. For example, the mobility status of UE should be considered in determining the priority of MUSIM gaps (eg.Type-1 MG configured to perform the inter-frequency measurement for handover). NW-A may want to deprioritize the MUSIM gap than legacy MG no matter what MUSIM gap is used for when UE is at cell edge and mobility measurement is time critical. And in another case that MG is used for positioning for any emergency service, etc. Based on above, it makes no sense to configure the MUSIM gap with higher priority in several specific and urgent cases and NW-A may be willing to prioritize the MG than MUSIM gap.

Type-1 MG is the legacy gap configured via GapConfig without suffix, and Type-2 MG is the legacy gaps configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17. Note that Type-1 MG has no priority nor association, whereas Type-2 MG has both.

For collision between the MUSIM gaps and the Type-2 gaps, in previous meeting [5] we reached that the priority-based handing rules in concurrent measurement gaps can be used in this scenario. However, the same solution can not be used in the collision between the MUSIM gaps and the Type-1 gaps. When the MGRP of Type-1 MG is large enough especially lager than the MUSIM gaps, the lager MGRP should be prioritized since it can avoid no measurement opportunity for one configured gap.

Proposal 4: Collision is be handled based on the MGRP of the collided gaps (especially for Type-1 gaps).

Conclusion

In this paper we provided our views on collision handling related to MUSIM gaps.

Observations:

- Observation 1:  It is improper to enable AP gap to own the lower priority since it only has one occasion and if the AP gap configures with the lower priority, the priority handling rule will apply for it and AP gap will be dropped.

Proposals:

- Proposal 1: The Aperiodic gap need to own the default higher priority  than other NW-A’s legacy gap and periodic MUSIM gaps.

- Proposal 2:  The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap when priority rules are used to handle the collision between MUSIM gaps.

- Proposal 3:

- The aperiodic gap which has higher priority than other periodic gaps, the priority handling rule shall be used if it collides with the periodic gaps (except the paging gap) .

- The paging gap should not be dropped, the kept/merged solution is used if the second gap is paging gap.

- Otherwise, the priority handling rule will be used among MUSIM gaps.

Proposal 4: Collision is be handled based on the MGRP of the collided gaps (especially for Type-1 gaps).

- References

- RP-221018, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation

- R4-2306357, WF on NR Dual Tx/Rx Multi-SIM,Vivo