3GPP TSG-RAN WG4 Meeting # 107		R4-2308067

# Incheon, KR, May 22 – May 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On clarification to the switching period location of the UL Tx Switching

Agenda item:	10.2.1

Document for:	Discussion and Approval

# Introduction

This contribution discusses an ambiguity of the location of the UL Tx switching period in relation to the incoming LS from RAN1 [1].

During the RAN4#106bis meeting a set draftCRs were endorsed (R4-2306647, R4-2306648, R4-2304950, R4-2306654) addressing the issue for Rel-17 and Rel-17. For Rel-18 the discussion will continue.

- Issue with the Uplink Tx Switching period location

## Ambiguity in the current TS 38.101-1

The TS 38.214 subclause 6.1.6 states that “the conditions under which the switching gap may be present and the location of the switching gap are defined for each of the cases in clauses 6.1.6.1, 6.1.6.2, and 6.1.6.3 respectively.”, but the clauses 6.1.6.1, 6.1.6.2 and 6.1.6.3 only define the duration of the switching gap, not the exact location.

For CA and SUL-based UL Tx switching the TS 38.101-1 subclauses 6.3A.3.3 and 6.3C.3 define the location of the switching gap for CA and SUL-based UL Tx Switching with a set of figure pairs that relate to the configuration of UplinkTxSwitchingPeriodLocation.

The switching periods described in Figure 6.3A.3.3.5-1a and Figure 6.3A.3.3.5-1b are located in either NR band A or band B as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod2T2T. Figure 6.3A.3.3.5-1a: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band A Figure 6.3A.3.3.5-1b: Time mask for switching between one carrier in band A and two contiguous carriers in band B, where the switching period is located in band B

Observation 1: The figures 6.3A.3.3.5-1a and -1b of 38.101-1 place a time instant at the boundary of slot/sub-slot, but it is not obvious what constitutes as the slot/sub-slot boundary.

Observation 2: Contrary to the statement in clause 6.1.6, the TS 38.214 does not define the time-location of the switching period. This was one of the triggers to the RAN1 discussions and lead to the LS to RAN4

Observation 3: For CA and SUL-based switching the TS 38.101-1 defines whether the switching period is located in time before or after a switching reference point in time, but the reference point used is a slot/sub-slot boundary, which is not clear.

The switching location cannot be fixed at a slot boundary as the uplink transmission may start at any point in the slot. Figure 1 shows a typical FDD 15kHz and TDD 30 kHz CA with TDD pattern of 10:2:2. E.g. with 140 us switching period, the TDD uplink starts at symbol#12, and every second time the FDD uplink after the switch alternates between start symbol #2 and #9.

Figure 1: FDD15-TDD30 with DDDSU (10:2:2), switching period on the FDD UL (140 us switching period)

The key question is that what is the role of the “uplinkTxSwitchingPeriodLocation” configuration when the scheduler creates a sufficient gap between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. E.g. in the above setup, what difference does the configuration of the “uplinkTxSwitchingPeriodLocation” make if the FDD slot is scheduled to stop transmission at symbol #10 at slot #1 and resume transmission on symbol #9 in slot #2 etc. i.e. the uplink allocations on the two uplink carriers are as in Figure 2.

Figure 2: FDD15-TDD30 with DDDSU (10:2:2), scheduler leaves a 140 us gap between the transmissions (140 us switching period)

If in the case of Figure 2 the uplinkTxSwitchingPeriodLocation configures the switching gap on the FDD carrier, if the switching gap is understood to start at the nominal start of the UL transmission, that would lead to cutting the first symbol(s) of the FDD transmission after the switch. This interpretation is depicted in Figure 3. This would be a very unfortunate interpretation as the gNB could not avoid the scheduled uplink to be cut no matter what it did.

Figure 3: uplinkTxSwitchingPeriodLocation on the FDD carrier – the first two symbols of the FDD UL transmission after the switch and last twi symbols before the switch are cut

Similarly, if on the case of Figure 2 the uplinkTxSwitchingPeriodLocation configures the switching gap on the TDD carrier, if the switching gap is understood to start at the nominal start of the UL transmission, that would lead to cutting the first symbol(s) of the TDD transmission after the switch and last symbols before the switch. This interpretation is depicted in Figure 4. This would be a very unfortunate interpretation as the gNB could not avoid the scheduled uplink to be cut no matter what it did.

Figure 4: uplinkTxSwitchingPeriodLocation on the TDD carrier – the first four symbols of the TDD UL transmission after the switch and last four symbols before the switch are cut

Observation 4: The switching period location as depicted in Figure 3 and Figure 4 is obviously wrong. When the scheduler leaves a gap between the end of the switch-from transmission and the start of the switch-to transmission it would be pointless to nevertheless impose a switching period on a scheduled part of the transmission.

Considering the issues and ambiguity described in the previous paragraphs, clarifications have been proposed added as shown in the endorsed draftCRs in R4-2306647 and R4-2306648.

Observation 5: RAN4 have endorsed draft CRs  to 38.101-1 at RAN4#106bis addressing the issues and ambiguity raised by RAN1.

This meeting CRs corresponding to the indorsed draft CRs have been submitted for approval.

Proposal 1: RAN4 shall agree to the CR to 38.101-1 Rel-16 in [2] and Rel-17 in [3].

By normal practice when CRs to earlier releases is agreed/endorsed corresponding Cat-A CRs are to be agreed/endorsed for later releases. In this specific case the latest Rel-18 were agreed needed further discussion. Nevertheless, to ensure alignment between the specification releases we believe that the same changes applied for Rel-16/17 should be mirrored to also Rel-18. This with the understanding that it is to serve as the baseline for further discussion.

Proposal 2: RAN 4 shall agree Cat-A CRs for 38.101-1 Rel-18 in [4]. This with the understanding that it is to serve as baseline for further discussion and potential additional modifications

## Ambiguity in the current TS 38.101-3

The RAN1 LS [1], states that both TS 38.101-1 for NR SA and TS 38.101-3 for EN-DC needs clarification.

For EN-DC-based UL Tx switching the TS 38-101-3 subclause 6.3B.4 defines the switching period location relative to the E-UTRAN subframe boundary so that the switching period is always on the NR side of the E-UTRAN subframe boundary.

The switching periods described in Figure 6.3B.4.1-1 are only located in NR carrier, and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod. Figure 6.3B.4.1-1: Time mask for switching between E-UTRA UL carrier and NR UL carrier, where the switching period is located in NR carrier

Observation 6: For EN-DC based switching the TS 38-101-3 unambiguously defines the switching period location relative to the E-UTRAN subframe boundary so that the switching period is always on the NR side of the sub-frame boundary. However, it is not clear what happens if the NR transmission doesn’t start right at the LTE sub-frame boundary.

Considering the issues and ambiguity described in the previous paragraphs, clarifications have been proposed added as shown in the endorsed draftCRs in R4-2304950 and R4-2306654.

Observation 7: RAN4 have endorsed draft CRs to 38.101-3 at RAN4#106bis addressing the issues and ambiguity raised by RAN1.

This meeting CRs corresponding to the indorsed draft CRs have been submitted for approval.

Proposal 3: RAN4 shall agree to the CR to 38.101-3 Rel-16 in [5] and Rel-17 in [6].

By normal practice when CRs to earlier releases is agreed/endorsed corresponding Cat-A CRs are to be agreed/endorsed for later releases. In this specific case the latest Rel-18 were agreed needed further discussion. Nevertheless, to ensure alignment between the specification releases we believe that the same changes applied for Rel-16/17 should be mirrored to also Rel-18. This with the understanding that it is to serve as the baseline for further discussion.

Proposal 4: RAN 4 shall agree Cat-A CRs for 38.101-1 Rel-18 in [7]. This with the understanding that it is to serve as baseline for further discussion and potential additional modifications

# Conclusion

In this paper we make the following observations and proposals:

Observation 1: The figures 6.3A.3.3.5-1a and -1b of 38.101-1 place a time instant at the boundary of slot/sub-slot, but it is not obvious what constitutes as the slot/sub-slot boundary.

Observation 2: Contrary to the statement in clause 6.1.6, the TS 38.214 does not define the time-location of the switching period. This was one of the triggers to the RAN1 discussions and lead to the LS to RAN4

Observation 3: For CA and SUL-based switching the TS 38.101-1 defines whether the switching period is located in time before or after a switching reference point in time, but the reference point used is a slot/sub-slot boundary, which is not clear.

Observation 5: RAN4 have endorsed draft CRs  to 38.101-1 at RAN4#106bis addressing the issues and ambiguity raised by RAN1.

Observation 4: The switching period location as depicted in Figure 3 and Figure 4 is obviously wrong. When the scheduler leaves a gap between the end of the switch-from transmission and the start of the switch-to transmission it would be pointless to nevertheless impose a switching period on a scheduled part of the transmission.

Proposal 1: RAN4 shall agree to the CR to 38.101-1 Rel-16 in [2] and Rel-17 in [3].

Proposal 2: RAN 4 shall agree Cat-A CRs for 38.101-1 Rel-18 in [4]. This with the understanding that it is to serve as baseline for further discussion and potential additional modifications

Observation 6: For EN-DC based switching the TS 38-101-3 unambiguously defines the switching period location relative to the E-UTRAN subframe boundary so that the switching period is always on the NR side of the sub-frame boundary. However, it is not clear what happens if the NR transmission doesn’t start right at the LTE sub-frame boundary.

Observation 7: RAN4 have endorsed draft CRs to 38.101-3 at RAN4#106bis addressing the issues and ambiguity raised by RAN1.

Proposal 3: RAN4 shall agree to the CR to 38.101-3 Rel-16 in [5] and Rel-17 in [6].

Proposal 4: RAN 4 shall agree Cat-A CRs for 38.101-1 Rel-18 in [7]. This with the understanding that it is to serve as baseline for further discussion and potential additional modifications

# References

- R1-2302198/R4-2304008, LS on Rel-16 UL Tx Switching period, RAN1

- R4-2308068, Rel-16 CR to 38 101-1 for Clarification of UL Tx Switching, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Huawei, Hisilicon, Vivo

- R4-2308069, Rel-17 CR to 38 101-1 for Clarification of UL Tx Switching, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Huawei, Hisilicon, Vivo

- R4-2308070, Rel-18 CR to 38 101-1 for Clarification of UL Tx Switching, Nokia, Nokia Shanghai Bell

- R4-2308071, Rel-16 CR to 38 101-3 for Clarification of UL Tx Switching, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Huawei, Hisilicon, Vivo

- R4-2308072, Rel-17 CR to 38 101-3 for Clarification of UL Tx Switching, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Huawei, Hisilicon, Vivo

- R4-2308073, Rel-18 CR to 38 101-3 for Clarification of UL Tx Switching, Nokia, Nokia Shanghai Bell