3GPP TSG-RAN WG4 Meeting #107 	 R4-2308106

Incheon, KR, May 22 – May 26, 2023

Title:	RF simulation parameters for MPR/PAR evaluations

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	8.28.2.2

Document for:	Approval

# 1	Introduction

This contribution relates to a work item agreed in RAN#94-e, namely “Further NR coverage enhancements” [1]. We consider power domain enhancements and the following objectives captured in the WID:

- Study and if necessary specify following power domain enhancements

  - […]

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

In this paper we discuss the further simulation assumptions for studying the UE RF performance for MPR/PAR objective. The parameters shown in Table 1 and Table 2 were agreed in RAN4 #105 [7].

Table 1 Simulation parameters for FR1

| Carrier frequency | 4GHz 700 MHz (optional) |
| --- | --- |
| Channel BW | Case 1: 20 MHz Case 2: 100 MHZ |
| SCS | Case 1: 15/30/60 kHz Case 2: 30 kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | - Pi/2 BPSK - QPSK |
| Waveform | DFT-S-OFDM |
| Allocation type | Sweep over the channel |
| Extension factors | 0-0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | - 3-tap, FD implementation - (0.335 1 0.335) - (0.28 1 0.28) - 2-tap: (1, 0.28) for FDSS QPSK w/o SE - Truncated RRC - No filter (reference case) NOTE1 |
| Power class | PC 3 |
| NOTE 1: A reference waveform) which meet the existing requirements for gain evaluation |  |

Table 2 Simulation parameters for FR2

| Carrier frequency | 28GHz |
| --- | --- |
| Channel BW | 400MHz |
| SCS | 120kHz |
| DMRS config | ZC, 2 symbols |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Allocation type | Sweep over the channel |
| Extension factors | 0- 0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | - 3-tap, FD implementation - (0.335 1 0.335) - (0.28 1 0.28) - 2-tap: (1, 0.28) for FDSS QPSK w/o SE - Truncated RRC - No filter (reference case) NOTE1 |
| Power class | PC 3 |
| NOTE 1: A reference waveform which meet the existing requirements for gain evaluation |  |

In addition to parameters shown in Table 1 and Table 2, the following agreements were made in RAN4 #104-e [5]

Agreement:

- RAN4 follows below RAN1 agreements and focus on prepare for RF simulations

  - Establish evaluation parameters and side-conditions if any for both transparent and non-transparent schemes

    - The parameters and side-conditions will be updated if needed according to RAN1 input

    - Share the agreements with RAN1 that could affect RAN1 link level simulation

  - RAN4 can perform evaluations without RAN1 input for both transparent and non-transparent schemes

    - No discussion on simulation results of non-transparent scheme at least in RAN4#105

Many agreements impacting the RF simulation assumptions were made in also RAN4 #104bis-e and RAN4 #106.

Agreement:

- For performance evaluation, consider symmetric extension for FDSS with spectrum extension. If consideration of asymmetric extension is needed or not is discussed depending on RAN1 input.

Agreement:

- QPSK is the targeted modulation for further coverage enhancements

  - At least for simulation study

- RAN4 shall prioritizes DFT-S-OFDM as a solution for coverage enhancements

  - FSS on CP-OFDM if companies can show gains

- RAN4 shall evaluate both FR1 and FR2 scenarios.

- RAN4 shall not consider other channels and signals (than PUSCH and the associated DMRS)

- RAN4 shall not consider intra band UL CA scenario in Rel-18 WI

Finally, the following agreements were made in RAN4 #106bis-e.

Agreement:

- Use the following equation as the baseline

  - Net Gain [dB]=(Pout_(Filter i)-Pout_Ref )+(SNR10%_Ref-SNR10%_(Filter i) )

  - Where Pout_(Filter i) is the achieved (TX) output power of the filtered  waveform being compared against the reference, Pout_Ref is the output power of the reference, SNR10%_Ref is the required SNR to achieve 10% BLER with the reference, and SNR10%_(Filter i) is the required SNR to achieve 10% BLER using the filtered waveform being compared against the reference.

- Use the following equation when providing the results for net gain

  - Net Gain [dB]=-DOPR_M0 +(SNR10%_Ref-SNR10%_(Filter i) )

Agreement:

- For evaluations, RAN4 should consider the spectrum extension factor (α) as: 0 (reference), 0.25, 0.111 (1/9), 0.125 and 0.375)

In addition to RAN4 agreements, the simulation parameters have been discussed in RAN1. Some of them impact also in RAN4 side [8].

# 2	Discussion

As shown in Section 1, there has been considerable progress in defining the evaluation assumptions in both RAN1 and RAN4. Based on those, the assumptions behind LLS (RAN1) and RF simulations (RAN4) seem to be well aligned.

On top of those, RAN4 #106bis-e made the following agreement:

Agreement:

- RAN4 agrees to analyse provided simulation results as attached to this WF via Excel and provide conclusions based on these at RAN4#107.

  - It is expected a decision on selecting the Rel-18 MPR/PAR reduction solution(s) according to the WID can be taken at RAN4#107

Based on the progress until now, we make the following observation, and proposal:

Observation 1: RF Simulation parameters are well aligned between RAN1 & RAN4, as well as between different companies.

Proposal 1: No further discussion is needed for RF simulation parameters for MPR/PAR evaluations

# 3.	Conclusion

In this paper we discuss simulation parameters for RF simulations. Based on the discussion we make the following observation and proposal:

Observation 1: RF Simulation parameters are well aligned between RAN1 & RAN4, as well as between different companies.

Proposal 1: No further discussion is needed for RF simulation parameters for MPR/PAR evaluations

# References

- RP-213579, “New WI: Further NR coverage enhancements”, China Telecom

- R4-2215515, “Enhancements to reduce MPR/PAR”, Nokia, Nokia Shanghai Bell

- R4-2217745, “WF on enhancements to reduce MPR/PAR”, Nokia, Nokia Shanghai Bell

- R1-2210326, “Final FL summary of power domain enhancements (AI 9.14.2)”, Moderator (Nokia, Nokia Shanghai Bell)

- R1-2008703, “Discussion on approaches and solutions for NR PUSCH coverage enhancement”, Nokia, Nokia Shanghai Bell

- R4-2308108, “RF simulation results for non-transparent schemes”, Nokia, Nokia Shanghai Bell

- R4-222084, “LS on RF simulation parameters to be used in RAN4 for power domain enhancements”

- R1-2212917, “LS to RAN4 for further information on RAN1 assumptions for LLS performance evaluation of MPR/PAR reduction solutions”