# 3GPP TSG-RAN WG4 Meeting # 107	                                  R4-2308176

# Incheon, KR, May 22 – May 26, 2023

Source:	ZTE Corporation

Title:	TP for TR38.718-03-01_3DL_2UL CA_n8A-n39A-n41A

Agenda Item:	7.11.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/2UL CA_n8A-n39A-n41A in TR38.718-03-01. Note that the 3DL/1UL CA_n8A-n39A-n41A have already been defined in the spec.

- Reference

- [1] TR38.718-03-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for3 bands DL with x bands UL (x=1,2),v0.5.0

- Text Proposal

----- Start of TP -----

  - 5.x			n8A-n39A-n41A

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| n8A-n39A-n41A | n8, n39, n41 |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n39A-n41A | CA_n8A-n39A CA_n8A-n41A CA_n39A-n41A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |

      - 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n39-n41, the TIB,c and RIB,c values have already been defined in the spec.

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n8A-n39A, CA_n8A-n41A and CA_n39A-n41A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

No IMDx issue caused by n8+n39 fall into the its own band n41Rx;

No IMDx issue caused by n39+n41 fall into the its own band n8 Rx;

No IMDx issue caused by n8+n41 fall into the its own band n39 Rx.

      - 5.x.2.2	REFSENS requirements

In terms of the co-existence studies, there are no additional MSD requirements.

----- End of TP -----