3GPP TSG-RAN4 Meeting #107												R4-2308194

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda Item:	8.20.2.1

Source: 	CMCC

Title: 	study on SBFD co-existence simulation assumption

Document for:	Discussion

# 1. Introduction

In last meeting, a WF on simulation assumption for adjacent co-existence study has been approved [1]. The timeline for co-existence simulation is listed as below, it’s noted RAN4 #108 meeting is the last meeting for simulation results. In this meeting, we show our initial simulation results.

| Date | RAN4 meeting | Target for high priority scenario |
| --- | --- | --- |
| 2023-04 | RAN4#106bis | - Deadline for official calibration phase, note 1 - start collecting co-ex study results |
| 2023-05 | RAN4#107 | - deadline for completeness of all assumptions - collecting co-ex study results |
| 2023-08 | RAN4#108 | - deadline for collection of simulation results - conclude co-existence results i.e. ACIR |
| 2023-10 | RAN4#108 bis | - final results check and summarizing |
| 2023-11 | RAN4#109 | - TR drafting |
| Note 1: companies that doesn’t show calibration results until this meeting could also provide final simulation results in future meeting but have to company with calibration results to confirm their simulation results are aligned with other companies. Note 2: if no simulation result is received in RAN4 #108, corresponding scenario would be skipped in this SI or show analysis and conclusions based on the results from TR 38.828 |  |  |

.

# 2. Discussion

Following show the simulation cases and scenarios for SBFD for information.

Simulation cases for SBFD

| Victim | Aggressor | Figures: Aggressor(left) and Victim(right) | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- |
| NR TDD DL | SBFD (DU) | Case 1 | NR TDD DL | High |
| NR TDD UL | SBFD (DU) | Case 2 | NR TDD UL | Low |
| SBFD (DU) | NR TDD DL | Case 3 | No system in adjacent channel | High |
| SBFD(DU) | NR TDD UL | Case 4 |  | Low |

Table 2.1-1: Scenarios for SBFD co-ex study

| FR | Scenario No. | Deployment Scenario1 (Aggressor -> Victim) | Priority |
| --- | --- | --- | --- |
| FR1 (4GHz) | 1 | Urban Macro -> Urban Macro | High |
|  | 2 | Urban Hotspot -> Urban Hotspot | Note 4 |
|  | 3 | Indoor -> Indoor | Low |
|  | 4 | UMa-to-UMi | Note 5 |
| FR2 (30GHz) | 5 | Urban Macro -> Urban Macro | High |
|  | 6 | Urban Hotspot -> Urban Hotspot | Note 4 |
|  | 7 | Urban Micro -> Urban Micro | Low |
|  | 8 | Indoor -> Indoor | Low |
| Note 1: The Urban Macro is agreed as baseline scenario for SBFD co-ex study with high priority in RAN4#104-e, while it does not preclude other scenarios. Note 2: The Urban Hotspot uses the same assumption as Urban Macro, except that Urban Macro uses random dropping method for UE while Urban Hotspot uses cluster-based dropping method for UE. Both random dropping and cluster-based dropping for calibration. Note 3: Consider Urban Macro scenario first for calibration purpose. Note 4: Companies are encouraged to provide simulation results for Urban Hotspot scenario as 2nd priority. [Editor’s Note: Agreement 2.2.1 of R4-2302888] Note 5: Companies also encouraged to simulate Uma-to-UMi co-existence scenario as 2nd priority. [Editor’s Note: Agreement 2.2.3 of R4-2302888] |  |  |  |

Following show our initial simulation results for SBFD. Due to limited simulation period, we will further update our simulation results in next meeting or in this meeting.

## FR1

### Simulation case 1

1. SBFD DL  TDD DL

1.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -30dB | - 20dB | - 10dB | Relative ACIR | +10dB | +20dB | … |  |  |
| FR1 4GHz Uma-Uma | CMCC | 1 SBFDDL>TDDDL | 5% | 93.32 | 67.76 | 28.89 | 4.813 | 0.478 | 0.048 |  |  |  |
|  |  |  | 50% | 15.83 | 1.759 | 0.125 | 0.010 | 0.0009616 | 9.517e-05 |  |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

1.2. Final simulation results plot

2. SBFD UL  TDD DL

2.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -30dB | - 20dB | - 10dB | Relative ACIR | +10dB | +20dB | … |  |  |
| FR1 4GHz Uma-Uma | CMCC | 1 SBFDUL>TDDDL | 5% | 0.345 | 0.033 | 0.003 | 0.0003264 | 3.264e-05 | 3.264e-06 |  |  |  |
|  |  |  | 50% | 0.0012127 | 0.0001186 | 1.176e-05 | 1.176e-06 | 1.176e-07 | 1.176e-08 |  |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

2.2. Final simulation results plot

### Simulation case 2

1. SBFD DL  TDD UL

1.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -10dB | Relative ACIR | +10dB | +20dB | +30dB | +40dB | +50dB |  |  |
| FR1 4GHz Uma-Uma | CMCC | 2 SBFDDL>TDDUL | 5% | 100 | 96.97 | 68.26 | 21.51 | 2.825 | 0.296 | 0.03 |  |  |
|  |  |  | 50% | 35.221722 | 6.0616167 | 0.6620113 | 0.0668615 | 0.0066935 | 0.0006694 | 6.694e-05 |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

1.2. Final simulation results plot

2. SBFD UL  TDD UL

2.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -10dB | - 5dB | Relative ACIR | +5dB | +10dB | +15dB | +20dB |  |  |
| FR1 4GHz Uma-Uma | CMCC | 2 SBFDUL>TDDUL | 5% | 21.595193 | 8.6398213 | 2.9547854 | 0.9694374 | 0.3102962 | 0.0985054 | 0.0311885 |  |  |
|  |  |  | 50% | 0.0565462 | 0.0178941 | 0.0056599 | 0.0017899 | 0.0005660 | 0.0001790 | 5.660e-05 |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

2.2. Final simulation results plot

### Simulation case 3

1. TDD DL  SBFD DL

1.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -20dB | - 10dB | Relative ACIR | +10dB | +20dB | +30dB | … |  |  |
| FR1 4GHz Uma-Uma | CMCC | 3 TDDDL>SBFDDL | 5% | 78.33 | 38.95 | 8.700 | 0.963 | 0.094 | 0.009 |  |  |  |
|  |  |  | 50% | 2.975 | 0.2028703 | 0.0153815 | 0.0014270 | 0.0001405 | 1.394e-05 |  |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

1.2. Final simulation results plot

2. TDD DL  SBFD UL

2.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Relative ACIR | 5dB | 10dB | 15dB | +20dB | +25dB | +30dB |  |  |
| FR1 4GHz Uma-Uma | CMCC | 3 TDDDL>SBFDUL | 5% | 79.46 | 56.70 | 31.26 | 13.57 | 4.838 | 1.593 | 0.511 |  |  |
|  |  |  | 50% | 0.144 | 0.046 | 0.014 | 0.0045674 | 0.0014444 | 0.0004568 | 0.0001444 |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

2.2. Final simulation results plot

## FR2

### Simulation case 1

1. SBFD DL  TDD DL

1.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -20dB | - 10dB | Relative ACIR | +10dB | +20dB |  | … |  |  |
| FR2 30GHz Uma-Uma | CMCC | 1 SBFDDL>TDDDL | 5% | 66.812991 | 27.501274 | 4.8093525 | 0.4976720 | 0.0468379 |  |  |  |  |
|  |  |  | 50% | 2.5035963 | 0.1790337 | 0.0143104 | 0.0013455 | 0.0001327 |  |  |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

1.2. Final simulation results plot

2. SBFD UL  TDD DL

2.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -20dB | - 10dB | Relative ACIR | +10dB | +20dB |  | … |  |  |
| FR2 30GHz Uma-Uma | CMCC | 1 SBFDUL>TDDDL | 5% | 3.5177880e-05 | 3.5177901e-06 | 3.5177902e-07 | 3.5177908e-08 | 3.5178098e-09 |  |  |  |  |
|  |  |  | 50% | 1.8159290e-07 | 1.8159258e-08 | 1.8159320e-09 | 1.8159940e-10 | 1.8128898e-11 |  |  |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

2.2. Final simulation results plot

### Simulation case 2

1. SBFD DL  TDD UL

1.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -20dB | - 10dB | Relative ACIR | +10dB | +20dB | +30dB | +40dB |  |  |
| FR2 30GHz Uma-Uma | CMCC | 1 SBFDDL>TDDUL | 5% | 92.030843 | 40.629590 | 7.1056505 | 0.7895261 | 0.0798948 | 0.0079995 | 0.0008000 |  |  |
|  |  |  | 50% | 3.6812940 | 0.3859149 | 0.0388037 | 0.0038825 | 0.0003883 | 3.8827391e-05 | 3.8827413e-06 |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

1.2. Final simulation results plot

2. SBFD UL  TDD UL

2.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -20dB | -15dB | -10dB | -5dB | Relative ACIR | +5dB | +10dB |  |  |
| FR2 30GHz Uma-Uma | CMCC | 1 SBFDUL>TDDUL | 5% | 28.815828 | 11.932588 | 4.2288326 | 1.4099705 | 0.4535658 | 0.1438917 | 0.0455489 |  |  |
|  |  |  | 50% | 0.2286446 | 0.0724667 | 0.0229357 | 0.0072549 | 0.0022944 | 0.0007256 | 0.0002294 |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

2.2. Final simulation results plot

### Simulation case 3

1. TDD DL  SBFD DL

1.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -20dB | - 10dB | Relative ACIR | +10dB | +20dB | +30dB | … |  |  |
| FR2 30GHz Uma-Uma | CMCC | 3 TDDDL>SBFDDL | 5% | 49.135042 | 15.106213 | 1.9526032 | 0.1899056 | 0.0189069 | 0.0018905 |  |  |  |
|  |  |  | 50% | 1.0213903 | 0.0745765 | 0.0066381 | 0.0006456 | 6.4321737e-05 | 6.4274480e-06 |  |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

1.2. Final simulation results plot

2. TDD DL  SBFD UL

2.1. Final simulation results table

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -20dB | - 15dB | - 10dB | -5dB | Relative ACIR | +5dB | +10dB |  |  |
| FR2 30GHz Uma-Uma | CMCC | 3 TDDDL>SBFDUL | 5% | 53.772359 | 29.545238 | 12.464491 | 4.5184019 | 1.5019988 | 0.4829292 | 0.1535336 |  |  |
|  |  |  | 50% | 0.1441545 | 0.045637 | 0.0144388 | 0.0045667 | 0.0014442 | 0.0004567 | 0.0001444 |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |
| Explanations: - The relative means was derived from the legacy and baseline ACLR ACS assumptions for legacy TDD and SBFD BS and UE; - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD UL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD gNB ACS; - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; - For SBFD -> TDD DL case: The relative ACIR is derived from SBFD gNB ACLR, UE ACLR and legacy TDD UE ACS; The offset ACIR for this case needs FFS. |  |  |  |  |  |  |  |  |  |  |  |  |

2.2. Final simulation results plot

# 3. Conclusions

In this contribution, our initial simulation results are listed as above. due to limited simulation period, we will update final simulation results in next meeting or late in this meeting. suggestion of ACIR for SBFD system will be provided after finishing all simulation scenarios and cases.

# 4. Reference

[1] R4-2220246, WF for co-existence study, CMCC, Samsung