3GPP TSG-RAN WG4 Meeting #107	R4-2308723

Inchon Korea, 22 – 26 May, 2023

Source:	ZTE Corporation

Title:	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR

Agenda Item:	8.10.3.1

Document for:	Approval

- Introduction

In the last meeting, great progress on the measurement without gaps for UEs reporting NeedForGapsInfo has been made [1]. However, there are still some important issues left to be discussed. In this contribution, we’d like to share our views on the no-gap measurement in the aspects of interruption, measurement reporting delay requirement and scheduling availability.

- Discussion

- 2.1 Interruption

The NeedForGap feature can bring gains for system throughput with no-gap measurement. And in Rel-18, it has been agreed that no-gap measurement shall consider the following two cases:

Case 1: without gap and no interruption

Case 2: without gap but interruption allowed

There is a need for RAN4 to design corresponding RRM requirements for each case. Specially, the following issues would be discussed in this subject one by one :

Issue 1-1-1: Framework of the interruption requirements

Issue 1-1-2: Requirements on the interruption length , if allowed

Issue 1-1-5: Requirements on the interruption ratio, if allowed

Issue 1-1-9: DRX based interruption ratio, if allowed

Issue 1-1-1: Framework of the interruption requirements

In the previous meetings, RAN4 has agreed to define interruption length and interruption ratio for the RRM requirements. Besides, due to the fact that the interruption would result in the missed ACK/NACK, it is fine to apply some restrictions for interruption. In the last meeting, some possible restrictions for interruption were proposed as follows:

- Option 1: The UE is only allowed to cause interruptions on PCell or activated Scell(s) immediately before and after an SMTC. The UE is not expected to cause interruption on each SMTC occasion. - Option 2: The UE is only allowed to cause interruptions on Pcell or activated Scell(s) in the certain time window before and after an SMTC. - Other options are not precluded.

Among these options, option 1 is preferred. Because option 1 defines the interruption location clearly. And if NW is aware of exact interruption location, NW would schedule UE more effectively. For option 2, if UE is allowed to cause interruption in a certain time window before and after an SMTC occasion, there are two possible cases:

Case 1: The interruption is immediately before and after an SMTC, similar to option 1;

Case 2: The interruption is not next to the SMTC, as Fig.1.

In case 2, the vacant RF chain would be activated for longer time, which causes that the activated RF chain may have impact on the serving RF chain for longer time compared with option 1. We are confused about the intention and using scenario of Case 2, and kindly to ask proponents to explain them during the discussion.

Fig. 1 Case 2: the interruption is not next to the SMTC

Additionally, we think the UE is not expected to cause interruption on each SMTC occasion. Because if the interruption could happen on each SMTC occasion, the scheduling restrictions would be applied on each SMTC occasion, which has a large negative impact on the system throughout.

Proposal 1: The UE is only allowed to cause interruptions immediately before and after an SMTC occasion.

Issue 1-1-2: Requirements on the interruption length , if allowed

When defining restrictions for interruption, it is important to know the interruption length. Companies shared their views during previous meetings, but after the discussion in the last meeting, there are only the following two options left:

- Option 1: As a starting point, the interruption length can be same as VIL defined for NCSG,e.g. - When UE reporting “[no-gap,TBD]” in [NeedForGapInfoNR, TBD] the interruption length can be VIL=1ms in FR1 and VIL=0.75ms in FR2. - When UE reporting “[others,TBD]” in [NeedForGapInfoNR, TBD] no interruption allowed - Option 2: As a starting point, when UE reporting “no-gap [TBD]” in [NeedForGapInfoNR, TBD] , the interruption length can be specified based on the same RTT assumption as for NCSG (0.5ms in FR1 and 0.25ms in FR2) interruption occasion.

In our view, option 2 is preferred. We think RF re-tuning results in the interruption in the no-gap measurement with interruption. So we shall take RTT assumption as base line, i.e. the interruption length is 0.5 ms in FR1 or 0.25ms in FR2. For option 1, UE performs more than RF re-tuning, UE also needs to prepare the baseband during the VIL, which is different from no-gap measurement with interruption.

Proposal 2: For the requirements on interruption length, apply the RTT assumption, i.e. the interruption length can be 0.5ms in FR1 or 0.25ms in FR2.

Issue 1-1-5: Requirements on the interruption ratio, if allowed

For the interruption ratio requirements, the upper boundary of interruption ratio has been defined in the last meeting. However, there are still some issues to be noted:

- FFS if the interruption rate can be captured in equation format - Tcycle definition is FFS - Option 1: Tcycle = SMTC x CSSF x Kp - Other options are not precluded

For the first bullet, we don’t have strong views. However, it’s necessary to discuss frequency layer issue and the definition of measurement cycle no matter to define the interruption ratio in equation form or not. Thus, we suggest to process other two bullets first, and if time allowed, we can go back to discuss this bullet.

For the second bullet, we are fine to consider SMTC, CSSF and Kp. There is no concept of ‘pattern’ similar to gap and ncsg in the no-gap measurement with interruption. Taking legacy no-gap measurement as reference, SMTC period is needed. And Kp is introduced for the collision between SMTC and MG/VIRP. For Kp, some companies have concern about the necessity considering UE is expected to use a spare RF chain. However, in our view, this assumption has not been concluded in the previous meetings. So we are fine to consider Kp in the definition of Tcycle, and open to further discuss the magnitude of Kp.

Proposal 3: We are open for whether to define interruption ratio in equation form or not, but we suggest to discuss frequency layer issue and the definition of Tcycle first.

Proposal 4: We are fine to consider SMTC, CSSF and Kp in the definition of Tcycle.

Issue 1-1-9: DRX based interruption ratio, if allowed

DRX is an important technology in the NR to achieve power saving. The options proposed in the last meeting  were as follows:

- FFS on DRX based interruption ratio - Option 1: - When DRX cycle is equal or smaller than 320ms, - no interruption is expected when configured SMTC occasions are misalignment with DRX ON duration; - otherwise, the interruption ratio is min(K, 2*L/(KNeedForGaps,i *1.5* max(DRX cycle, SMTCi) *CSSFi)). - When DRX cycle is larger than 320ms, no interruption is expected

We can understand the general idea that UE is expected to result in the interruption during the DRX off duration. Generally, we are fine with this motivation, because the interruption ratio could be reduced (even to zero) in this way.

Proposal 5: UE is expected to perform measurement during DRX off duration.

- 2.2 Measurement reporting delay requirements

Issue 1-2-1 Requirement for intra/inter-freq measurement without gap when interruption allowed (case 2)

Issue 1-2-2: Requirement for inter-freq measurement without gap (Inter-f case 1)

When defining the measurement requirements for no-gap measurement with interruption,the update of the definition of inter-freq SSB based measurement without gaps is needed to include the case when n UE indicates ‘nogap-withinterruption[TBD]’ via ‘needForGap-r18[TBD]’.

Generally, the following options were proposed in the last meeting:

Option 1: If UE indicate ’nogap-withinterruption[TBD] via NeedForGapInfoNR-r18[TBD], UE shall be able to identify a new detectable intra/inter frequency cell within…. Option 2: in the introduction part, the definition for intra/inter-freuqency measurement without gap can be updated as: “A measurement is defined as inter-frequency measurement without gaps if the UE indicates ‘no-gap’ via interFreq-needForGap for inter-frequency measurement.

For option 2, in the current specification TS 38133 clause 9.3.1, the definition of inter-freq no-gap measurement is as follows:

A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) for UE capable of interFrequencyMeas-NoGap provided - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE. For UE supporting nr-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoNR for inter-frequency measurement, - An inter-frequency SSB measurement is defined as measurement without gap if - the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE

It is noted that the definition of no-gap measurement does not consider the impact of NeedForGap. Thus in the introduction part, the definition of inter-freq no-gap measurement shall be updated. And the introduction of intra-freq measurement has considered the impact of NeedForGap in the TS38.133 clause 9.2.1. There is no need to update the definition of intra-freq no-gap measurement.

Proposal 6: The definition of inter-freq no-gap measurement needs to be updated in the TS 38.133 clause 9.3.1.

## 2.3 Scheduling availability

Issue 1-4-3: On top of which existing requirements to define scheduling restriction requirements

In the last meeting, it has been agreed that the requirements for NCSG can be taken as base line to define the scheduling availability. In our understanding, although there are no concepts such as VIRP and ML in the no-gap measurement, the scheduling restriction requirements focus on the SSB to be measured / SMTC window without consideration of VIRP or ML. We can reuse the scheduling restriction requirements in the TS38133 clause 9.3.10 for the SMTC occasion with interruption. However, for the SMTC occasion where UE doesn’t perform measurement on, there is no scheduling restriction. We suggest to discuss how to distinguish the two cases.

Proposal 7: Existing scheduling restriction requirements in the TS38133 clause 9.3.10 can be reused for no-gap measurement with interruption. And there is no scheduling restriction for no-gap measurement without interruption.

Issue 1-4-4: Default SMTC pattern

In order not to apply scheduling restriction on each SMTC occasion, some companies proposed the following option in the last meeting:

Default SMTC pattern should be defined to restrict the scheduling restriction occasions if RAN4 doesn’t define a dedicated measurement pattern for interruption occasions

In our view, with default SMTC pattern, NW could be aware of which SMTC occasions the UE performs measurement on, and NW can schedule UE more effectively to improve system throughout.

Proposal 8: We are fine to introduce the default SMTC pattern.

- Conclusion

In this contribution, we have the following observations and proposals for measurement without gaps:

Proposal 1: The UE is only allowed to cause interruptions immediately before and after an SMTC occasion.

Proposal 2: For the requirements on interruption length, apply the RTT assumption, i.e. the interruption length can be 0.5ms in FR1 or 0.25ms in FR2.

Proposal 3: We are open for whether to define interruption ratio in equation form or not, but we suggest to discuss frequency layer issue and the definition of Tcycle first.

Proposal 4: We are fine to consider SMTC, CSSF and Kp in the definition of Tcycle.

Proposal 5: UE is expected to perform measurement during DRX off duration.

Proposal 6: The definition of inter-freq no-gap measurement needs to be updated in the TS 38.133 clause 9.3.1.

Proposal 7: Existing scheduling restriction requirements in the TS38133 clause 9.3.10 can be reused for no-gap measurement with interruption. And there is no scheduling restriction for no-gap measurement without interruption.

Proposal 8: We are fine to introduce the default SMTC pattern.

- Reference

- [1]  R4-2306331 “WF on NR_MG_enh2 Part 2”, Intel Corporation