3GPP TSG-RAN WG4 Meeting # 107 	    R4-2308789

Incheon, South Korea, 22nd – 26th May 2023

Agenda Item:	8.23.3.1

Source: 	Ericsson

Title:	Response to RAN1 LS on measurement definitions for positioning with bandwidth aggregation

Document for:	Approval

# Introduction

RAN1#112bis-e agreed to reuse legacy definition of DL-RSTD, UL-TDoA, UE Rx-Tx time difference, gNB Rx-Tx time difference for positioning measurement definition based on bandwidth aggregation. In this contribution we present our view on response to RAN1 LS.

# Discussion

RAN1#112bis-e discussed measurement definitions for positioning with bandwidth aggregation and concluded the following:

The legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference is reused with the assumption that the subframe timings of the intra-band contiguous carriers are the same.

- Note: multiple PRS/SRS resources which can be used to determine the start of subframe can be from multiple intra-band continuous carriers,

- Note: no RAN1 spec impact

- Send an LS to RAN4 to confirm RAN1’s understanding

Legacy definitions do not capture bandwidth related aspects and therefore needs to be updated to also consider defining positioning measurements based on bandwidth aggregation.

Proposal 1: RSTD measurement definition is updated by incorporating the following text.

“A UE capable of PRS bandwidth aggregation shall perform DL RSTD by aggregating PRS resources across multiple intra-band contiguous PFLs.”

Proposal 2: UE Rx-Tx measurement definition is updated by incorporating the following text.

“A UE capable of PRS/SRS bandwidth aggregation shall perform UE Rx-Tx time difference measurement by aggregating PRS resources across multiple DL intra-band contiguous PFLs and SRS in multiple UL intra-band contiguous PFLs. The PRS resource aggregation and SRS resource aggregation shall be performed on the same number of PFLs.”

Proposal 3: UL-RTOA measurement definition is updated by incorporating the following text.

“A base station capable of SRS resource aggregation shall perform TUL-RTOA measurement by aggregating SRS resources across multiple intra-band contiguous carriers.”

Proposal 4: gNB Rx-Tx measurement definition is updated by incorporating the following text.

“A base station capable of PRS/SRS bandwidth aggregation shall perform gNB Rx-Tx time difference measurement by aggregating SRS resources across multiple UL intra-band contiguous PFLs and PRS in multiple DL intra-band contiguous PFLs. The PRS resource aggregation and SRS resource aggregation shall be performed on the same number of PFLs.”

# Summary

In this contribution we presented our view on positioning measurement definitions to adapt legacy positioning measurement for bandwidth aggregation based positioning measurements. We proposed some update to legacy positioning measurements and propose to send response to RAN1 LS based on the proposals below.

Proposal 1: RSTD measurement definition is updated by incorporating the following text.

“A UE capable of PRS bandwidth aggregation shall perform DL RSTD by aggregating PRS resources across multiple intra-band contiguous PFLs.”

Proposal 2: UE Rx-Tx measurement definition is updated by incorporating the following text.

“A UE capable of PRS/SRS bandwidth aggregation shall perform UE Rx-Tx time difference measurement by aggregating PRS resources across multiple DL intra-band contiguous PFLs and SRS in multiple UL intra-band contiguous PFLs. The PRS resource aggregation and SRS resource aggregation shall be performed on the same number of PFLs.”

Proposal 3: UL-RTOA measurement definition is updated by incorporating the following text.

“A base station capable of SRS resource aggregation shall perform TUL-RTOA measurement by aggregating SRS resources across multiple intra-band contiguous carriers.”

Proposal 4: gNB Rx-Tx measurement definition is updated by incorporating the following text.

“A base station capable of PRS/SRS bandwidth aggregation shall perform gNB Rx-Tx time difference measurement by aggregating SRS resources across multiple UL intra-band contiguous PFLs and PRS in multiple DL intra-band contiguous PFLs. The PRS resource aggregation and SRS resource aggregation shall be performed on the same number of PFLs.”

# References

- R1-2304082, “LS on measurement definitions for positioning with bandwidth aggregation”, ZTE.

# Appendix

3GPP TSG-RAN WG4 Meeting #107		R4-230xxxx

Incheon, South Korea, 22nd – 26th May 2023

Title:	Response to LS on measurement definitions for positioning with bandwidth aggregation

Response to:	R1-2304082

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	RAN WG4

To:	RAN WG1

Cc:	N/A

Contact Person:

Name:	Deep Shrestha

E-mail Address:	deep.shrestha@ericsson.com

Attachments:	N/A

1. Overall Description:

RAN4 thanks RAN1 for LS on measurement definitions for positioning with bandwidth aggregation.

RAN4#107 discussed the measurement definition for bandwidth aggregation-based positioning measurements and agreed on the following:

- Existing RSTD measurement definition is updated by incorporating the following text.

“A UE capable of PRS bandwidth aggregation shall perform DL RSTD by aggregating PRS resources across multiple intra-band contiguous PFLs.”

- Existing UE Rx-Tx measurement definition is updated by incorporating the following text.

“A UE capable of PRS/SRS bandwidth aggregation shall perform UE Rx-Tx time difference measurement by aggregating PRS resources across multiple DL intra-band contiguous PFLs and SRS in multiple UL intra-band contiguous PFLs. The PRS resource aggregation and SRS resource aggregation shall be performed on the same number of PFLs.”

- Existing UL-RTOA measurement definition is updated by incorporating the following text.

“A base station capable of SRS resource aggregation shall perform TUL-RTOA measurement by aggregating SRS resources across multiple intra-band contiguous carriers.”

- Existing gNB Rx-Tx measurement definition is updated by incorporating the following text.

“A base station capable of PRS/SRS bandwidth aggregation shall perform gNB Rx-Tx time difference measurement by aggregating SRS resources across multiple UL intra-band contiguous PFLs and PRS in multiple DL intra-band contiguous PFLs. The PRS resource aggregation and SRS resource aggregation shall be performed on the same number of PFLs.”

2. Actions:

To RAN WG2 group.

ACTION: 	RAN4 kindly requests RAN1 to take the above agreements into consideration.

3. References:

- R1-2304082, “LS on measurement definitions for positioning with bandwidth aggregation”, ZTE.

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #108			21 – 25, August 2023							Toulouse, France

TSG RAN WG4 Meeting #108bis		09 – 13, October 2023						Xiamen, China