3GPP TSG-RAN WG4 Meeting #107              	R4-2308836

Incheon, South Korea, 22th May- 26th May, 2023

Agenda item:	5.2.9.6.2

Source:	Samsung

Title:	Discussion and simulation results on PUSCH demodulation requirement for Rel-17 71GHz

Document for:	Discussion

- Introduction

In the previous RAN4#106 meeting, the updated test setup for PUSCH demodulation requirement with 71GHz was agreed. The related agreement was captured into the WF [1].

In this contribution, the updated simulation results including ideal and impairment results are provided for requirement.

# 2	Discussion and simulation results

In this section, the view on remaining issue about test setup for PUSCH requirement is provided. Regarding the MCS for 2x2 condition, the following agreement are made as

MCS and number Tx/Rx branches 16 QAM for PUSCH requirements

- For 1x2 Low, use TDLA30-650 for 100MHz CBW, TDLA10-650 for 400MHz CBW. - For 2x2 Low, use TDLA30-200 for 100MHz CBW, TDLA10-200 for 400MHz CBW - Bring simulation results next meeting - MCS 16 - TDLA30-200 - 120 kHz and 100 MHz - TDLA10-200 - 120 kHz and 400 MHz - TDLA10-200 - 480 kHz and 400 MHz - MCS 12 - TDLA30-650 - 120 kHz and 100 MHz - TDLA10-650 - 120 kHz and 400 MHz - TDLA10-650 - 480 kHz and 400 MHz - Decide between MCS 12 and MCS 16 for 2x2 requirements based on the results provided.

Based on our initial result, the SNR with MCS 16 even with low doppler scenario is around larger than 15dB. With considering the margin, it can be close to 20dB, higher than the limitation of link budget for FR2-2. Therefore, we think it is not feasible to define requirement with MCS 16 for TDLA30-200 channel condition with 2x2, considering the link budget for FR2-2.

Observation 1: Similar performance for MCS 16 is achieved with TDLA30-650 and TDLA30-200 channels.

Observation 2: The achieved SNR with MCS 16 for TDLA30-200 channel condition with 2x2 is higher than the link budget of FR2-2

Proposal 1: apply MCS 12 for PUSCH requirement with 2x2 for FR2-2

# 3	Simulation results

In this section, we provide the updated simulation results based on agreed simulation assumption in the last meeting.

Table 1: ideal and impairment simulation results for PUSCH with CP-OFDM

| Case | SCS | CBW | MCS | Channel model | DMRS | Mapping type | Data length | PTRS | Antenna configuration | SNR (ideal) | SNR (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 120 | 100 | 4 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | Disabled | 1x2 | -2.57 | -0.57 |
| 1 | 120 | 100 | 4 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | Disabled | 2x2 | 1.52 | 3.52 |
| 2 | 120 | 100 | 16 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | 9.17 | 1.17 |
| 3 | 120 | 100 | 16 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 17.03 | 19.03 |
| 4 | 120 | 100 | 16 | TDLA30-200 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 16.80 | 18.80 |
| 5 | 120 | 100 | 12 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 11.60 | 13.60 |
| 7 | 120 | 100 | 20 | TDLD30-200 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | 10.82 | 12.82 |
| 8 | 120 | 400 | 4 | TDLA10-650 | 1+1 (0,8) | Type B | 10 | Disabled | 1x2 | -2.68 | -0.68 |
| 9 | 120 | 400 | 4 | TDLA10-650 | 1+1 (0,8) | Type B | 10 | Disabled | 2x2 | 1.74 | 3.74 |
| 10 | 120 | 400 | 16 | TDLA10-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | 9.39 | 11.39 |
| 11 | 120 | 400 | 16 | TDLA10-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 18.42 | 20.42 |
| 12 | 120 | 400 | 16 | TDLA10-200 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 18.1 | 20.1 |
| 12 | 120 | 400 | 12 | TDLA10-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 12.2 | 14.2 |
| 15 | 120 | 400 | 20 | TDLD10-200 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | 10.86 | 12.86 |
| 16 | 480 | 400 | 4 | TDLA10-650 | 1+1 (0,8) | Type B | 10 | Disabled | 1x2 | -2.74 | -0.7 |
| 17 | 480 | 400 | 4 | TDLA10-650 | 1+1 (0,8) | Type B | 10 | Disabled | 2x2 | 1.42 | 3.42 |
| 18 | 480 | 400 | 16 | TDLA10-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | 8.78 | 10.78 |
| 19 | 480 | 400 | 16 | TDLA10-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 16.68 | 18.68 |
| 20 | 480 | 400 | 16 | TDLA10-200 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 16.3 | 18.3 |
| 20 | 480 | 400 | 12 | TDLA10-650 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 2x2 | 11.3 | 13.3 |
| 23 | 480 | 400 | 20 | TDLD10-200 | 1+1 (0,8) | Type B | 10 | K_PTRS=2 L_PTRS=1 | 1x2 | 10.73 | 12.73 |

Table 2: ideal and impairment simulation results for PUSCH with DFT-s-OFDM

| Case | SCS | PRB | MCS | Channel model | DMRS | Mapping type | Data length | PTRS | Antenna configuration | SNR (ideal) | SNR (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 120 | 64 | 4 | TDLA30-650 | 1+1 (0,8) | Type B | 10 | Disabled | 1x2 | -2.50 | -0.5 |
| 1 | 480 | 64 | 4 | TDLA10-650 | 1+1 (0,8) | Type B | 10 | Disabled | 1x2 | -2.68 | -0.68 |

# 4	Conclusion

In this contribution, the updated ideal and impairment simulation results for 71GHz PUSCH requirement are provided for requirement derivation

Observation 1: Similar performance for MCS 16 is achieved with TDLA30-650 and TDLA30-200 channels.

Observation 2: The achieved SNR with MCS 16 for TDLA30-200 channel condition with 2x2 is higher than the link budget of FR2-2

Proposal 1: Apply MCS 12 for PUSCH requirement with 2x2 for FR2-2

# References

R4-2302971, WF on FR2-2 BS demodulation requirements

R4-2302163, Simulation results summary