3GPP TSG-RAN WG4 Meeting #107	R4-23xxxxx

# Incheon, 22nd - 26th May, 2023

Title: 	Discussion on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	8.8.4.2

Document for:	Discussion

Background

During RAN4#106-bis-e meeting, WF [1] on UE demodulation and CSI requirements for FR2 multi-Rx DL reception was approved. In this contribution, we share our views about UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception.

Discussion

Channel model

- Proposals - Option 1: Consider including TDLA30-300 for 120kHz/200MHz requirements, if introduced - Option 2: Consider TDLA30-300 for 100 MHz/120 kHz. - Option 3: Consider TDLA30-75 for 100MHz/120KHz

For the channel model, both TDLA30-300 and TDLA30-75 channel model are used for the legacy FR2 performance requirements. We think both Options are feasible and we slightly prefer to consider TDLA30-300 channel model for FR2 multi-Rx performance requirements.

Consider TDLA30-300 channel model for FR2 multi-Rx performance requirements.

PTRS port

- Proposals - Option 1: One PTRS port per TRP for both sDCI and mDCI schemes - Option 2: One port PTRS for s-DCI SDM scheme

UE supporting one port per TRP for PTRS for single-DCI SDM scheme is up to UE capability, so we prefer to only configure one port PTRS for single-DCI SDM scheme for FR2 multi-Rx demodulation requirements.

| supportTwoPortDL-PTRS-r16 Indicates whether UE supports 2-port DL PT-RS. UE supports this feature should indicate support singleDCI-SDM-scheme-r16 for the band. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Only configure one port PTRS for single-DCI SDM scheme for FR2 multi-Rx demodulation requirements.

Time offset between TRPs

- Proposals - Option 1: Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us}. - Option 2: Use time offset of TRP1 with respect to TRP2 as {0.2us, -0.05us} for different test cases. - Option 3: For NR FR2-1 multi-Rx chain DL reception, consider sDCI and mDCI based SDM scheme with timing offset between TRxPs as follow - Timing offset of the second TRxP from the first TRxP [us] as -0.0625 or 0.25. - Option 4: Select following value for timing offset of the second TRxP from the first TRxP for demodulation cases. Timing offset[us]. Multi-DCI based multi-TRP; non-overlapping; -0.0625. full-overlapping; -0.0625. Single-DCI based multi-TRP; SDM; 0.25.

In FR1 multi-TRP discussion small negative or large positive timing offset are selected for different cases to satisfy the test coverage due to assumption of one FFT processing window as baseline. Following same principle or assumption, smaller timing offset should be selected considering smaller coverage for FR2 scenario deployment. By scaling of the FR1 timing offset value for multi-TRP cases, -0.0625us and 0.25us can be reasonable value. The value for m-DCI case can be scaled from the FR1 m-DCI case, i.e. -0.0625 us, while the value for s-DCI case can be scaled from the FR1 s-DCI case 2, i.e. 0.25 us.

Select following value for timing offset of the second TRxP from the first TRxP for demodulation cases.

|  |  | Timing offset[us] |
| --- | --- | --- |
| Multi-DCI based multi-TRP | non-overlapping | -0.0625 |
|  | full-overlapping | -0.0625 |
| Single-DCI based multi-TRP | SDM | 0.25 |

Frequency offset between TRPs

- Proposals - Option 1: Select 3000Hz frequency offset for all demodulation cases as starting point. - Option 2: 600Hz - Option 3: 400Hz

The frequency offset can be select as the 0.1ppm that is the maximum BS frequency error based on TS 38.104. So we propose to select 3000Hz frequency offset for all demodulation cases that is the value at the carrier frequency 30GHz.

Select 3000Hz frequency offset for all demodulation cases.

Proposals

In this contribution, we discuss on UE PDSCH demodulation requirements for NR FR2 multi-Rx chain DL reception. Our observations and proposals are:

- Consider TDLA30-300 channel model for FR2 multi-Rx performance requirements.

Only configure one port PTRS for single-DCI SDM scheme for FR2 multi-Rx demodulation requirements.

Select following value for timing offset of the second TRxP from the first TRxP for demodulation cases.

|  |  | Timing offset[us] |
| --- | --- | --- |
| Multi-DCI based multi-TRP | non-overlapping | -0.0625 |
|  | full-overlapping | -0.0625 |
| Single-DCI based multi-TRP | SDM | 0.25 |

Select 3000Hz frequency offset for all demodulation cases.

Reference

R4-2306000, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, RAN4#106-bis-e, Qualcomm