3GPP TSG-RAN WG4 Meeting #107	R4-2308888

# Incheon, 22nd - 26th May, 2023

Title: 	Discussion on reference tunnel deployment scenario

Source: 	Huawei, HiSilicon

Agenda item:	8.13.5.4

Document for:	Discussion

Background

During RAN4#106-bis-e meeting, WF [1] on demodulation requirements for Rel-18 FR2 HST was approved. In this contribution, we share our views about HST FR2 reference tunnel deployment scenario.

Discussion

Demodulation requirements

Way forward: - Option 1: RAN4 discuss whether to define BS demodulation requirements for the tunnel deployment scenario based on the simulation results. If RAN4 study the new channel model, it should be differentiated from the exiting tunnel model: Scenario 3-NR350/500. - Option 2: BS demodulation requirements can be defined only if new channel model with multi-path propagation introduced - Option 3: Introduce BS demodulation requirements for tunnel scenario Way forward: - Option 1: RAN4 discuss whether to define UE demodulation requirements for the tunnel deployment scenario based on the simulation results. - Option 2: UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced, - Option 2a - When UE is little far away from the RRH, no new channel model and new demodulation requirements needed. - Only when UE is around the RRH, new demodulation requirements with multi-path fading model can be considered. - Option 3: Introduce the UE demodulation requirements with DPS transmission scheme with 1a and 1b for tunnel scenario, FFS on Uni-directional or Bi-directional RRH deployment.

For the BS side, in Rel-17 FR2 HST WI, various deployments are studied and only one set of requirements are defined using Bi-directional Scenario-B model considering that Bi-directional Scenario-B model provide largest frequency jump among all deployments (i.e. Uni-directional Scenario-A, Uni-directional Scenario-B and Bi-directional Scenario-B).

In the tunnel scenario, if the existing FR2 HST-DPS channel model is reused with some modification based on agreed Ds and Dmin, the performance is similar as the Scenario-A since that large frequency jump is not observed, as shown in the following Figure 2.1-1. So we proposed that BS demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

a) BS side										b) UE side

Figure 2.1-1 Doppler trajectory for Uni-directional Scenario-A and tunnel deployment

Large frequency jump is not observed if the existing FR2 HST-DPS channel model Scenario is reused with some modification based on agreed Ds and Dmin.

BS demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

For the UE side, similar reason as we discussed above, we propose that UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

Proposals

In this contribution, we discuss on HST FR2 reference tunnel deployment scenario. Our observations and proposals are:

- Large frequency jump is not observed if the existing FR2 HST-DPS channel model Scenario is reused with some modification based on agreed Ds and Dmin.

- BS demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

UE demodulation requirements can be defined only if new channel model with multi-path propagation introduced.

Reference

R4-2305892, WF on demodulation requirements for Rel-18 FR2 HST, RAN4#106-bis-e, Samsung