3GPP TSG-RAN4 Meeting #107	R4-2309292

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on transmit timing accuracy test cases

Agenda item:	5.2.8.5

Document for:	Discussion

Introduction

The WI description for RAN4 work related to NR over NTN was presented in [1]. For the past meetings, RAN4 has made progress in developing core requirements for NTN new features, as well as creating the performance testing configuration and specification to test such requirements. This WI was considered complete by RAN plenary in RAN #99, with the maintenance work moving forward to finalize the pending details on this work item. This document discusses pending open issues on the Way Forward  from RAN4 #106 [2]

Discussion

Currently, in test case A.14.3.1 (Transmit Timing for Satellite Access), there is the current step

4) The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Clause 7.1C.2 Table 7.1C.2.1-1 until the UE transmit timing offset is within NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc±Te_NTN[±Tmargin] respective to the first detected path (in time) of DL SSB. Skip this step for test 2 with DRX configured.

The value Tmargin is under discussion. It is supposed to account for “innacuracies” on the UE estimation of timing advance, as caused by the lack of precision on the common TA and ephemeris parameters (which are discrete numbers). But if the test cases are further relaxed, this will introduce even more relaxation on the transmit timing for NTN. It is our point of view that the current “extra margin” in NTN is sufficient to cover for the value of  Tmargin on top of other inaccuracies. Moreover, even if this value is significant and needs to be addressed by further relaxation, this should be accounted in the discussions for the core requirements, not the performance part.

The innacuracies addressed by Tmargin can be already sufficiently accommodated by the extra relaxation of the transmit timing requirement in NTN

If further relaxation is expected due to the lack of inaccuracy caused by Tmargin the topic should be discussed as part of the core part, not the performance part.

Do not add Tmargin in the transmit timing test case to account for inaccuracies due to lack of numerical precision in the satellite assistance information.

Conclusion

The paper has analyzed an open discussion from the Rel-17 WI that remains as an editorial note on the test cases for this specification:

- Proposal: Do not add Tmargin in the transmit timing test case to account for innacuracies due to lack of numerical precision in the satellite assistance information.

- References

- RP-213691, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2303147, WF on NR NTN RRM requirements, Qualcomm, October 2022