3GPP TSG-RAN4 Meeting #107	R4-2309319

# Incheon, Korea Republic, May 22nd – May 26th 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.133 on BFD/CBD Requirement for Rel-18 intra-band CA Type-2 UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA-Core |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the discussion for work item NonCol_intraB_ENDC_NR_CA, the following agreement is achieved: - Agreements - For non-collocated FR1 intra-band non-contiguous CA, Type 2 UE can be configured with BFD/CBD measurements on up to two serving cells in the same band Therefore, the corresponding restriction for UE performing BFD/CBD on no more than 1 serving cell per band shall be removed for Type-2 UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The restriction for UE performing BFD/CBD on no more than 1 serving cell per band is revised for Type-2 UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE is only to perform BFD/CBD on no more than 1 serving cell per band, which had been identified as problematic for FR1 intra-band NR-CA Type-2 UE, that is intended for non-collocated CA deployment in which the link qualities of serving cells in the same band could be different. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2, 8.5.3, 8.5.5, 8.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < START OF CHANGE #1 >

### 8.5.2	Requirements for SSB based beam failure detection

#### 8.5.2.1	Introduction

The requirements in this clause apply for each SSB resource in the set  configured for a serving cell, provided that the SSB configured for beam failure detection is actually transmitted within the UE active DL BWP during the entire evaluation period specified in clause 8.5.2.2. For UE not supporting [intraBandNRCA-NonCollocated-r18],the requirements in this clause could not be applicable if UE is required to perform beam failure detection on more than 1 serving cell per band. For UE supporting [intraBandNRCA-NonCollocated-r18], the requirements in this clause apply when UE is required to perform beam failure detection on no more than 2 serving cell per band if these 2 serving cells in non-contiguous carriers, and no more than 1 serving cell per band otherwise.

Table 8.5.2.1-1: PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

## < END OF CHANGE #1 >

## < START OF CHANGE #2 >

### 8.5.3	Requirements for CSI-RS based beam failure detection

#### 8.5.3.1	Introduction

The requirements in this clause apply for each CSI-RS resource in the set  of resource configurations for a serving cell, provided that the CSI-RS resource(s) in set for beam failure detection are actually transmitted within the UE active DL BWP during the entire evaluation period specified in clause 8.5.3.2. UE is not expected to perform beam failure detection measurements on the CSI-RS configured for BFD if the CSI-RS is not QCL-ed, with QCL-TypeD when applicable, with the RS in the active TCI state of any CORESET configured in the UE active BWP. For UE not supporting [intraBandNRCA-NonCollocated-r18], the requirements in this clause apply when UE is required to perform beam failure detection on no more than 1 serving cell per band. For UE supporting [intraBandNRCA-NonCollocated-r18], the requirements in this clause apply when UE is required to perform beam failure detection on no more than 2 serving cell per band if these 2 serving cells in non-contiguous carriers, and no more than 1 serving cell per band otherwise.

Table 8.5.3.1-1: PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 0dB |
| Bandwidth (PRBs) | 48 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

## < END OF CHANGE #2 >

## < START OF CHANGE #3 >

### 8.5.5	Requirements for SSB based candidate beam detection

#### 8.5.5.1	Introduction

The requirements in this clause apply for each SSB resource in the set  configured for a serving cell, provided that the SSBs configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.5.5.2. For UE not supporting [intraBandNRCA-NonCollocated-r18], the requirements in this clause apply when UE is required to perform beam failure detection on no more than 1 serving cell per band. For UE supporting [intraBandNRCA-NonCollocated-r18], the requirements in this clause apply when UE is required to perform beam failure detection on no more than 2 serving cell per band if these 2 serving cells in non-contiguous carriers, and no more than 1 serving cell per band otherwise.

## < END OF CHANGE #3 >

## < START OF CHANGE #4 >

### 8.5.6	Requirements for CSI-RS based candidate beam detection

#### 8.5.6.1	Introduction

The requirements in this clause apply for each CSI-RS resource in the set  configured for a serving cell, provided that the CSI-RS resources configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.5.6.2. For UE not supporting [intraBandNRCA-NonCollocated-r18], the requirements in this clause apply when UE is required to perform beam failure detection on no more than 1 serving cell per band. For UE supporting [intraBandNRCA-NonCollocated-r18], the requirements in this clause apply when UE is required to perform beam failure detection on no more than 2 serving cell per band if these 2 serving cells in non-contiguous carriers, and no more than 1 serving cell per band otherwise.

## < END OF CHANGE #4 >