3GPP TSG-RAN WG4 Meeting # 107	                                 	                        R4-2309367

# Incheon, South Korea, May 22 – May 26, 2023

Agenda item:	8.5.3.1.4

Source:	Apple Inc.

Title:	Further Discussion on CQI Reporting Requirements for 8RX UEs in FR1

Release:	Rel-18

Document for:	Discussion

# Introduction

Following the discussion at RAN4#106bis-e, we use this contribution to clarify our position on the remaining unresolved issues. This WI is focused on:

- Enable 8Rx for CPE/FWA/vehicle/industrial devices - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers - Specify the SDR requirements with 8 MIMO layers

# Discussion

During RAN4#106bis-e [1], some open items have been closed. Here, for completeness, we present the full set of items last discussed during the previous meeting. Here, we add our proposals on the still open items.

Report quantity configuration:

- Use the legacy configuration, report quantity 'cri-RI-PMI-CQI ' UE will report i1 while gNB will use i2 randomly (1 out 2 possibilities) Issue 4-2: The highest modulation order for 8Rx CQI test

Highest modulation order for 8Rx CQI test:

- Up to 64QAM

Tx EVM assumption for derivation of SNR values for CQI test:

- Option 1: No Tx EVM is assumed

- Option 2: Same Tx EVM values as for PDSCH demodulation performance is assumed, i.e. Tx EVM = 6% for up to  64QAM, Tx EVM = 3% for 256QAM, Tx EVM = 2.5% for 1024QAM

Here we believe that we should maintain consistency, at least in terms of setup, with the additionally defined PDSCH demodulation requirements for 8Rx. Since TxEVM is not critical at the SNR values we are considering for the CQI tests, there’s no harm in keep the setting the same for PDSCH performance and CQI reporting requirements.

Proposal#1: Maintain the same TxEVM assumption as the values used for PDSCH demodulation performance requirements.

SNR points:

- Option 1: [2,3] dB and [8,9] dB

- Option 2: [4,5] dB and [8,9] dB

- Option 3: [4,5] dB and [10,11] dB if report quantity 'cri-RI-i1-CQI' is agreed

- Option 4: [16,17]dB for 256QAM, [7,8]dB for 64QAM, [1,2]dB for 16QAM

- Option 5: [7,8] dB for 64QAM, [1,2] dB for 16QAM in case only up to 64QAM considered

Based on the agreed report quantify configuration, Option 3 is out of the table. Based on the agreed maximum modulation format of 64QAM, Option 4 is also out of the table. From our simulation results, we prefer Option 1: [2,3] dB and [8,9] dB to be set as requirements for 8Rx UEs.

Proposal#2: Use Option 1, [2,3] dB and [8,9] dB, for the single CQI test considered for 8Rx UEs.

Whether to define PMI and RI requirements:

- No need to define PMI and RI requirements

# Conclusion

In this contribution we provide our views on 8Rx UE CQI reporting requirements. Our latest proposals are summarized below:

Proposal#1: Maintain the same TxEVM assumption as the values used for PDSCH demodulation performance requirements.

Proposal#2: Use Option 1, [2,3] dB and [8,9] dB, for the single CQI test considered for 8Rx UEs.

# References

- R4-2305888, “WF on 8RX UE performance requirements”, Huawei, HiSilicon