3GPP TSG RAN WG4 Meeting #107	R4-2309645

Incheon, Korea, May 22-26, 2023

Agenda Item:	8.33.2

Source: 	Ericsson

Title:	Further analysis of RRM requirements for SL relay enhancement

Document for:	Discussion

# Introduction

In the last RAN4 meeting a WF to specify necessary RRM requirements for the enhancements related to the UE based relay was approved [1]. The WF contains the open issues identified during the meeting [2].

In this contribution we further analyze the RRM requirements to support enhanced relay operation.

# Analysis of RRM core requirements

There are two main and new scenarios related to SL relay enhancement which impacts RAN4 RRM requirements

- Discovery and (re)selection in UE-to-UE relay

- Selection/reselection of SL relay UE in multipath

## Discovery and (re)selection in UE-to-UE relay

In the last RAN4 meeting the following was agreed regarding the selection/reselection of the relay UE in UE-to-UE relay scenario [1]:

- The legacy requirements for selection/reselection of relay UE in clause 12.10 of TS 38.133 can be used as baseline and wait for RAN2 progress to evaluate whether changes are needed.

From the perspective of the UE’ physical layer perspective, the remote UE will use the existing Rel-17 UE-2-network relay (U2N) mechanism to select or reselect a relay UE to establish the UE-to-UE relay connection with the destination UE. In the existing Rel-17 U2N relay mechanism the remote UE selects the relay UE to establish connection with the network based on the SL-RSRP or SD-RSRP measured on the relay UE. In RAN2#121bis-e, RAN2 made the following agreement regarding the relay UE selection by the remote UE in the UE-to-UE relay scenario:

Agreement:

Each remote UE (source or destination) can trigger relay selection based on the direct link quality.  FFS interaction between discovery and selection.

Based on the above analysis and the latest RAN2 agreements, from layer-1 perspective the time to select/reselect a relay UE in UE-to-UE relay should be the same as defined for selection/reselection of a relay UE in UE-to network scenario in section 12.10, TS 38.133. Therefore, the selection/reselection delay of the relay UE defined in section 12.10 can be reused for the selection/reselection delay of the relay UE in 1-hop UE-to-UE relay scenario.

## Selection/reselection of SL relay UE in multipath

In the last RAN4 meeting the following was agreed regarding the selection/reselection of the relay UE in multipath scenario [1]:

- RAN4 can wait for RAN2 to progress further to decide on U2U selection and reselection considering m-path scenarios

Figure 1 illustrates the multipath scenario. The remote UE (UE1) needs to select or reselect a relay UE (UE2) for establishing or re-establishing the indirect path (i.e. UE-to-network connection). As shown in figure 1, UE1 and UE2 should be connected to the same gNB via their respective Uu1 and Uu2 interfaces respectively. However, UE1 and UE2 can be connected to or served by different cells provided that both cells are operated by the same gNB. Similarly, the Uu1 and SL (between UE1 and UE2) can be configured on the same/shared carrier frequency or they can be configured on different carriers e.g. SL on a dedicated SL carrier.

The selection or reselection of a relay UE (UE2) for establishing or re-establishing the indirect path (U2N) with the gNB already exists in Rel-17. The difference here is that the remote UE (UE1) will also have a direct path. Therefore, the selection/reselection of the relay UE (UE2) for the indirect path in the multipath scenario is based on the existing principle including the physical layer design. This means the remote UE (UE1) selects/reselects the relay UE (UE2) based on the SL-RSRP or SD-RSRP measured on the relay UE (UE2). The time to select/reselect a relay UE for the indirect path (U2N) should be the same as defined for selection/reselection of a relay UE in U2N relay scenario in section 12.10, TS 38.133. However, the selection/reselection of the relay UE (UE2) is triggered by the RRC reconfiguration, which will cause interruption to UE’s serving cell on the direct path i.e. on Uu1.

Furthermore, according to Rel-17 procedure there can be different combination of WAN and SL DRX operation on the Uu and SL as follows:

- UE1 can be in non-DRX on Uu and can be configured with SL DRX cycle for SL.

- UE1 can be configured with WAN DRX cycle for Uu and with SL DRX cycle for SL.

- UE1 can be configured with WAN DRX cycle for Uu and can be in non-SL DRX on SL.

- UE1 can be in non-DRX on Uu and also in non-SL DRX on SL.

In the DRX operational scenarios #1 and # 2, the SL DRX will cause interruption on the WAN signals on the direct path. The transitions between active and non-active part of the SL DRX causes interruption on the WAN signals on the direct path of UE1. Especially in scenario #1, the interruption on the WAN signals on the direct path of UE1 by the SL DRX can have quite severe impact on the WAN operation of UE1. Therefore, at least the maximum allowed interruption on the WAN due to the SL DRX in scenario # 1 should be defined in TS 38.133.

gNB

ss

Uu2

UE2

UE1

SL

Uu1

Figure 1: Remote/source UE (UE1) having simultaneous multiple path operation with gNB comprising of a direct path with the gNB (Uu) and one indirect path with the gNB via UE2 (relay UE).

# Summary

The following are the observations and the proposals related to the RRM core requirements for sidelink relay enhancements:

Single hop UE-to-UE relay scenario:

- Observation #1: The UE-to-UE relay scenario comprises of source UE connected to the destination UE via relay UE forming 1-hop UE-to-UE relay.

- Observation #2: From layer-1 perspective the time to select/reselect a relay UE in UE-to-UE relay should be the same as defined for selection/reselection of a relay UE in UE-to network (U2N) relay scenario in section 12.10, TS 38.133.

- Proposal #1: The requirements for the selection/reselection of the relay UE defined in section 12.10 (including measurement period in Table 12.10.2-1), TS 38.133 are reused for the selection/reselection delay of the relay UE in 1-hop UE-to-UE relay scenario.

Multi-path relay scenario:

- Observation #3: In multi-path relay scenario, the source/remote UE (UE1) has two simultaneous paths with the same gNB: one direct path (WAN/Uu) to gNB and one indirect path to the same gNB via UE-to-Network (U2N) relay UE (UE2).

- Observation #4: The selection/reselection of the relay UE (UE2) for the indirect path (U2N) by the remote UE (UE1) is based on the existing Rel-17 mechanism used for the relay UE (UE2) selection/reselection in the U2N scenario.

- Observation #5: The relay UE (UE2) selection/reselection in the U2N scenario (indirect path) is triggered by the RRC reconfiguration, which will cause interruption on the WAN signals of the remote UE (UE1) on the direct path.

- Observation #6: In multipath scenario, the remote UE (UE1) can be in non-DRX on both direct path with gNB and on the SL with the relay UE (UE2) or it can be configured with the WAN DRX cycle for Uu operation and the SL DRX cycle for the SL operation according to the Rel-17 DRX mechanism.

- Observation #7: In a multipath scenario when the remote UE (UE1) is in non-DRX on the direct path and is configured with SL DRX cycle, then the SL DRX operation will cause interruption on the direct path of UE1.

- Proposal #2: The time to select/reselect a relay UE for the indirect path (U2N) is the same as defined for selection/reselection of a relay UE in U2N relay scenario in section 12.10, TS 38.133.

- Proposal #3: Define the maxmum interruption length of each interruption occurred on the direct path when the remote UE (UE1) triggered by the RRC reconfiguration, selects/reselects the relay UE (UE2) for the indirect path. The value of the maximum interruption length is FFS.

- Proposal #4: Define the interruption requirements related to the interruption on the direct path when the remote UE (UE1) is in non-DRX on the direct path and is configured with the SL DRX cycle for the SL operation on the indirect path. The values of the interruption probability and the maxmum interruption length are FFS.

# References

- R4-2306367, WF on RRM requirements for SL relay enhancement, LG Electronics

- R4-2306258, Topic summary for [106-bis-e][227] NR_SL_relay_enh, Moderator (LGE)