3GPP TSG-RAN WG4 Meeting #107	R4-2309717

# Incheon, Korea, May 22nd – May 26th, 2023

# Title:	Updates for NTN UE terminal requirements discussion in above 10 GHz

# Source:	THALES

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	8.27.4

Release	Rel-18

# Introduction

The following Satellite 5G NR NTN (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

From Article 5 of the ITU Radio Regulations (RR) the mobile operation of ESIMs is also described in the following footnotes,

- 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15). (WRC-15)

- 5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). (WRC-19)

NTN UE terminal types in bands above 10 GHz were discussed at RAN4#105 (see also R4-2220573, “WF on NTN UE RF requirements”) and in RAN4#105 Main session report. Thanks to these discussions, a few agreements have been formulated, such as:

Agreement: - For NTN UE with parabolic antenna, to define the conducted requirement and further discuss how to define radiated requirement for it (e.g. EIRP limits, etc) Issue 4-2: For NTN UE with phase antenna array, radiated requirement only to be defined? Agreement: - For NTN UE with phase antenna array if defined, only radiated requirements are to be specified.

This contribution therefore provides material for discussion with respect to NTN UE terminal types in above 10 GHz. The reader should also consider as reference the documents submitted in RAN4#106 AI 9.25.2 (see for instance R4-2302535, THALES) and in RAN#106-bis-e AI 9.27.2.

During RAN#106 main session, the following agreements have been made:

Agreement: - Discuss whether and how to define the requirements for beam tracking for NTN UE. Agreement: - Define the radiated requirement as the first priority.

However, there were also some issues with respect to UE types, UE type differentiation, beam correspondence requirements, satellite RF specific requirements for NTN UE. One can further easily see that in some cases the discussions are not relevant since are related to implementation issue and should not be considered part of the standardization work.

During RAN4#106-bis-e it was also agreed:

Agreement: - Encourage companies to provide the regulation differences leading to different RF/RRM requirements between the mobile and fixed NTN UEs, and further discuss whether RAN4 needs to define the different UE types. - Discuss possible differentiation of NTN UE types in above 10 GHz, based on RF parameters (noise figure, antenna types, antenna gain and transmission power, or alternatively minimum EIRP in the peak direction, Effective Isotropic Sensitivity EIS, etc.). - The impact on RF and RRM requirements will be investigated, and analysis on RF requirement impact is prioritized.

For further information, please also consider:

- R4-2302527 (“NTN UE Terminal Types for above 10 GHz”, THALES) presented in RAN4#106.

- R4-2305844 (“NTN UE terminal reference architecture for above 10 GHz”, THALES, Hughes/EchoStar) presented in RAN4#106-bis-e for the NTN UE reference architecture in above 10 GHz.

- R4-2302998 (“WF for system parameters of above 10 GHz NTN band”, THALES) agreed in RAN4#106.

Proposal-1: 	RAN4 needs to define the off-axis EIRP requirement for SAN and ESIM type of devices.

Proposal-2: 	The off-axis EIRP requirements should be defined for ESIM devices operating over the band n511.

Proposal-3: 	RAN4 shall decide the baseline of requirements based on FCC requirement or ITU recommendation S.524.

Proposal-4: 	The defined off-axis EIRP density levels shall meet the permissible mask levels of validation and support the procedures of self-monitoring.

Moreover, in R4-2304364 (“Off-Axis EIRP requirements for NTN-TN NR-Uu interface”, Verizon) submitted at RAN4#106-bis-e, the following proposals have been made:

Issue 1-1: UE types

- Encourage companies to provide the regulation differences leading to different RF/RRM requirements between the mobile and fixed NTN UEs, and further discuss whether RAN4 needs to define the different UE types.

- Discuss possible differentiation of NTN UE types in above 10 GHz, based on RF parameters (noise figure, antenna types, antenna gain and transmission power, or alternatively minimum EIRP in the peak direction, Effective Isotropic Sensitivity EIS, etc.).

  - The impact on RF and RRM requirements will be investigated, and analysis on RF requirement impact is prioritized.

Issue 2-1:  Beam tracking capability

Tentative agreement:

the (Rx and Tx) beam pointing error are compliant with at least the relevant ETSI harmonized standard. (e.g.

      - EN 303 978    “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands”;

      - EN 303 979    “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands”).

However, there were no agreements during official meeting and corresponding NTN UE WF for Ka-band with respect to off-axis EIRP requirement (see R4-2306624, “WF on NTN UE RF requirements in Ka-band”, ZTE).

# Discussion with respect to UE NTN terminal

## 2.1	General aspects

All NTN UE antenna parameters have been adapted from TR 38.821 by considering the following parameters adapted to Ka-Band and decisions from RAN4#106 (updated with R4-2302998, WF for system parameters of above 10 GHz NTN band):

- Downlink frequency: 17.3 - 20.2 GHz

- Uplink frequency: 27.5 GHz - 30 GHz

- 60 cm antenna aperture diameter (2*a=60cm)

TR 38.821 presents VSAT UE as a more general description of NTN UE. There could further be different implementations (e.g. with parabolic, phased-array, hybrid antennas), however it seems there is not much difference between the characteristics of those NTN UE terminal types. Therefore, the current very small aperture terminal already covers different implementations and RAN4 could consider only one type of terminal for Ka-Band.

## 2.2	NTN UE with Parabolic antenna

Figure 1 shows the antenna pattern of a NTN UE transmit antenna reflector 0.6 m diameter and  operating at  28750 MHz. Figure 1 therefore shows an example with parabolic/dish antenna of 0.6 m diameter and  operating at  28750 MHz.

Figure 1. Antenna gain pattern of a NTN UE transmit parabolic antenna (R4-2302527)

## 2.3	Phased Array Antenna

Figure 2 shows the antenna pattern of a NTN UE transmit antenna with Dx=Dy=0,6 m, operating at  28750 MHz. Figure 2 here below shows an example with phased array antenna operating at 28750 MHz using a tapered illumination law.

Figure 2. Antenna gain pattern of a NTN UE transmit phased array antenna (R4-2302527)

The following reference radiation patterns should be adopted for angles between the direction considered and the axis of the main beam for frequencies in the range from 2 to 31 GHz:

G    32  –  25 log 		dBi		for  min          48°

  –10				dBi		for    48°        180°

where:

min    1° or 100 /D degrees, whichever is the greater, for D/ ≥ 50.

min =  2° or 114 (D/)–1.09 degrees, whichever is the greater, for D/ < 50.

With respect to the adjacent antenna lobes (i.e. adjacent antenna lobes to the main directional lobes, or secondary antenna lobes from the antenna radiation pattern), the following recommendation can be used (see [ITU-R S.465-6]):

If one compares the ITU-R S.465-6 limit with the one of NTN UE using parabolic/dish antenna (as seen in Figure 1) or using phased-array antenna (as seen in Figure 2), one will easily notice that both NTN UE antenna characteristics (i.e. using parabolic/dish antenna or using phased-array antenna) respect the ITU-R S.465-6 limit.

In fact, the characteristics of NTN UE in above 10 GHz using Parabolic/Dish antenna or Phased-Array antenna are currently very similar. Both types of antennas can be considered for VSAT (Very Small Aperture Terminal) and should not be differentiated from each other as separate NTN types of terminals.

## 2.4	ITU and FCC regulatory information

As a matter of fact,

- ITU recommendation S.524-9 describes:

4 that earth stations operating in GSO networks in the FSS transmitting in the 27.5-30 GHz frequency band be designed in such a manner that at any angle, , which is 2 or more off the main lobe axis of the earth station antenna, the e.i.r.p. density in any direction within 3 of the GSO should not exceed the following values: Angle off-axis Maximum e.i.r.p. per 40 kHz 2    7 (19 – 25 log ) dB(W/40 kHz) 0 7    9.2 –2 dB(W/40 kHz) 0 9.2    48 (22 – 25 log ) dB(W/40 kHz) 48    180 –10 dB(W/40 kHz). For any direction in the region outside 3° of the GSO, the above levels may be exceeded by no more than 3 dB.

- FCC recommendation 25.218 describes:

(i) Digital earth station operation in the conventional or extended Ka-band. (1) For co-polarized transmissions in the plane tangent to the GSO arc: 32.5–25log(θ); dBW/MHz; for; 2.0° ≤ θ ≤ 7°.. 11.5; dBW/MHz; for; 7° ≤ θ ≤ 9.2°.. 35.5–25log(θ); dBW/MHz; for; 9.2° ≤ θ ≤ 19.1°.. 3.5; dBW/MHz; for; 19.1° < θ ≤ 180°.. where θ is as defined in paragraph (c)(1) of this section. (2) For co-polarized transmissions in the plane perpendicular to the GSO arc: 35.5–25log(θ); dBW/MHz; for; 3.5° ≤ θ ≤ 7°.. 14.4; dBW/MHz; for; 7° < θ ≤ 9.2°.. 38.5–25log(θ); dBW/MHz; for; 9.2° < θ ≤ 19.1°.. 6.5; dBW/MHz; for; 19.1° < θ ≤ 180°.. where θ is as defined in paragraph (c)(1) of this section. (3) The EIRP density levels specified in paragraphs (i)(1) and (2) of this section may be exceeded by up to 3 dB, for values of θ > 7°, over 10% of the range of theta (θ) angles from 7–180° on each side of the line from the earth station to the target satellite.

According to the analysis further performed in this document for different allocated NTN UE transmission bandwidth, current NTN UE parameters/characteristics used for coexistence analysis are already respecting ITU and FCC regulations, as can be seen below:

(a)                                                                              (b)

Figure 3. EIRP density (dBW/Breference), where (a) Breference=40 kHz for ITU S.254, and (b) Breference=1 MHz for FCC 25.218, with UE NTN of 1.44 MHz allocation

(a)                                                                              (b)

Figure 4. EIRP density (dBW/Breference), where (a) Breference=40 kHz for ITU S.254, and (b) Breference=1 MHz for FCC 25.218, with UE NTN of 5 MHz allocation

(a)                                                                              (b)

Figure 5. EIRP density (dBW/Breference), where (a) Breference=40 kHz for ITU S.254, and (b) Breference=1 MHz for FCC 25.218, with UE NTN of 20 MHz allocation

(a)                                                                              (b)

Figure 6. EIRP density (dBW/Breference), where (a) Breference=40 kHz for ITU S.254, and (b) Breference=1 MHz for FCC 25.218, with UE NTN of 200 MHz allocation

Therefore, current NTN UE parameters/characteristics used for coexistence analysis are already respecting ITU and FCC regulations. In any case, for the sake of moving forward and speed up the RAN4 NTN UE related work in Rel-18, the following proposal can be made:

Proposal 10: For NTN UE in Ka-band, RAN4 to use/cite/reference ITU recommendation S.524 for radiated off-axis requirement.

In fact, ITU recommendation S.524-9 is generally applicable and should be considered as main reference. However, this does not necessary mean that 3GPP has to generate new coexistence simulations based on ITU recommendation, since ITU recommendation is already applicable and can be simply cited or reference.

## 2.5	NTN UE system information discussion

The following table is to get feedback from the delegates on the following issues:

Proposal 11: Further consider the following table for discussion in RAN4:

| Question | Initial Working Hypothesis | Feedback from Companies |
| --- | --- | --- |
| Should HO aspects in Ka-band be considered for Rel-18? | Yes. Preference for considering HO aspects in Ka-band for Rel-18. | Company X: Company Y: Company Z: |
| Which is the (minimum) number of satellites to which a VSAT/NTN UE will connect in Ka-band? | At least 2 satellites in Rel-18. It is further assumed: - UE is capable of forming 1 single beam only at any given instant of time (regardless of antenna system implementation). - At least two cases should be identified based on single-beam re-pointing time between satellites - The following example cases are considered: A) 1 mechanically-steered UE beam; B) 1 electronically-steered UE beam. | Company X: Company Y: Company Z: |
| Which is the maximum (recommended) interruption time (switching time) between 2 consecutive satellites during HO procedure? | Preference for seamless HO. At physical layer, an interruption of ~20-40ms may be acceptable. No packet loss is expected. TBD | Company X: Company Y: Company Z: |
| Any view for the initial Satellite search time requirement of the NTN UE? | Implementation issue not 3GPP related. TBD | Company X: Company Y: Company Z: |
| Any view of the tracking accuracy? | Implementation issue based on regulatory requirement (see e.g. EN 303 978 and EN 303 979). TBD | Company X: Company Y: Company Z: |
| Other | TBD | Company X: Company Y: Company Z: |

As a matter of fact, the (Rx and Tx) beam pointing error are compliant with at least the relevant ETSI harmonized standard, e.g.:

        EN 303 978, “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands”;

        EN 303 979, “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands”.

Proposal 12: HandOver aspects in Ka-band shall be considered for Rel-18 as working hypothesis.

Proposal 13: RAN4 shall consider at least 2 satellites (as working hypothesis) for NTN UE HandOver procedures in Ka-band.

Proposal 14: Seamless HandOver shall be considered as working hypothesis by RAN4 for NTN UE in above 10 GHz.

Proposal 15: Initial satellite search time can be considered as implementation issue by RAN4 (no requirement for initial satellite search time shall be specifically considered by RAN4).

Proposal 16: Tracking accuracy can be considered as implementation issue by RAN4 based on regulatory requirement (relevant regulatory requirement will be referenced in the NTN UE TS, however no requirement for tracking accuracy will be specifically designed by RAN4).

# Proposals resubmitted from R4-2305844

Proposal 6: RAN4 shall specify a generalized NTN UE terminal reference architecture for above 10 GHz as follows (the details can be left for implementation):

Note 1: The Up-Converter and the Tx Power Amplifier are part of the Transmission chain.

Note 2: The Rx LNA (Low-Noise Amplifier) and the Down-Converter are part of the Reception chain.

Note 3: RF represents the Radio Frequency region and IF the Intermediate Frequency region.

Note 4: DP is the Duplexer and ACU is the Antenna Control Unit.

Proposal 7: NTN UE beam steering, NTN UE beam switching capabilities, NTN UE beam switching granularity, NTN UE beam tracking capability or satellite beam tracking capability, etc. shall be left for implementation (and not part of 3GPP requirements).

Proposal 8: NTN UE beam steering, NTN UE beam switching capabilities, NTN UE beam switching granularity, NTN UE beam tracking capability or satellite beam tracking capability, etc. can be considered for implementation for example based on -3 dB NTN UE beam width and satellite constellation parameters (e.g. satellite orbit and velocity).

Proposal 9: RAN4 should specify an optional method for the NTN UE or the Network to disable NTN UE UL transmission if and when required.

Proposal 10: NTN UE power can be used to differentiate different types of NTN UE depending on the deployment scenario (e.g. land, vessel, aircraft, etc) in concordance with regulatory requirements. However, in order to simplify the normalisation of NTN UE above 10 GHz, only one single NTN UE power class could be specified in Rel-18.

# Proposals resubmitted from R4-2302527

Proposal 1: Do not separate Parabolic/Dish antenna or Phased-Array antenna in different NTN UE-types definitions. RAN4 to consider Parabolic/Dish antenna or Phased-Array antenna in a single common NTN UE-type definition for above 10 GHz, since similar performance characteristics.

Proposal 2: [Option 1] RAN4 to use the following NTN UE parameters:

Table 1. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Receiver noise figure | (dB) |  | 1,2 |
| Feeder loss | (dB) |  | -0,50 |
| Sky temperature | (K) |  | 30 |
| Ground temperature | (K) |  | 10 |
| Antenna temperature | (K) |  | 40 |
| G/T figure of merit | (dB/K) |  | 16,5 |

NOTE1: T_a = T_Sky + T_Ground

NOTE2: The antenna temperatures are based on e.g. ITU-R Rec. P372 and Rec. P618.

NOTE3: T_sky is computed using [ITU-R Rec. P.618-13] as expressed below

# 3 Noise temperature As attenuation increases, so does emission noise. For earth stations with low-noise front-ends, this increase of noise temperature may have a greater impact on the resulting signal-to-noise ratio than the attenuation itself. The sky noise temperature at a ground station antenna may be estimated by: Tsky = Tmr (1 – 10–A/10) + 2.7 × 10–A/10 K (63) where: Tsky : sky noise temperature (K) at the ground station antenna A : total atmospheric attenuation excluding scintillation fading (dB) Tmr : atmospheric mean radiating temperature (K).

Proposal 3: [Option 2] RAN4 to use the following NTN UE parameters:

Table 2. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Equivalent Receiver Noise Figure | (dB) |  | 2,1 |
| Feeder loss | (dB) |  | -0,50 |

Proposal 4 (resubmitted since agreed WF R4-2306624 indicates “tentative agreement”): It is assumed for the NTN capable UE operating in above 10 GHz that the (Rx and Tx) beam pointing error are compliant with the relevant ETSI harmonized standard, e.g.

- EN 303 978, “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands”;

- EN 303 979, “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands”.

# Conclusions

Proposal 1: Do not separate Parabolic/Dish antenna or Phased-Array antenna in different NTN UE-types definitions. RAN4 to consider Parabolic/Dish antenna or Phased-Array antenna in a single common NTN UE-type definition for above 10 GHz, since similar performance characteristics.

Proposal 2: [Option 1] RAN4 to use the following NTN UE parameters:

Table 1. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Receiver noise figure | (dB) |  | 1,2 |
| Feeder loss | (dB) |  | -0,50 |
| Sky temperature | (K) |  | 30 |
| Ground temperature | (K) |  | 10 |
| Antenna temperature | (K) |  | 40 |
| G/T figure of merit | (dB/K) |  | 16,5 |

NOTE1: T_a = T_Sky + T_Ground

NOTE2: The antenna temperatures are based on e.g. ITU-R Rec. P372 and Rec. P618.

NOTE3: T_sky is computed using [ITU-R Rec. P.618-13] as expressed below

# 3 Noise temperature As attenuation increases, so does emission noise. For earth stations with low-noise front-ends, this increase of noise temperature may have a greater impact on the resulting signal-to-noise ratio than the attenuation itself. The sky noise temperature at a ground station antenna may be estimated by: Tsky = Tmr (1 – 10–A/10) + 2.7 × 10–A/10 K (63) where: Tsky : sky noise temperature (K) at the ground station antenna A : total atmospheric attenuation excluding scintillation fading (dB) Tmr : atmospheric mean radiating temperature (K).

Proposal 3: [Option 2] RAN4 to use the following NTN UE parameters:

Table 2. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Equivalent Receiver Noise Figure | (dB) |  | 2,1 |
| Feeder loss | (dB) |  | -0,50 |

Proposal 4 (resubmitted since agreed WF R4-2306624 indicates “tentative agreement”): It is assumed for the NTN capable UE operating in above 10 GHz that the (Rx and Tx) beam pointing error are compliant with the relevant ETSI harmonized standard, e.g.:

- EN 303 978, “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands”;

- EN 303 979, “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands”.

Proposal 5: RAN4 shall specify a generalized NTN UE terminal reference architecture for above 10 GHz as follows (the details can be left for implementation):

Note 1: The Up-Converter and the Tx Power Amplifier are part of the Transmission chain.

Note 2: The Rx LNA (Low-Noise Amplifier) and the Down-Converter are part of the Reception chain.

Note 3: RF represents the Radio Frequency region and IF the Intermediate Frequency region.

Note 4: DP is the Duplexer and ACU is the Antenna Control Unit.

Proposal 6: NTN UE beam steering, NTN UE beam switching capabilities, NTN UE beam switching granularity, NTN UE beam tracking capability or satellite beam tracking capability, etc. shall be left for implementation (and not part of 3GPP requirements).

Proposal 7: NTN UE beam steering, NTN UE beam switching capabilities, NTN UE beam switching granularity, NTN UE beam tracking capability or satellite beam tracking capability, etc. can be considered for implementation for example based on -3 dB NTN UE beam width and satellite constellation parameters (e.g. satellite orbit and velocity).

Proposal 8: RAN4 should specify an optional method for the NTN UE or the Network to disable NTN UE UL transmission if and when required.

Proposal 9: NTN UE power can be used to differentiate different types of NTN UE depending on the deployment scenario (e.g. land, vessel, aircraft, etc) in concordance with regulatory requirements. However, in order to simplify the normalisation of NTN UE above 10 GHz, only one single NTN UE power class could be specified in Rel-18.

Proposal 10: For NTN UE in Ka-band, RAN4 to use/cite/reference ITU recommendation S.524 for radiated off-axis requirement.

Proposal 11: Further consider the following table for discussion in RAN4:

| Question | Initial Working Hypothesis | Feedback from Companies |
| --- | --- | --- |
| Should HO aspects in Ka-band be considered for Rel-18? | Yes. Preference for considering HO aspects in Ka-band for Rel-18. | Company X: Company Y: Company Z: |
| Which is the (minimum) number of satellites to which a VSAT/NTN UE will connect in Ka-band? | At least 2 satellites in Rel-18. It is further assumed: - UE is capable of forming 1 single beam only at any given instant of time (regardless of antenna system implementation). - At least two cases should be identified based on single-beam re-pointing time between satellites - The following example cases are considered: A) 1 mechanically-steered UE beam; B) 1 electronically-steered UE beam. | Company X: Company Y: Company Z: |
| Which is the maximum (recommended) interruption time (switching time) between 2 consecutive satellites during HO procedure? | Preference for seamless HO. At physical layer, an interruption of ~20-40ms may be acceptable. No packet loss is expected. TBD | Company X: Company Y: Company Z: |
| Any view for the initial Satellite search time requirement of the NTN UE? | Implementation issue not 3GPP related. TBD | Company X: Company Y: Company Z: |
| Any view of the tracking accuracy? | Implementation issue based on regulatory requirement (see e.g. EN 303 978 and EN 303 979). TBD | Company X: Company Y: Company Z: |
| Other | TBD | Company X: Company Y: Company Z: |

Proposal 12: HandOver aspects in Ka-band shall be considered for Rel-18 as working hypothesis.

Proposal 13: RAN4 shall consider at least 2 satellites (as working hypothesis) for NTN UE HandOver procedures in Ka-band.

Proposal 14: Seamless HandOver shall be considered as working hypothesis by RAN4 for NTN UE in above 10 GHz.

Proposal 15: Initial satellite search time can be considered as implementation issue by RAN4 (no requirement for initial satellite search time shall be specifically considered by RAN4).

Proposal 16: Tracking accuracy can be considered as implementation issue by RAN4 based on regulatory requirement (relevant regulatory requirement will be referenced in the NTN UE TS, however no requirement for tracking accuracy will be specifically designed by RAN4).

# References

- R4-2220239, “WF for above 10GHz band definition and system parameters”, THALES.

- R4-2220573, “WF on NTN UE RF requirements”, ZTE.

- R4-2213361, “Discussion on Ka adjacent band NTN-TN NR coexistence scenarios”.

- TR 38.811.

- TR 38.821.

- ITU-R Rec. P.618-13.

- ITU-R Rec. S.465-6.

- R.C. Hansen “Phased Arrays” in “Antenna Engineering Handbook” by John L. Volakis, editor. McGraw Hill. 4th edition. 2007.

- R4-2302998, “WF for system parameters of above 10 GHz NTN band”, THALES.

- R4-2302527, “NTN UE Terminal Types for above 10 GHz”, THALES, Inmarsat, Hispasat.

- https://www.rfwireless-world.com/Terminology/BUC-vs-LNB.html

- https://www.idirect.net/wp-content/uploads/2021/02/OpenBMIP-Standard-version-11-T0000683-RevE-04022021.pdf

- https://www.idirect.net/wp-content/uploads/2021/02/OpenAMIP-Standard-Revision-E.pdf

- https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_106/Inbox/Chair_Notes/Main_Session_Notes

- FCC 25.218: https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25/subpart-C/section-25.218#p-25.218(c)(1)

- ITU-R Rec. S.524-9: https://www.itu.int/rec/R-REC-S.524/en

-END-