3GPP TSG-RAN WG4 Meeting #107                           		R4-2309732

# Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	TP to TS 36.181: test models clarifications (4.9)

Agenda Item:	9.7.3.5

Document for:	Approval

# Conclusions

Motivation for this TP is to clarify the intended use of the Test models – so far it is unclear in section 4.9.2.2 what the needs for all those TMs is.

Proposal 1: Approve the attached TP to TS 36.181.

# TP to TS 36.181

------------------------------ Modified section ------------------------------

##### 4.9.2.2.1	E-UTRA Test Model 1.1 (E-SAN-TM1.1)

The test model in E-TM1.1 in TS36.141 [xx] apply for E-SAN-TM1.1 and carrier bandwidth is only limited to 1.4MHz.

This model shall be used for tests on:

-	SAN output power

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

##### 4.9.2.2.2	E-UTRA Test Model 1.2 (E-SAN-TM1.2)

The test model in E-TM1.2 in TS36.141 [xx] apply for E-SAN-TM1.2 and carrier bandwidth is only limited to 1.4MHz.

This model shall be used for tests on:

-	Unwanted emissions

-	ACLR

-	Operating band unwanted emissions

##### 4.9.2.2.3	E-UTRA Test Model 2 (E-SAN-TM2)

The test model in E-TM2 in TS36.141 [xx] apply for E-SAN-TM2 and carrier bandwidth is only limited to 1.4MHz.

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

##### 4.9.2.2.4	E-UTRA Test Model 3.1 (E-SAN-TM3.1)

The test model in E-TM3.1 in TS36.141 [xx] apply for E-SAN-TM3.1 and carrier bandwidth is only limited to 1.4MHz.

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation (at max power)

##### 4.9.2.2.5	E-UTRA Test Model 3.2 (E-SAN-TM3.2)

The test model in E-TM3.2 in TS36.141 [xx] apply for E-SAN-TM3.2 and carrier bandwidth is only limited to 1.4MHz.

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for 16QAM modulation

##### 4.9.2.2.6	E-UTRA Test Model 3.3 (E-SAN-TM3.3)

The test model in E-TM3.3 in TS36.141 [xx] apply for E-SAN-TM3.3 and carrier bandwidth is only limited to 1.4MHz.

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for QPSK modulation

------------------------------ End of modified section ------------------------------