3GPP TSG-RAN WG4 Meeting #107                                           R4-2310276

Incheon, South Korea, May 22nd – 26th, 2023

Title:	draft LS on lower MSD capability

Response to:

Release:	Rel-18

Work Item:	NR_ENDC_RF_FR1_enh2

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: 	Liu Ye

E-mail Address: 	leo.liuye@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

After sending the LS on lower MSD to RAN2 in R4-2306594, RAN4 continues the discussion and reached some further agreements. To facilitate RAN2 discussion on the signalling design, RAN4 would like to share the progress with RAN2 in this follow up LS.

- MSD for different power classes

-	UE reports the lower MSD capability class per MSD type for the highest supported power class for the band combination

▪	UE can additionally report lower MSD capability class per MSD type for other power classes if requested by the network/regulator

- MSD orders

-	No need to report the order for harmonic/ harmonic mixing/cross band isolation MSD types

▪	Lower MSD capability class reported should apply for all specified orders of the MSD type

-	IMD order is up to 5 in Rel-18

- MSD types

-  The following MSD types/orders are agreed to be reported based on the existing spec

▪	Harmonic, harmonic mixing, crossband isolation, IMD with order=2/3/4/5

-  It is agreed to add a new special lower MSD type as “ALL”, which means all above mentioned MSD types/orders could meet the reported lower MSD threshold/lower MSD capability class

▪	FFS on details of the “ALL” type

-  Other new MSD types may be added later

- Candidate MSD thresholds

-  The maximum threshold is around 20dB

▪	FFS on the concrete values for thresholds

▪	FFS on whether 2 or 3 bits will be used for threshold range

- Whether to report CBW of aggressor UL and victim DL

-  Channel bandwidth of aggressor UL and victim DL are not necessary to be included in the essential information for lower MSD capability

RAN4 is still working on the details for approaches for a UE to indicate the improved lower MSD performance. If any update, RAN4 will keep RAN2 informed.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above information into account for the following RAN2 work on the signalling design for lower MSD.

3. Date of Next TSG WG RAN4 Meetings:

RAN WG4 Meeting #108						Aug. 21 – Aug. 25, 2023								Toulouse, FR

RAN WG4 Meeting #109						Oct. 09 – Oct. 13, 2023								Xiamen, CN