3GPP RAN WG4 Meeting #108		R4-2311233

Toulouse, France, August 21-25, 2023

Agenda item:	8.20.1

Source:	Apple

Title:	Rationale for LP-WUS separate band

Work Item:	FS_NR_LPWUS

Document for:	Discussion

# 1	Introduction

The study on low-power Wake-up Signal and Receiver for NR has been added to the 3GPP Rel-18 work plan with the following objectives [1]:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

Over the past several meetings, RAN4 has been discussing aspects related to the RAN1 LS on LP-WUS [2], with the latest agreements captured in [3-8].

This contribution provides the rationale for the separate band WUS system architecture.

# 2	Discussion

Dedicated spectrum for the WUS can enable a low-complexity UE receiver and avoid reducing the spectral efficiency of cellular communications by not configuring the WUS in-channel with the wanted cellular data.  A reuse of spectrum licensed for broadcast operation can be a viable scenario.  Potential spectrum that can be used for WUS broadcast is the TV whitespaces spectrum.  TV channels 19 through 36 span the 500 – 608 MHz frequency range.  Each TV channel is 6 MHz wide with 0.309 MHz guard band on each side.  In total, there are 11.14 MHz available with this guard band spectrum.  Additionally, the band plans discussed in the LTE 6G broadcast work item can be considered for this purpose, enabling broadcast operators to make choices about the services and business models they wish to pursue with their spectrum [9]:

# Band plan From [1] two band plan options not mutually exclusive were identified - Define a single band covering the entire UHF frequency range 470 – [698/702] MHz - The requirements are relaxed for this band (TBD which requirements and how much) and/or - This band is assumed to be implemented with more than one filter in the UE (TBD how many filters and the characteristics of those filters) - Define smaller bands - At least one of the bands overlaps with Band n105 DL to reuse n105 Rx filter - Specify the following bands 470-542 MHz, 540-606 MHz and 602-702 MHz. - Other Agreement from online session: - Define the band plan based on available data for BS and UE. - Full UHF band, i.e., 470 – 698MHz, - with reduced UE blocking - Narrow band, i.e., 612 – 652 MHz, which is the same as the downlink operating band of n105. - If data for other frequency ranges becomes available before the conclusion of the work item, it can be considered either as a separate band or as a modification to initially defined bands. Way Forward - Further clarification and study is needed to elaborate on and quantify requirements with respect to reduced UE blocking for the full UHF band. - All technical requirements for these bands are still to be defined.

Consulting ITU agreements, we refer to the following summary of digital audio broadcasting systems [10]:

This background material provides the context for the spectrum feasibility of operating a broadcast service dedicated to transmitting wake-up signals for cellular UEs

A wake-up signal (WUS) broadcast network can be overlaid over a cellular network, as shown in Figure 1 below.

Figure 1: Potential WUS network layout

The WUS network can utilize low-frequency spectrum and existing high-power broadcasting infrastructure to cover multiple mobile broadband (MBB) cells with one WUS node.  In this model, the WUS broadcast network and the cellular MBB network could belong to the same operator or different operators.

To further rationalize the potential of a separate band WUS network architecture, we consider the potential convergence between the tasks of broadcasting wake-up signals and broadcasting other multicast information to users in the network.  With this convergence, broadcast nodes may be utilized to transmit wake-up signals, emergency messages, and other broadcast information to users.

Proposal 1:	RAN4 shall continue to study the scenario of WUS in a separate band from the main radio.

# 3	Conclusions

This contribution provides the rationale for the separate band WUS system architecture, with the following proposal:

Proposal 1:	RAN4 shall continue to study the scenario of WUS in a separate band from the main radio.

# 4	References

- RP-222644, “Study on low-power Wake-up Signal and Receiver for NR,” vivo, 3GPP RAN #97, September 2022

- R1-2212999, “LS to RAN4 on low-power wake-up receiver architectures,” 3GPP RAN1 #111, November 2022

- R4-2303711, “WF on LP-WUS UE RF,” vivo, 3GPP RAN4 #106, February 2023

- R4-2303712, “Reply LS to RAN1 on LP-WUR architectures,” 3GPP RAN4 #106, February 2023

- R4-2303489, “WF on RF requirements for 5G Broadcast,” Qualcomm, 3GPP RAN4 #106, February 2023

- R4-2306617, “WF on LP-WUS,” vivo, 3GPP RAN4 #106bis, April 2023

- R1-2304251, “Reply LS to RAN4 on LP WUR architectures,” 3GPP RAN1 #112bis, April 2023

- R4-2310299, “WF on LP-WUS SI,” vivo, 3GPP RAN4 #107, May 2023

- R4-2303489, “WF on RF requirements for 5G Broadcast,” Qualcomm, 3GPP RAN4 #106, February 2023

- “Trends in broadcasting: an overview of development,” https://www.itu.int/en/ITU-D/Technology/Documents/Broadcasting/TrendsinBroadcasting.pdf