3GPP TSG-RAN WG4#108bis	R4-2316268

Xiamen, China, October 9th – 13th, 2023

Agenda item:	4.1.1.2

Source: 	Qualcomm France

Title: 	MSD for CA_n12A-n14A

Document for:	Approval

# Introduction

Analysis and proposals on CA_n12A-n14A MSD are provided in this contribution.

# Considerations

The following test point is used in the analysis.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n12 | n14 | 708.5 | 15 | 15 | 20 (RBstart=59) | 760.5 | 5 | TBD | >ACLR2 |
| n14 | n12 | 792 | 10 | 15 | 20 (RBstart=0) | 743.5 | 5 | TBD | >ACLR2 |

We used the conventional 3GPP assumptions for the TX for the PA noise measurements, as well as in factoring the CIM3/CIM5 contributions.

- 20 MHz 100RB DFT-s-OFDM

- 30dBc ACLR

- 4dB Post-PA loss

- Image -28dB

- LO -28dBc

- CIM3 -60dBc

- CIM5 -70dBc

The parameters and MSD results are shown below.

# Conclusion

MSD analysis was conducted for CA_n12A-n14A.

Proposal 1: Use the following analysis results when discussing requirements for CA_n12A-n14A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n12 | n14 | 708.5 | 15 | 15 | 20 (RBstart=59) | 760.5 | 5 | 3.3 | >ACLR2 |
| n14 | n12 | 792 | 10 | 15 | 20 (RBstart=0) | 743.5 | 5 | 3.4 | >ACLR2 |

# Reference