3GPP TSG-RAN WG4 Meeting #108bis	R4-231xxxx

Xiamen, China, Oct. 9 ‒ Oct. 13, 2023

Title: 	WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements

Agenda Item:	5.36.2

Source: 	Ericsson

Document for:	Approval

# Topic #1: Work plan

Work plan

Agreement:

Target completion date: June 2024. RAN4#108bis (Oct 2023): - Discussion on test scope, simulation assumptions and test parameters. RAN4#109 (Nov 2023): - Finalize the test scope. - Agree on part of baseline assumptions, test parameters and test metrics. RAN4#110 (Feb 2024): - Agree on baseline assumptions, test parameters and test metrics. - Collection of initial simulation results. - CR work split (if necessary). RAN4#110bis (Apr 2024): - Collection of updated & additional simulation results if any. - Review draft CRs. RAN4#111 (May 2024): - Agreement on the test requirement value. - Endorse draft CRs.

# Topic #2: NR PDCCH reception in symbols with LTE CRS REs

Whether to define PDCCH demodulation requirements for Rel-18 eDSS feature

Agreement:

- Define PDCCH demodulation requirements for eDSS

Evaluation scenario

Agreement:

- 2-symbol CORESET

Symbol #0: LTE PDCCH/PCFICH Symbol #1: NR PDCCH overlapping with LTE CRS Symbol #2: NR PDCCH

- Common assumptions

  - Single non-overlapping CRS rate matching patterns for PDCCH demodulation.

  - 4 CRS ports for LTE

  - LTE PDCCH/PCFICH is transmitted in symbol #0

gNB assumption for PDCCH symbols overlapped with LTE CRS

Agreement:

- PDCCH data and DMRS REs overlapped with LTE CRS are punctured.

UE receiver assumption (e.g., channel estimation)

Agreement:

- PDCCH channel estimation is assumed to use only the clean PDCCH symbol.

Simulation assumption for evaluation

Way forward:

- FFS on the parameter configuration for the PDCCH

  - Option 1

| - Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLC300-100 |
| Antenna configuration | 1x2, 1x4; 2x2, 2x4 |
| CRS | 4 port CRS without power boosted |
| DCI payload (excluding CRC) | 60 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 REGs |

| Test number | Bandwidth (MHz) | Symbols with PDCCH | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 2 | 4 | TDLA30-10 | 1x2, 1x4 Low | 1 |  |
| 2 | 10 | 2 | 8 | TDLC300-100 | 2x2, 2x4 Low | 1 |  |

  - Option 2

    - Cover all PDCCH Aggregation levels for evaluation purpose. (Not for requirements definition purpose)

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Bandwidth | 20MHz |
| LTE Bandwidth | 20MHz |
| Channel model | TDL-C 300-100 |
| Correlation | Low |
| Number of BS antennas | 4Tx |
| Number of UE antennas | 2 Rx |
| DCI payload (excluding CRC) | 60 bits |
| Interleaving | Non-Interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBs |
| CRS | single 4 port CRS pattern |
| Aggregation level | 1,2,4,8,16 |
| CORESET | Scenario a): CORESET: 2nd and 3rd symbols, LTE: 4 CRS Ports Scenario b): CORESET: 2nd symbol, LTE: 4 CRS Ports |
| Channel estimation | Scenario a): Use clean symbol Scenario b): Use legacy channel estimation |
| Transmitter | DMRS REs and data REs overlapped with CRS are punctured |
| Receiver | Puncture the CRS REs |
| Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE | 3dB |

# Topic #3: PDSCH demodulation requirements with two overlapping CRS rate matching patterns

Whether to define the PDSCH demodulation requirements with two overlapping CRS rate matching patterns

Agreement:

- Not define PDSCH demodulation requirements.