3GPP TSG-RAN WG4 Meeting RAN4#109      				R4-2318899 Chicago, U.S.A.

November 13 – 17, 2023

Agenda Item:	8.22.2.6

Source:	Lenovo, Motorola Mobility

Title:	Correction of Carrier Phase Measurement Errors Due to Carrier Frequency Offsets

Document for:  Approval

- Introduction

The effect of carrier frequency offset on carrier phase measurements was discussed previously in contributions [1-3].  In [2], it was shown that if the frequency offset of the TRP relative to the PRU is different than the frequency offset of the TRP relative to the UE, then the carrier phase difference measurements will have a mean error which depends on the difference of these carrier frequency offsets and the number of symbols separating the DL-PRS within the slot used for carrier phase difference measurement.

In [2], it was shown that the mean error in the carrier phase difference measurement due to carrier frequency offsets is given by

n Δφm,UETX- Δφm,PRUTX

where Δφm,UETX and Δφm,UETX denote the phase rotation over one symbol of the carrier of the m-th TRP relative to the UE and the PRU, respectively, and n is the difference in the symbol indices of the two DL-PRS symbols.  Here it is assumed that positioning reference symbol received from the m-th TRP is received n symbols later than the positioning reference symbol received from the l-th TRP as in Figure 1.

Figure 1: TRP m DL-PRS located n = 4 symbols after TRP l DL-PRS

To allow the LMF to correct for this error, it was proposed in [2] that the carrier frequency offsets should be measured by both the UE and the PRU and that these carrier frequency offsets should be reported to the LMF.  Additionally, it was proposed that the UE and the PRU report the number of symbols separating the DL-PRS symbols on which the phase or phase difference measurements were taken.

In [3], an alternative approach was proposed for removing the carrier phase offsets due to carrier frequency offsets without the need to report the carrier frequency offsets to the LMF.  With this alternative approach, a common reference time is defined for the phase measurements and the carrier phase offsets due to the carrier frequency offsets in the time interval between the reference time and the DL-PRS measurement are removed from the carrier phase measurements.

In previous contributions [2,3], it was assumed that the UE and the PRU take DL-PRS measurements in the same time slot.  However, due to scheduling or other restrictions, it may not always be possible for these measurements to be taken simultaneously. In this contribution, we consider the impact of carrier frequency offset in the case that the measurements taken by the UE and the PRU are offset in time.  We further consider how a reference can be defined for a measurement time window to remove errors due to the carrier frequency offsets before the measurements are reported to the LMF.

- Carrier Frequency Offset

Carrier phase positioning assumes the existence of initial phase offsets among the DL-PRS transmitters, i.e., gNBs, and between the DL-PRS transmitters and the UE and PRU receivers which are static.  In general, however, there will be frequency offsets among the TRP transmitters which will cause the carrier phase offsets to change relative to each other over time.  Similarly, there will be carrier frequency offsets between the TRP transmitters and the UE and PRU receivers which will cause these phase offsets to change over time.

The carrier frequency tolerance for the base station is defined in TS 38.104 and is given in the following two tables:

Table 6.5.1.2-1: Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

Table 9.6.1.3-1: OTA frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±0.05 ppm |
| Medium Range BS | ±0.1 ppm |
| Local Area BS | ±0.1 ppm |

The carrier frequency tolerance for the UE is defined in TS 38.101-1 for FR1.  In section 6.4.1 of TS 38.101-1, we have the requirement:

The UE basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the NR Node B.

In [2], the effect of carrier frequency offset on the carrier phase difference measurement when the UE and the PRU take measurements on the same positioning subframe as in Figure 1.  For this case, it was shown that the mean error in the carrier phase difference measurement between two TRP’s, l and m (in the same subcarrier), due to carrier frequency offsets is given by

n Δφm,UETX- Δφm,PRUTX

where Δφm,UETX and Δφm,UETX denote the phase rotation over one symbol of the carrier of the m-th TRP relative to the UE and the PRU, respectively, and n is the difference in the symbol indices of the two DL-PRS symbols. Here it is assumed that positioning reference symbol received from the m-th TRP is received n symbols later than the positioning reference symbol received from the l-th TRP.

In the more general case, the UE and the PRU may not be able to take measurements on the same DL-PRS subframes.  In Figure 2, the PRU takes phase measurements on DL-PRS from l-th and m-th TRP’s in the first slot, while the UE takes measurements on these same DL-PRS in the N+1-th slot.  As is shown in the Appendix, the mean error in the carrier phase difference measurement between two TRP’s, l and m (on the same subcarrier), due to carrier frequency offsets is given by

N L Δφm,lTX+n Δφm,UETX- Δφm,PRUTX .

where N is the number of slots separating the measurements, L is the number of symbols per slot, and n is the number of symbols separating the PRS symbols within the slot.

Figure 2: PRU and UE DL-PRS measurements separated by N slots

Given that the frequency error of the TRP, the UE, and the PRU are all limited to ± 0.1 ppm, the maximum frequency difference between the TRP and the UE and between the TRP and the PRU are both limited to 0.2 ppm.  In the case that the UE receiver carrier frequency is at the range minimum and the PRU receiver carrier frequency is at the range maximum, this mean error term can be expressed in degrees as

n Δφm,UETX- Δφm,PRSTX≤n Ts∙2∙10-7 fc ∙360 .

If additionally, it is assumed the transmitter frequency of the m-th TRP is at the range maximum and the transmitter frequency of the l-th TRP is at the range minimum, then it follows that

N L Δφm,lTX≤N L Ts∙2∙10-7 fc ∙360 ,

so that finally we have

N L Δφm,lTX+n Δφm,UETX- Δφm,PRUTX≤N L+n Ts∙2∙10-7 fc ∙360,

where N is the number of slots separating the measurements by the PRU and the UE, L is the number of symbols per slot, and n is the number of symbols separating the DL-PRS symbols from the l-th and m-th TRP (within the same time slot).

Figure 3:  Maximum mean phase difference measurement error vs. separation of DL-PRS symbols (UE and PRU take measurements in the same slot)

The maximum value of the mean error of the carrier phase difference measurement as a function of the carrier frequency is shown in Figure 3 and Figure 4.  In Figure 3, it is assumed that the PRU and UE take measurements on the same DL-PRS slot so that N = 0.  Conversely, in Figure 4 it is assumed that the symbol separation within the time slot so that n = 1, and the number of slots separating the PRU and UE DL-PRS measurements is varied from N = 0 to N = 5.  From these figures, it is apparent that the maximum value of the mean error will be very large if the UE and the PRU take measurements on different slots and these errors are not corrected.

Figure 4:  Maximum mean phase difference measurement error vs. number of slots separating PRU and UE measurements

- Correction of Errors Due to Carrier Frequency Offsets

As seen in the previous section, carrier frequency offsets can lead to very large carrier phase difference measurement errors.  As a result, these errors must be corrected if the measurements are to be meaningful.  In [2], it was proposed that the carrier phase difference measurement errors due to carrier frequency offsets be corrected at the LMF.  To allow the LMF to correct for these errors, it was proposed that the carrier frequency offsets should be measured by both the UE and the PRU and that these carrier frequency offsets should be reported to the LMF.

A drawback with this approach is the bandwidth required to signal the carrier frequency offsets back to the LMF.  Furthermore, since the carrier frequency offsets will change over time, it may be necessary to report the carrier frequency offsets each time that carrier phase or carrier phase difference measurements are sent to the LMF.  Finally, if carrier phase is reported, then the symbol index must also be reported, and alternatively if carrier phase difference is reported, then the difference in the symbol indices of the positioning reference symbols used to generate the difference must be reported.

  - Phase Measurements Referred to a Common Time Reference

    - PRU and UE Measurements in the Same Slot

An alternative approach was proposed in [3] in which carrier phase measurements are referred to a common reference time as shown in Figure 5 below.  The steps are the following:

- A common reference time for measurements is defined.  In Figure 5, the common reference time is the first symbol of the slot.

- The carrier frequency offset is estimated by the UE and the PRU for each TRP for which DL-PRS phase measurements are taken.  Let Δωl,UETX and Δωl,PRSTX denote the carrier frequency offset of the l-th TRP relative to the UE and the PRU, respectively.  The symbol-to-symbol phase rotations are then given by

Δφl,UETX= Δωl,UETX Ts

and

Δφl,PRUTX= Δωl,PRUTX Ts

for the UE and the PRU, respectively.

- The DL-PRS phase measurement is referred to the common reference time by subtracting the phase rotation due to the carrier frequency offset in the time interval since the common reference time.

Let θi,k,l denote the UE’s carrier phase measurement for the DL-PRS received from the l-th TRP in subcarrier i with symbol index k.  The referred carrier phase measurement for the UE is then given by

θi,k,l=modθi,k,l-k Δφl,UETX, 2π  .

Similarly, let Ωi,k,l denote the PRU’s carrier phase measurement for the DL-PRS received from the l-th TRP in subcarrier i with symbol index k.  The referred carrier phase measurement for the PRU is then given by

Ωi,k,l=modΩi,k,l-k Δφl,PRUTX, 2π  .

- The carrier phase difference is computed as the difference between the referred carrier phase measurements.  For the UE, the carrier phase difference between the DL-PRS received from TRP l on subcarrier i with symbol index k and the DL-PRS received from TRP m on subcarrier i with symbol index k+n is defined as

modθi,k,l-θi,k+n,m, 2π .

For the PRU and the same pair of DL-PRS, the carrier phase difference is defined as

modΩi,k,l-Ωi,k+n,m, 2π  .

- Either the referred carrier phase measurements or the differences between these referred carrier phase measurements are reported to the LMF.

In the Appendix of [3], it is shown that by referring the carrier phase measurements to a common reference time, the effect of the carrier frequency offsets are removed so long as the estimates of the carrier frequency offsets are accurate.

Figure 5: Example of referring carrier phase measurements to a common reference time

An additional advantage of referring the carrier phase measurements to a common reference time is that referred carrier phase measurements for DL-PRS received from the same TRP on the same subcarrier can be averaged. Due to the symbol-to-symbol phase rotations of Δφl,UETX at the UE, carrier phase measurements on DL-PRS which are n symbols apart will have a mean phase offset of n Δφl,UETX and thus should not be averaged without first referring to a common reference time.

In Figure 5, an example is provided illustrating how carrier phase measurements can be referred to a common reference time.  In this example, the common reference time for carrier phase measurements is the first symbol of the slot.  The measurement of the DL-PRS in subcarrier i with symbol index 2 from TRP l is referred to the first symbol of the slot by subtracting twice the symbol-to-symbol phase rotation due to its carrier frequency offset, so that

θi,2,l=modθi,2,l-2 Δφl,UETX, 2π

Similarly, the measurement of the DL-PRS in subcarrier i with symbol index 6 from TRP m is referred to the first symbol of the slot by subtracting six times the symbol-to-symbol phase rotation due to its carrier frequency offset and is given by

θi,6,m=modθi,6,m-6 Δφm,UETX, 2π

In summary, to correct for carrier phase difference measurement errors due to carrier frequency offsets without the need to signal these carrier frequency offsets and the indices of the measured symbols to the LMU, we have the following proposals:

Proposal 1:	Define a common reference time and refer the DL-PRS carrier phase measurements to this reference time by subtracting the phase rotation due to the carrier frequency offset in the time interval between the DL-PRS and the reference time from the carrier phase measurement.

Proposal 2:	Define the referred carrier phase difference as the difference between the referred carrier phase measurements.

Proposal 3:	Define the same common reference time for the UE and the PRU.

Proposal 4:	The UE and the PRU report either the referred carrier phase measurements or the carrier phase difference measurements computed using the referred carrier phase measurements.

    - PRU and UE Measurements in Different Slots

As discussed in Section 2, in the general case the UE and the PRU may not be able to take measurements on the same DL-PRS subframes.  The most general case is shown in Figure 6 below and in the Appendix. In this figure, the symbol indices k and k' can be any integer and are not limited to the number of symbols per slot and are used to number the symbols consecutively across slot boundaries.

Figure 6: PRU and UE DL-PRS measurements referred to a common reference time

Refer to the example of Figure A2 above.  In this example, the carrier phase measurements will be referred to symbol within a slot between slot 1 and slot N+1. Let kref denote the symbol index to which the measurements are to be referred. Typically, kref will be the first symbol of a slot.

As in the previous section in which the UE and the PRU take measurements in the same slot, let θi,k,l denote the UE’s carrier phase measurement for the DL-PRS received from the l-th TRP in subcarrier i with symbol index k.  The referred carrier phase measurement for the UE is then given by

θi,k,l=modθi,k,l-k-kref Δφl,UETX, 2π  .

Similarly, let Ωi,k,l denote the PRU’s carrier phase measurement for the DL-PRS received from the l-th TRP in subcarrier i with symbol index k.  The referred carrier phase measurement for the PRU is then given by

Ωi,k,l=modΩi,k,l-k-kref Δφl,PRUTX, 2π  .

By using the referred carrier phase measurements rather than the carrier phase measurements, the carrier phase difference measurements errors due to carrier frequency offsets are removed. As a result, the same Proposals 1-4 in the previous section can be used to remove the errors due to carrier frequency offsets when the UE and PRU take measurements in different slots.

- Summary

To correct for carrier phase difference measurement errors due to carrier frequency offsets without the need to signal these carrier frequency offsets and the indices of the measured symbols to the LMU, we have the following proposals:

Proposal 1:	Define a common reference time and refer the DL-PRS carrier phase measurements to this reference time by subtracting the phase rotation due to the carrier frequency offset in the time interval between the DL-PRS and the reference time from the carrier phase measurement.

Proposal 2:	Define the referred carrier phase difference as the difference between the referred carrier phase measurements.

Proposal 3:	Define the same common reference time for the UE and the PRU.

Proposal 4:	The UE and the PRU report either the referred carrier phase measurements or the carrier phase difference measurements computed using the referred carrier phase measurements.

# References

- R1-2204807, “Design Aspects of Carrier Phase Measurements for NR Positioning”, Intel, #109-e.

- R4-2311991, “Effect of Carrier Frequency Offset on Carrier Phase Positioning,” Lenovo, Motorola Mobility, RAN4#108.

- R4-2316796,” Carrier Frequency Offset Correction for CPP,” Lenovo, Motorola Mobility, RAN4#108bis.

- 3GPP TS 38.104 NR; Base Station (BS) radio transmission and reception

- 3GPP TS 38.101-1 User Equipment radio transmission and reception; Part 1: Range1 Standalone

Appendix 1: CPD Measurement Errors Due to CFO with PRU and UE Measurements in Different Slots

To examine the effect of carrier frequency offset, we consider the simplest case in which two TRP’s with indices l and m, are transmitting DL-PRS and the channels between each of two TRP’s and the UE are assumed to be line-of-sight (no multipath).  We further assume that the carrier transmitted by each TRP has a fixed frequency offset (within the required ± 0.1 ppm frequency tolerance) relative to the frequency of the serving cell of the UE.

Let yi,k,l denote the demodulated DL-PRS symbol received from TRP l on the i-th subcarrier and k-th symbol which is given by

yi,k,l=ri,k,l ej-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0+ni,k ,

where ri,k,l denotes the known reference symbol, ωc denotes the carrier frequency (in radians), Δω denotes the subcarrier spacing, Δωl,UETX denotes the frequency offset of the carrier of the l-th TRP relative to that of the UE receiver, τl=dlc denotes the propagation delay corresponding to the distance dl between the l-th TRP and the UE, φUERX0 denotes the initial phase of the UE carrier, φlTX0 denotes the initial phase (for n=0) of the transmitter carrier for the l-th TRP, and ni,k is the additive noise at the output of the demodulator for the k-th symbol of the i-th subcarrier.

The phase increment Δφl,UETX denotes the change in the phase of the carrier of the l-th TRP relative to that of the UE carrier over the period of one symbol given by

Δφl,UETX= Δωl,UETX Ts,

where Ts is the symbol period.  The phase term Δωl,UETXπ Δω is due to the frequency offset of the subcarrier with the correlator demodulator and is computed as

argΔω2π02πΔωej Δωl,UETX t dt .

It can be noted that the term Δωl,UETX τl due to carrier frequency offset was considered previously in [1].

Multiplying yi,k,l by the conjugate ri,k,l* of the known positioning reference symbol for the l-th TRP (assumed to have unit amplitude) yields

wi,k,l=yi,k,l ri,k,l*=ej-ωc+i Δω+Δωl,UETX τl+Δωl,UETXπ Δω+φlTX0+k Δφl,UETX-φUERX0+ri,k,l* ni,k.

Similarly, for the i-th subcarrier of the k+n-th symbol received from TRP m, we have

wi,k+n,m=yi,k+n,m ri,k+n,m*=ej-ωc+i Δω+Δωm,UETX τm+Δωm,UETXπ Δω+φmTX0+k+n Δφm,UETX-φUERX0+ri,k+n,m* ni,k+n

Multiplying wi,k+n,m by the conjugate of wi,k,l yields

wi,k,l*wi,k+n,m=ej-ωc+i Δω τm-τl-Δωm,UETXτm+Δωl,UETXτl+Δωm,UETXπ Δω-Δωl,UETXπ Δω+φmTX0-φlTX0+ k Δφm,UETX-Δφl,UETX+ n Δφm,UETX+ejωc+i Δω τl+Δωl,UETXτl- Δωl,UETXπ Δω-φlTX0-k Δφl,UETX+φUERX0 ri,k+n,m* ni,k+n+ej-ωc+i Δω τm-Δωm,UETXτm+Δωm,UETXπ Δω+φmTX0+k+n Δφm,UETX-φUERX0ri,k,lni,k*+ri,k,lni,k* ri,k+n,m* ni,k+n,

=ej-ωc+i Δω τm-τl-Δωm,UETXτm+Δωl,UETXτl+Δωm,UETXπ Δω-Δωl,UETXπ Δω+φmTX0-φlTX0+k Δφm,UETX-Δφl,UETX+ n Δφm,UETX        +ni,k,k+n,l,m,

where the term

Δωm,UETXπ Δω-Δωl,UETXπ Δω+φmTX0-φlTX0+k Δφm,UETX-Δφl,UETX+ n Δφm,UETX

depends on the initial phases of the TRP transmitters and the carrier frequency offsets of the TRP transmitters relative to the UE receiver and is unknown to the UE.  The term

-Δωm,UETXτm+Δωl,UETXτl,

depends on both the carrier frequency offsets and the propagation delays τm and τl and is also unknown to the UE.  The noise term ni,k,k+n,l,m is implicitly defined in the above as the sum of three noise terms.

We define the term φm,lTX0 as the difference between the initial phase offsets of the m-th and l-th TRP’s given by

φm,lTX0=φmTX0-φlTX0 ,

the term Δωm,lTX  as the difference between the frequency offsets of the m-th and l-th TRP’s relative to the UE given by

Δωm,lTX=Δωm,UETX-Δωl,UETX ,

and Δφm,lTX as the change in the phase of the carrier of the m-th TRP relative to that of the l-th TRP over the period of one symbol given by

Δφm,lTX=Δφm,UETX-Δφl,UETX .

With this notation, we have

wi,k,l*wi,k+n,m=ej-ωc+i Δω τm-τl-Δωm,UETXτm+Δωl,UETXτl+Δωm,lTXπ Δω +φm,lTX0+k Δφm,lTX+ n Δφm,UETX+ni,k,k+n,l,m .

It should be noted that the term Δφm,UETX depends on the frequency offset of the m-th TRP to the carrier frequency of the UE.

Figure A1: PRU and UE DL-PRS measurements separated by N slots

We now consider the possibility that the PRU in an earlier slot as in Figure A1.  In the following the indices k and k' can be any integer and are not limited to the number of symbols per slot and are used to to number the symbols consecutively across slot boundaries.  The indices n and n' denote the offsets of the second DL-PRS symbol relative to the first DL-PRS in the slots measured by the UE and PRU, respectively.

Let L denote the number of symbols per slot and let N denote the number of slots separating the measurements taken by the PRU and the UE as in Figure X below.

ui,k',l*ui,k'+n',m=ej-ωc+i Δω τm'-τl'-Δωm,PRSTXτm'+Δωl,PRSTXτl'+Δωm,lTXπ Δω+φm,lTX0+k' Δφm,lTX+ n' Δφm,PRUTX+n'i,k',k'+n',l,m .

where ui,k',l and ui,k'+n',m are defined in the same manner as the measurements wi,k,l and wi,k+n,m taken at the UE, and the delays τm' and τl' between the TRP’s and the PRU are known. The phase increment Δφm,PRUTX denotes the change in the phase of the carrier frequency of the m-th TRP relative to that of the PRU receiver over the period of one symbol.

As noted in [1], the terms

-Δωm,UETXτm+Δωl,UETXτl , and

-Δωm,PRSTXτm'+Δωl,PRSTXτl'

are very small both in absolute terms and relative to the other terms.  In particular

-Δωm,UETXτm+Δωl,UETXτl≤foffset,maxτm+τl

-Δωm,PRSTXτm'+Δωl,PRSTXτl'≤foffset,maxτm'+τl' .

For a carrier frequency of 5 GHz with a frequency error of 0.1 ppm and a maximum range of 200 meters, the corresponding phase term is given by

2 π ∙ 5∙109∙10-7∙200+2003∙108=0.004 radians=0.24 degrees ,

and as a result, these terms can be ignored.

With this approximation, we have

wi,k,l*wi,k+n,m≈ej-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,UETX+ni,k,k+n,l,m ,

and

ui,k',l*ui,k'+n',m≈ej-ωc+i Δω τm'-τl'+Δωm,lTXπ Δω+φm,lTX0+k'Δφm,lTX+ n'Δφm,PRUTX++n'i,k',k'+n',l,m .

With knowledge of the delays τl' and τm' from the known locations of the TRP’s and the PRU, the LMF can determine the phase term due to the differential propagation delay between the TRP’s and the PRU given by

ωc+i Δω τm'-τl' .

With this knowledge, the LMF can use ui,k,l*ui,k+n,m to estimate the remaining phase term given by

Δωm,lTXπ Δω+φm,lTX0+k' Δφm,lTX+ n' Δφm,PRUTX  .

It should be noted that the remaining phase term is not solely dependent on the difference of the initial phase offsets given by the term φm,lTX0, but is also dependent on the frequency offsets and the locations of the measured positioning reference symbols.

If we define the arg∙ function to have range in the interval 0, 2π, then

argwi,k,l*wi,k+n,m

=argej-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,UETX+ni,k,k+n,l,m

=modargej-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,UETX

+ arg1+ni,k,k+n,l,me-j-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,UETX,2π

=modmod-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+n Δφm,UETX,2π

+ arg1+ni,k,k+n,l,me-j-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+ n Δφm,UETX,2π .

Since the mean of ni,k,k+n,l,m is zero, the expected value of this last “arg” term is zero and

Eargwi,k,l*wi,k+n,m

=mod-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+n Δφm,UETX,2π .

Subtraction of the PRU-based estimate of the transmitter phase terms yields

modEargwi,k,l*wi,k+n,m-Δωm,lTXπ Δω+φm,lTX0+k' Δφm,lTX+ n' Δφm,PRUTX ,2π

=modmod-ωc+i Δω τm-τl+Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+n Δφm,UETX,2π-Δωm,lTXπ Δω+φm,lTX0+k' Δφm,lTX+ n' Δφm,PRUTX ,2π .

If we assume that the PRU-based estimate of the transmitter phase term is equal to the actual value so that

Δωm,lTXπ Δω+φm,lTX0+k' Δφm,lTX+ n' Δφm,PRUTX =Δωm,lTXπ Δω+φm,lTX0+k' Δφm,lTX+ n' Δφm,PRUTX ,

then

Δωm,lTXπ Δω+φm,lTX0+k Δφm,lTX+n Δφm,UETX-Δωm,lTXπ Δω+φm,lTX0+k' Δφm,lTX+ n' Δφm,PRUTX

=k Δφm,lTX+n Δφm,UETX-k' Δφm,lTX+ n' Δφm,PRUTX

=k-k' Δφm,lTX+n Δφm,UETX- n' Δφm,PRUTX .

As a result,

modEargwi,k,l*wi,k+n,m-Δωm,lTXπ Δω+φm,lTX0+k' Δφm,lTX+ n' Δφm,PRUTX ,2π

≈modωc+i Δω τl-τm+k-k' Δφm,lTX+n Δφm,UETX- n' Δφm,PRUTX,2π .

From this, it can be observed that the expected value of the carrier phase difference estimate has a non-zero error term given by

k-k' Δφm,lTX+n Δφm,UETX- n' Δφm,PRUTX .

We consider the special case in which case the DL-PRS measured by the PRU are exactly N slots before the DL-PRS measured by the UE (i.e., the DL-PRS for the l-th and m-th TRP have the same symbol locations within the slot). In this case, we have

n=n' and k=k'+NL

where L is the number of symbols per slot.  In this special case, the expected value of the carrier phase difference estimate is given by

NL Δφm,lTX+nΔφm,UETX- Δφm,PRUTX .

Appendix 2:  CP Measurements Referred to A Common Time Reference when PRU and UE Measurements are in Different Slots

We now consider how to refer carrier phase measurements to a common reference time in the case that the PRU and the UE take carrier phase measurements in different slots in a manner such that the mean error of the carrier phase difference measurement is removed.  We consider the more general case of Figure A1 in which the symbol indices (within the slot) of the DL-PRS in the slot measured by the PRU and the slot measured by the UE are not necessarily the same.  As in the prior Appendix, the indices k and k' can be any integer and are not limited to the number of symbols per slot and are used to to number the symbols across slot boundaries.  The indices n and n' denote the offsets of the second DL-PRS symbol relative to the first DL-PRS in the slots measured by the UE and PRU, respectively.  In this case, the mean error of the carrier phase difference error is given by

k-k' Δφm,lTX+n Δφm,UETX- n' Δφm,PRUTX .

The objective now is to define referred carrier phase measurements such that when the carrier phase difference measurement is computed using these referred carrier phase measurements, the mean error will be zero.

Figure A2: PRU and UE DL-PRS measurements referred to a common reference time

Refer to the example of Figure A2 above.  In this example, the carrier phase measurements will be referred to symbol within a slot between slot 1 and slot N+1. Let kref denote the symbol index to which the measurements are to be referred. Typically, kref will be the first symbol of a slot.

As previously, let θi,k,l denote the UE’s carrier phase measurement for the DL-PRS received from the l-th TRP in subcarrier i with symbol index k.  The referred carrier phase measurement for the UE is then given by

θi,k,l=modθi,k,l-k-kref Δφl,UETX, 2π  .

Similarly, let Ωi,k,l denote the PRU’s carrier phase measurement for the DL-PRS received from the l-th TRP in subcarrier i with symbol index k.  The referred carrier phase measurement for the PRU is then given by

Ωi,k,l=modΩi,k,l-k-kref Δφl,PRUTX, 2π  .

With these referred carrier phase measurement definitions, the mean error in the carrier phase difference measurement errors are removed.  Furthermore, for any subcarrier of any TRP, the measurements taken in different symbols can be averaged to improve the signal-to-noise ratio.