3GPP TSG-RAN WG1 Meeting #109	R4-2319646

Chicago, USA, November 13th – 17th 2023

Agenda Item:	8.28.6

Source:	Ericsson

Title:	NCR-MT demodulation results

Document for:	Discussion

# 1	Introduction

During recent meetings and in offline discussions, the requirement scope and parameterization for NCR-MT PDSCH and PDCCH demodulation requirements has been agreed. In this contribution, we present our simulation results for PDCCH and PDSCH.

# 2	Discussion

## 2.1	PDSCH results

PDSCH simulations have been carried out with FRCs discussed offline:

### 2.1.1 FR1

| Case number | Bandwidth (MHz) | SCS (kHz) | MCS | Propagation condition | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 70% max throughput 1% BLER |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 70% max throughput 1% BLER |

| Parameter | Unit | Case 1/2 | Case 3/4 | Case 5/6 | Case 7/8 |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 10 | 40 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 30 | 30 |
| Allocated resource blocks | PRBs | 52 | 52 | 106 | 106 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 4 | 13 | 4 | 13 |
| Modulation |  | QPSK | 16QAM | QPSK | 16QAM |
| Target Coding Rate |  | 0.30 | 0.48 | 0.30 | 0.48 |
| Number of MIMO layers |  | 1 | 1 | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  | 4096 | 13064 | 8456 | 26632 |
| Transport block CRC per Slot |  | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  | 1 | 2 | 2 | 4 |
| Binary Channel Bits Per Slot |  | 13728 | 27456 | 27984 | 55968 |

Based on these scenarios and FRCs, the results are as follows:

| Case number | Bandwidth (MHz) | SCS (kHz) | MCS | Propagation condition | Antenna configuration | 70% throughput (dB) | 1% BLER (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | -3.4 | 3.4 |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | -6.7 | -2.2 |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 4.6 | 11.7 |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 0.5 | 5.3 |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | -3.3 | 1.9 |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | -6.8 | -3.0 |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 4.2 | 9.7 |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 0.5 | 4.6 |

### 2.1.2 FR2

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 1 | 100 | 120 | TDLA30-75 | 2x2 | 70% max throughput 1% BLER |

| Parameter | Unit | Case 1 |
| --- | --- | --- |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 13 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.30 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  | 5504 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 1 |
| Binary Channel Bits Per Slot |  | 18282 |

Based on this scenario and FRC, our result is as follows:

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | 70% throughput (dB) | 1% BLER (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 100 | 120 | TDLA30-75 | 2x2 | -2.9 | 1.1 |

## 2.2	PDCCH

It was agreed at RAN4#108bis that the PDCCH requirements will be based on the existing 52 bit PDCCH format used for IAB-MT. With this agreement, a single simulation configuration is needed:

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | SNR (dB) for 1% |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 48 | 1 | 8 | TDLA30-10 | 2x4 Low | -3.9 |

## 2.2	CSI reporting

During RAN4#108bis, it was agreed not to define PMI requirements for NCR-MT. No further simulations are needed for CSI.

# References

R4-3216923, WF on NCR-MT demodulation requirements, ZTE corporation, RAN4#108bis, October 2023