3GPP TSG-RAN WG4 Meeting #110	                               R4-2400583

Athens, Greece, Feb. 26 – Mar. 1, 2024

Source: 	Murata Manufacturing Co Ltd.

Title: 	MSD for higher order combinations of CA_n40A-41A and CA_n39A-41A supporting simultaneous Rx/Tx

Agenda Item:	7.23.2 Identification of simultaneous Rx/Tx capability for band combinations and UE RF requirements	[LTE_NR_Simult_RxTx_R18-Core]

Document for:	Approval

Introduction

In the last RAN4#109 meeting, the WF [1] was approved to provide the missing MSD requirements for higher order combinations whose fallbacks support simultaneous Rx/Tx operation, which have not been specified.

In this contribution	we evaluate IMD calculation and resulting MSD for higher order combinations of CA_n40A-n41A and CA_n39A_41A, in other words CA_n8A-n40A-n41A, CA_n28A-n40A-n41A, CA_n40A-n41A-n79A, CA_28A-39A-41A and CA_39A-41A-79A.

Discussion

IMD relations

Higher order combinations of CA_40A-41A

Below IMD relations can be observed for higher order combinations of CA_n40A-n41A whose fallback combinations support simultaneous Rx/Tx operation.

- CA_n8A-n40A-n41A

  - IMD5 between n41 UL and n8 UL is falling into n40 DL

- CA_n28A-n40A-n41A

  - IMD5 between n40 UL and n28 UL is falling into n41 DL

- CA_n40A-n41A-n79A

  - IMD2 between n40 UL and n79 UL is falling into n41 DL

  - IMD4 between n40 UL and n79 UL is falling into n41 DL

  - IMD5 between n40 UL and n79 UL is falling into n41 DL

  - IMD2 between n41 UL and n79 UL is falling into n40 DL

  - IMD5 between n41 UL and n79 UL is falling into n40 DL

Higher order combinations of CA_39A-41A

Below IMD relations can be observed for higher order combinations of CA_n39A-n41A whose fallback combinations support simultaneous Rx/Tx operation.

- CA_n28A-n39A-n41A

  - IMD2 between n41 UL and n28 UL is falling into n39 DL

  - IMD2 between n39 UL and n28 UL is falling into n41 DL

- CA_n39A-n41A-n79A

  - IMD2 between n39 UL and n79 UL is falling into n41 DL

  - IMD5 between n39 UL and n79 UL is falling into n41 DL

  - IMD2 between n41 UL and n79 UL is falling into n39 DL

  - IMD5 between n41 UL and n79 UL is falling into n39 DL

IMD and MSD calculations

Assumptions

Figure 1 and Figure 2 show the reference architecture for the IMD and MSD analysis for the higher order combinations of CA_n40A-41A or CA_n39A-41A using shared antenna and separate antenna between MHB and LB/UHB respectively.

(a)

(b)

Figure 1. Reference architecture for (a) LB + MHB (b) MHB+UHB using shared antenna

(a)

(b)

Figure 2. Reference architecture for (a) LB + MHB (b) MHB+UHB using separate antenna

We performed IMD calculation based on the below linearity parameters (IP2, IP4 and IP5), filter attenuation, diplexer isolation and PCB and antenna isolation which are presented in Table 1 and Table 2.

Table 1 Intercept point parameters

|  | IP2 | IP4 | IP5 |
| --- | --- | --- | --- |
| PA forward | 27 | 31 | 27 |
| PA reverse | 38 | 33 | 32 |
| Switch | 112 | 55 | 53 |
| Filter/Duplexer | 100 | 55 | 53 |
| Diplexer | 115 | 55 | 53 |
| LNA | 6 | -5 | -10 |

Table 2 Filter attenuation/isolation, PCB and antenna isolation

| n41 filter attenuation at LB | 35 |
| --- | --- |
| n41 filter attenuation at n40 | 30 |
| n41 filter attenuation at n79 | 40 |
| n40 filter attenuation at LB | 35 |
| n40 filter attenuation at n41 | 40 |
| n40 filter attenuation at n79 | 30 |
| n39 filter attenuation at LB | 30 |
| n39 filter attenuation at n41 | 35 |
| n39 filter attenuation at n79 | 40 |
| LB filter attenuation at n39 | 45 |
| LB filter attenuation at n40 | 45 |
| LB filter attenuation at n41 | 45 |
| n79 filter attenuation at n39 | 40 |
| n79 filter attenuation at n40 | 40 |
| n79 filter attenuation at n41 | 40 |
| Diplexer isolation | 20 |
| Antenna isolation | 15 |
| PCB isolation | 60 |

IMD and MSD calculation

IMD and corresponding MSD performance was calculated after correlated MRC for each band combinations. The correction factors were included in the resulting IMD calculation to account for the differences between CW based IMD calculation and modulation IMD calculation. The correction factor is highly dependent on the two ULs bandwidth (or UL configuration), DL BW and IMD order because the partial portion of the full IMD product is failing in the victim DL bandwidth.

Shared antenna architecture

Table 3 and Table 4 summarize the IMD and MSD for shared antenna architecture.

Table 3 Results for IMD calculation and MSD for higher order combinations of CA_n40A-n41A using shared antenna architecture

Table 4 Results for IMD calculation and MSD for higher order combinations of CA_n39A-n41A using shared antenna architecture

Therefore, we propose to use below tables for RF requirements of MSD for higher order combinations of CA_n40A-41A and CA_n39A-41A supporting simultaneous Rx/Tx in case of shared antenna architecture.

Table 5 3DL/2UL MSDs for CA_n8-n40-n41

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n8-n40-n41 | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2360 | 3.0 | TDD | IMD5 |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | TDD | N/A |

Table 6 3DL/2UL MSDs for CA_n28-n40-n41

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n40-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2545 | 3.3 | TDD | IMD5 |

Table 7 3DL/2UL MSDs for CA_n40-n41-n79

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n40-n41-n79 | n40 | 2330 | 5 | 25 | 2330 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2520 | 26.0 | TDD | IMD2 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2550 | 5.7 | TDD | IMD4 |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A | TDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2550 | 3.4 | TDD | IMD5 |
|  | n79 | 4800 | 40 | 216 | 4800 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2330 | 23.7 | TDD | IMD2 |
|  | n41 | 2520 | 10 | 50 | 2520 | N/A | TDD | N/A |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2330 | 3.0 | TDD | IMD5 |
|  | n41 | 2530 | 10 | 50 | 2540 | N/A | TDD | N/A |
|  | n79 | 4960 | 40 | 216 | 4960 | N/A | TDD | N/A |

Table 8 3DL/2UL MSDs for CA_n28-n39-n41

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n39-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | N/A | 5 | N/A | 1895 | 32.6 | TDD | IMD2 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | 1985 | 5 | 25 | 1895 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2620 | 32.6 | TDD | IMD2 |

Table 9 3DL/2UL MSDs for CA_n39-n41-n79

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n39-n41-n79 | n39 | 1900 | 5 | 25 | 1900 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 26.0 | TDD | IMD2 |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
|  | n39 | 1890 | 5 | 25 | 1890 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2680 | 9.2 | TDD | IMD5 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |
|  | n39 | N/A | 5 | N/A | 1900 | 24.8 | TDD | IMD2 |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
|  | n39 | N/A | 5 | N/A | 1900 | 3.1 | TDD | IMD5 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |

Separate antenna architecture

Table 10 and Table 11 summarize the IMD and MSD for separate antenna architecture.

Table 10 Results for IMD calculation and MSD for higher order combinations of CA_n40A-n41A using separate antenna architecture

Table 11 Results for IMD calculation and MSD for higher order combinations of CA_n39A-n41A using separate antenna architecture

Therefore, we propose to use below tables for RF requirements of MSD for higher order combinations of CA_n40A-41A and CA_n39A-41A supporting simultaneous Rx/Tx in case of separate antenna architecture.

Table 12 3DL/2UL MSDs for CA_n8-n40-n41

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n8-n40-n41 | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2360 | 8.0 | TDD | IMD5 |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | TDD | N/A |

Table 13 3DL/2UL MSDs for CA_n28-n40-n41

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n40-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2545 | 18.2 | TDD | IMD5 |

Table 14 3DL/2UL MSDs for CA_n40-n41-n79

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n40-n41-n79 | n40 | 2330 | 5 | 25 | 2330 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2520 | 33.6 | TDD | IMD2 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2550 | 12.7 | TDD | IMD4 |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A | TDD | N/A |
|  | n40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2550 | 9.4 | TDD | IMD5 |
|  | n79 | 4800 | 40 | 216 | 4800 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2330 | 35.7 | TDD | IMD2 |
|  | n41 | 2520 | 10 | 50 | 2520 | N/A | TDD | N/A |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2330 | 6.5 | TDD | IMD5 |
|  | n41 | 2530 | 10 | 50 | 2540 | N/A | TDD | N/A |
|  | n79 | 4960 | 40 | 216 | 4960 | N/A | TDD | N/A |

Table 15 3DL/2UL MSDs for CA_n28-n39-n41

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n39-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | N/A | 5 | N/A | 1895 | 45.0 | TDD | IMD2 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | 1985 | 5 | 25 | 1895 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2620 | 42.7 | TDD | IMD2 |

Table 16 3DL/2UL MSDs for CA_n39-n41-n79

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n39-n41-n79 | n39 | 1900 | 5 | 25 | 1900 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2650 | 33.6 | TDD | IMD2 |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
|  | n39 | 1890 | 5 | 25 | 1890 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2680 | 18.2 | TDD | IMD5 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |
|  | n39 | N/A | 5 | N/A | 1900 | 32.0 | TDD | IMD2 |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |
|  | n39 | N/A | 5 | N/A | 1900 | 4.7 | TDD | IMD5 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | TDD | N/A |

Discussion

It can be seen that the MSD using shared antenna architecture is lower than separate antenna architecture with the help of additional diplexer isolation between LB/UHB and MHB. Because the MSD is highly dependent on the baseline architecture, we would like to discuss more which architecture would be appropriate by providing below two proposals.

Proposal 1: Use Table 5-9 for the RF requirements of MSD of higher order combinations of CA_n40A-41A and CA_n39A-41A supporting simultaneous Rx/Tx in case of shared antenna architecture.

Proposal 2: Use Table 12-16 for the RF requirements of MSD of higher order combinations of CA_n40A-41A and CA_n39A-41A supporting simultaneous Rx/Tx in case of separate antenna architecture.

Conclusion

In this contribution, we presented the IMD relations for higher order combinations of CA_n40A-n41A and CA_n39A-41A supporting simultaneous Rx/Tx determining the IMD downlink overlaps and resulting MSDs, which have not been specified yet. With the introduction of the results, missing RF requirements can be identified by making the following proposals.

Proposal 1: Use Table 5-9 for the RF requirements of MSD of higher order combinations of CA_n40A-41A and CA_n39A-41A supporting simultaneous Rx/Tx in case of shared antenna architecture.

Proposal 2: Use Table 12-16 for the RF requirements of MSD of higher order combinations of CA_n40A-41A and CA_n39A-41A supporting simultaneous Rx/Tx in case of separate antenna architecture.

References

[1]	R4-2321992, “WF on simultaneous Rx-Tx requirements”, Apple, 3GPP TSG RAN WG4 Meeting #109, Chicago, United States, Nov 13th – 17th, 2023