3GPP TSG-RAN WG4 Meeting #110                                                      R4-2400722

Athens, Greece, 26th February – 1st March, 2024

Source:	Qualcomm Incorporated

Title:	(NR_SL_enh2-Core) PEMAX,CA for SL CA

Agenda item:	8.22.1.3

Document for:	Approval

# 1.	Introduction

In RAN4#108bis a LS was sent to RAN2 requesting their opinion on how to select a value for PEMAX,CA . In RAN4#109 RAN2 sent a reply LS to RAN4’s inquiry. In this paper we further discusses the RAN2 reply.

# 2. 	Discussion

# In RAN4#108-bis there was an LS sent to RAN2 asking their opinion on the appropriate selection for PEMAX,CA based on two options given below [1]:

For SL CA PEMAX,CA, it has been agreed below Pcmax equation while how to define PEMAX,CA is not clear.

PCMAX_L = MIN{10 log10 ∑ pEMAX,C - ΔTC, PPowerClass, SL_CA – MAX(MAX(MPR,  A-MPR) + ΔTIB,c+ΔTC, P-MPR ), PRegulatory, PEMAX,CA}

PCMAX_H = MIN {10 log10 ∑ pEMAX,C , PPowerClass, SL_CA, PRegulatory, PEMAX,CA }

For SL CA PEMAX,CA, the following alternatives are considered:

- Alt.1 : Reuse the existing IE for SL CA, i.e.,

  - PEMAX,CA  = IE sl-maxTransPower (cc1) + IE sl-maxTransPower(cc2)

- Alt.2 : Define new IE for SL CA, i.e.,

  - PEMAX,CA  = new IE, sl-maxTransPower-CA

In RAN4#110 RAN2 sent a reply to RAN4’s inquiry stating that Alt2 is the most appropriate selection. The RAN2 reply is given below [2].

RAN2 understands that Alt.2 is consistent with the principle of defining PEMAX,CA for NR UL CA in TS 38.101. Additionally, adopting Alt.2 can give the network more flexibility in controlling PCMAX of NR SL CA. Therefore, RAN2 agreed to take Alt.2 to define the PEMAX,CA for NR SL CA.

- Alt.1: Reuse the existing IE for SL CA, i.e.,

  - PEMAX,CA  = IE sl-maxTransPower (cc1) + IE sl-maxTransPower(cc2)

- Alt.2: Define new IE for SL CA, i.e.,

  - PEMAX,CA  = new IE, sl-maxTransPower-CA

Based on the RAN2 reply given above we believe that the most suitable thing to do is to allow PEMAX,CA,   defined by IE, sl-maxTransPower-CA, to be UE configurable. This way the UE can select IE, sl-maxTransPower-CA as it desires to optimize the transmit power for a given operating scenario.

Proposal: Allow PEMAX,CA, defined by IE, sl-maxTransPower-CA   to be UE configurable

# Conclusion

In this paper we present the following proposal for defining PEMAX,CA  for SL CA.

Proposal: Allow PEMAX,CA, defined by IE, sl-maxTransPower-CA   to be UE configurable

# References

[1] R4-2317751, LS on a capability of UE power class and IE on PEMAX,CA for SL CA, RAN4#108bis, Oct. 9th    23th, 2023

[2] R2-2313605, Response LS on PEMAX,CA for SL CA, RAN4#109, Nov. 13th – 17th , 2023

[3] R1-2312264, FL Summary #3 for AI 8.2.3: Sidelink CA operation, , Nov. 13th – 17th, 2023