3GPP TSG-RAN WG4 Meeting # 110	R4-2401173

# Athens, Greece, Feb. 26 – Mar. 1, 2024

Source: 	Xiaomi

Title: 	Discussion on RRM requirement for FeMIMO

Agenda Item:	8.21.2.1

Document for:	Discussion

# 1 Introduction

In RAN4#109 meeting, it’s agreed to provide simulation results regarding to feasibility of TDCP test. In this contribution, we further discuss whether and how to design test case.

- Discussion

In last meeting, the WF is below:

Issue 1-1-1: Is it feasible to define TDCP accuracy requirement for TDCP? Agreement: - No RRM core requirement to be defined, and further discuss the following options: - Option 1: Define accuracy and test cases as part of performance requirement - Option 2: Do not define accuracy requirement but define test cases for particular configurations as part of performance requirements - Option 3: Do not define accuracy requirement and test cases as part of performance requirements

Since the correlation value are different for different channel models, e.g. TDL-A and TDL-D, as shown below:

Fig.1 ideal channel correlation value for TDL-A and TDL-D

It’s hard to define a general requirement for all channel models. In last meeting, simulation results are provided for TDL-A channel model. Therefore, we prefer option 2.

Proposal 1: Do not define accuracy requirement but define test cases for TDL-A.

In the next, we discuss the test metric, side condition and threshold for performance verification.

SNR condition

Based on our simulation results[1], we suggested at SNR should be at least higher than 15dB. At the same time, there are also some test issues if SNR is too high. Therefore, we suggest to choose SNR = 15dB.

Proposal 2: For TDCP test, SNR can be set as 15dB.

Test metric

There are several possible Test metrics:

      - Test metric based on instant value delta between estimated and ideal value

- apply for low doppler case

      - Test metric based on mean value delta between estimated and ideal value

- apply for both low and high doppler case

      - Test metric based on Median value delta between estimated and ideal value

- apply for both low and high doppler case

In last meeting, we provide simulation results for all the 3 types of test metrics.

Test metric 1 can apply for low doppler case. For low doppler case, e.g. smaller than 30Hz, the ideal correlation value distribution will concentrate to 1 with small variation. it’s possible to use a constant value over time as ideal value.

Test metric 2 and 3 are similar and can apply for both low and high doppler case. Since in last meeting, all the company have provided simulation results for CDF curves, then it’s more convenient to define test metric based on Median value(50% point in CDF curve).

Median value delta is defined between estimated and ideal median value. From our simulation results, ideal median value is quite similar to Bessel function in most cases.

Table 1: Compare ideal value from TDL-A channel and from Bessel function

|  | Doppler = 10Hz | Doppler = 30Hz | Doppler = 75Hz | Doppler = 100Hz | Doppler = 200Hz | Doppler = 300Hz |
| --- | --- | --- | --- | --- | --- | --- |
| Median value derived from channel model | 0.9998 | 0.9978 | 0.9860 | 0.9742 | 0.9104 | 0.8405 |
| Value from Bessel function | 0.9998 | 0.9978 | 0.9862 | 0.9755 | 0.9037 | 0.79 |

Therefore, we propose that:

Proposal 3: Test metric is defined as Median value delta between estimated median and ideal median value, where ideal median value can be derived from Bessel function.

Threshold

There are two possible ways to define the threshold:

- Based on correlation value delta without quantization

- Based on index delta after quantization

Based on correlation value delta without quantization

We provide simulation results for delta between ideal and measured correlation value before quantization for SNR = 15dB.

Table: Median value difference between ideal and estimated for SNR = 15dB:

|  | Doppler = 10Hz | Doppler = 30Hz | Doppler = 75Hz | Doppler = 100Hz | Doppler = 200Hz | Doppler = 300Hz |
| --- | --- | --- | --- | --- | --- | --- |
| Ideal Median value from Bessel | 0.9998 | 0.9978 | 0.9862 | 0.9755 | 0.9037 | 0.79 |
| Estimated median value for SNR = 15dB | 0.9587 | 0.9552 | 0.9437 | 0.9280 | 0.8532 | 0.7492 |
| Median value delta for SNR = 15dB | 0.0411 | 0.0426 | 0.0425 | 0.0475 | 0.0505 | 0.0408 |

Observation 1: The Median value delta without quantization for all dopplers is smaller than 0.0505 for SNR = 15dB.

Based on index delta after quantization

Measured correlation value index after quantization for SNR = 15dB is as below:

Fig.2 Reported index after quantization

Therefore, the index delta is as below:

Table: Median value index difference between ideal and estimated for SNR = 15dB:

|  | Doppler = 10Hz | Doppler = 30Hz | Doppler = 75Hz | Doppler = 100Hz | Doppler = 200Hz | Doppler = 300Hz |
| --- | --- | --- | --- | --- | --- | --- |
| Ideal index | 0 | 0 | 3 | 5 | 9 | 11or12 |
| Measured index | 7 | 8 | 8 | 9 | 11 | 12 |
| Index delta | 7 | 8 | 5 | 4 | 2 | 1 or 2 |

The Median value index delta after quantization is different depending on dopplers. it’s rational since the quantization step is larger when correlation value is smaller. Then for high doppler case, the index delta is smaller.

Observation 2: The Median value index delta after quantization is different depending on dopplers for SNR = 15dB.

If only test case is designed for TDCP, we are open to further discuss whether threshold is defined before or after quantization.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: Do not define accuracy requirement but define test cases for TDL-A.

Proposal 2: For TDCP test, SNR can be set as 15dB.

Proposal 3: Test metric is defined as Median value delta between estimated median and ideal median value, where ideal median value can be derived from Bessel function.

Observation 1: The Median value delta without quantization for all dopplers is smaller than 0.0505 for SNR = 15dB.

Observation 2: The Median value index delta after quantization is different depending on dopplers for SNR = 15dB.

- Reference

[1] R4-2318849, Discussion on RRM requirement for FeMIMO, Xiaomi.