3GPP TSG-RAN WG4 Meeting #110	R4-2401534

Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on updated the Pemax of Sidelink CA for TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-2-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The PCMAX equation for SL CA is not correct since the term PEMAX,CA is not for sum. Also, the IE for PEMAX,CA needs to be updated according to the reply LS R4-2400015. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Change ‘10 log10 ∑ pEMAX,CA’ to ‘10 log10 ∑ pEMAX,C’ in PCMAX_L and PCMAX_H. 2. Add ‘PEMAX,CA’ in PCMAX_L and PCMAX_H. 3. Update the IE for PEMAX,CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Pcmax equation for CA is not right and the IE for PEMAX,CA is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

6.2E.4A	Configured transmitted power for Sidelink CA

For intra-band contiguous SL CA operation, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in subclause 6.3.2 and subclause 6.3.4 respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX,c  is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤  PCMAX,  ≤  PCMAX_H

For SL transmission of intra-band contiguous CA when same slot pattern is used in all aggregated component carriers.

PCMAX_L = MIN{10 log10 ∑ pEMAX,C - TC,PEMAX,CA,  PPowerClass, SL_CA – MAX(MAX(MPR,  A-MPR) + ΔTIB,c+TC, P-MPR ), PRegulatory }

PCMAX_H = MIN{10 log10 ∑ pEMAX,C , PEMAX,CA, PPowerClass, SL_CA, PRegulatory }

where

-	For the total transmitted power PCMAX,PSSCH/PSCCH, PEMAX, CA is the value given by the new IE sl-maxTransPower-CA for maximum transmitted power of SL CA, defined by TS 38.331;

-	PPowerClass, SL_CA is the maximum UE power specified in Table 6.3.1-1 without taking into account the tolerance;

-	MPR and A-MPR are specified in subclause 6.3.2 and subclause 6.3.3 respectively;

-	TIB,c  and P-MPR are specified in clause 6.2.4 in TS38.101-1;

-	TC is the highest value TC,c among all component carriers c in the subframe over both timeslots. TC,c = 1.5 dB when NOTE 3 in Table 6.2.1-1 in TS38.101-1 applies, otherwise TC,c = 0 dB;

-	PRegulatory= 10 - Gpost connector dBm when V2X UE is within the protected zone in ETSI TS 102 792 of CEN DSRC tolling system and operating in Band n47; PRegulatory= 33 - Gpost connector dBm otherwise.

The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on 0dB PSD offset between PSSCH and PSCCH.

For intra-band SL CA operation, when at least one different numerology/slot pattern is used in aggregated cells, the same requirement as specified in clause 6.2E.4.3 in TS38.101-1 shall be applied.

The measured configured maximum output power PUMAX,c for sidelink CA operation, when at least one slot has a different transmission numerology or slot pattern, the same requirement as specified in clause 6.2E.4.3 in TS38.101-1 shall be applied.

<End of Change>