3GPP TSG-RAN WG4 Meeting #110	R4-2402725

# Athens, Greece, February 26 – March 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM Core Requirements Maintenance for NR Sidelink CA

Agenda item:	8.22.2.1

Document for:	Discussion

Introduction

The WF on RRM requirements related to NR SL CA as agreed in RAN4#109 is found in [1]. The agreements are provided below.

Agreement: - Introduce constraints on interruptions to WAN at SL carrier addition/release ONLY due to paging and SIB reception for UE in RRC_IDLE/INACTIVE mode. - Companies to further discuss on the RLF and CBD parts. Prepare the draft CR based on the agreed scenario for the interruption constraint in this meeting.

In this paper, we discuss the highlighted FFS issues left for further discussion.

Discussion

In this section, we discuss the open issues on RRM requirements for NR SL CA.

Constraints on interruptions to WAN at SL carrier addition/release due to RLF and CBD

The discussion is related to the case where a UE performs NR SL CA while it may have a parallel Uu communication path with gNB.

UE’s behavior during a radio link failure (RLF) and beam candidate bean detection (CBD) procedures is also specified with respective to the related actions and corresponding RRM requirements. These procedures are in general critical and can result in UE’s loss of its serving cell in worst case. In that case, UE needs to perform RRC re-establishment. Therefore, allowing interruption to this procedure may have a negative impact in UE’s failure rate and recovery opportunities. We believe allowing constraints to protect these procedures from any interruption is preferred in Release 18.

Proposal 1:  RAN4 requirement specification corresponding to interruption to WAN due to NR SL component carrier addition/release should have constraints to protect RLF and CBD procedures.

Conclusions

This document has made the following observations and proposals.

Proposal 1: RAN4 requirement specification corresponding to interruption to WAN due to NR SL component carrier addition/release should have constraints to protect RLF and CBD procedures.

# References

- R4-2321585, WF on R18 NR SL RRM requirements (part 2), Oppo, RAN4#109.