3GPP TSG-RAN WG4 Meeting #110	R4-2402736

# Athens, Greece, February 26 – March 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results for RedCap Positioning with Frequency Hopping

Agenda item:	8.14.3.3

Document for:	Information

Introduction

The Rel-18 WID [1] for NR positioning contains several enhancements, including:

- Support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning.

NR RedCap positioning has been studied in the FS_NR_pos_enh2 study item by RAN1 with company contributions and outcome of the study being captured in TR 38.859 [2]. In RAN4#108 [3], the link simulation assumptions for RedCap positioning with FH are agreed. The agreed parameters for link level simulations are captured in Table 1 and Table 2 in [4]. Also, the agreed performance metrics for RSTD and UE Rx-Tx measurements are provided in Section 3 [4].

The initial preliminary simulation results on RedCap positioning with FH are provided in [5]. In this contribution we further provide the simulation results for RedCap positioning. It is worth noting that the results are generated without RF/BB impairments. Also, note that PRB overlapping is considered between the frequency hops without phase shift and the results are provided for 1 Rx and 2 Rx antennas at the UE.

Simulation Assumptions

Simulation assumptions for RedCap positioning with FH captured in [4] are considered. The PRS configurations as listed in Table 1 were used in the simulation study.

| Carrier frequency | SCS (kHz) | PRS BW in RB | sampling rate (Tc) | Num Symbols | Comb Size | Repetition Factor (Rep) | Number of overlapping PRBs between hops |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 GHz | 15 | 272 | 32 | 4 | 4 | 6 | 1 |
| 4 GHz | 30 | 272 | 16 | 4 | 4 | 6 | 1 |
| 4 GHz | 60 | 132 | 16 | 4 | 4 | 6 | 1 |
| 40 GHz | 60 | 272 | 8 | 4 | 4 | 6 | 1 |
| 40 GHz | 120 | 272 | 4 | 4 | 4 | 6 | 1 |

Table 1: PRS configurations used for RSTD study for SCS = 15kHz, 30kHz, and 60 kHz.

Simulation Results

In this section we provide the simulation results for the above listed simulation assumptions. The results are reported for 5-percentile and 95-percentile CDF of the DL RSTD error in Tc and for 90-percentile CDF of the UE Rx-Tx error in Tc for given Es/Iot for three cells [4].

FR2 RSTD Results, 2 Rx

AWGN Channel

The DL RSTD errors in Tc for AWGN channel, derived from the 5%-ile and 95%-ile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB and (-3, -6, -6) dB for 2 Rx antennas at the UE are listed in Table 2 and Table 3, respectively.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | RSTD (cell-1, cell-2) |  | RSTD (cell-1, cell-3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| 40 GHz / 60 kHz | 272 | 8 | 4 / 4 | 6 | 1 | -8 | 8 | -8 | 8 |
|  |  |  |  | 6 | 4 | -4 | 4 | -4 | 4 |
| 40 GHz / 120 kHz | 272 | 4 | 4 / 4 | 6 | 1 | -4 | 4 | -4 | 4 |
|  |  |  |  | 6 | 4 | -2 | 2 | -2 | 2 |

Table 2: RSTD error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | RSTD (cell-1, cell-2) |  | RSTD (cell-1, cell-3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| 40 GHz / 60 kHz | 272 | 8 | 4 / 4 | 6 | 1 | -8 | 8 | -8 | 8 |
|  |  |  |  | 6 | 4 | -4 | 4 | -4 | 4 |
| 40 GHz / 120 kHz | 272 | 4 | 4 / 4 | 6 | 1 | -4 | 4 | -4 | 4 |
|  |  |  |  | 6 | 4 | -2 | 2 | -2 | 2 |

Table 3: RSTD error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

TDL-A Channel

The DL RSTD errors in Tc for TDL-A channel, derived from the 5%-ile and 95%-ile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB are listed in Table 4.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | RSTD (cell-1, cell-2) |  | RSTD (cell-1, cell-3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| 40 GHz / 60 kHz | 272 | 8 | 4 / 4 | 6 | 1 | -8 | 8 | -8 | 8 |
|  |  |  |  | 6 | 4 | -6 | 6 | -6 | 6 |
| 40 GHz / 120 kHz | 272 | 4 | 4 / 4 | 6 | 1 | TBD | 4 | -12 | 4 |
|  |  |  |  | 6 | 4 | -4 | 4 | TBD | 4 |

Table 4: Rx-Tx error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

FR1 RSTD Results, 1 Rx

TDL-A Channel

The DL RSTD errors in Tc for TDL-A channel, derived from the 5%-ile and 95%-ile of the CDF, for Es/IoT of three cells (-6, -10, -10) and (-3, -3, -6) dB are listed in Tables 6 and 7, respectively.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | RSTD (cell-1, cell-2) |  | RSTD (cell-1, cell-3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | -64 | 64 | -64 | 64 |
|  |  |  |  | 6 | 4 | -40 | 48 | -56 | 48 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | -32 | 32 | -32 | 32 |
|  |  |  |  | 6 | 4 | -24 | 24 | -24 | 20 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | -24 | 24 | -32 | 24 |
|  |  |  |  | 6 | 4 | -20 | 22 | TBD | 18 |

Table 6: RSTD error for TDL-A channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-6, -10, -10) dB.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | RSTD (cell-1, cell-2) |  | RSTD (cell-1, cell-3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-ile | 95%-ile | 5%-ile | 95%-ile |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | -32 | 32 | -32 | 32 |
|  |  |  |  | 6 | 4 | -32 | 32 | -32 | 32 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | -32 | 16 | -32 | 16 |
|  |  |  |  | 6 | 4 | -20 | 16 | -20 | 16 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | -24 | 16 | -24 | 16 |
|  |  |  |  | 6 | 4 | -16 | 16 | -30 | 16 |

Table 7: RSTD error for TDL-A channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

FR1 UE Rx-Tx Results, 2 Rx

AWGN Channel

The UE Rx-Tx errors in Tc for AWGN channel, derived from the 90%-ile of the CDF, for Es/IoT of three cells (-3, -13, -13) dB and (0, -6, -6) dB are listed in Table 8 and Table 9, respectively.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | 16 | 16 | 16 |
|  |  |  |  | 6 | 4 | 8 | 8 | 8 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | 8 | 8 | 8 |
|  |  |  |  | 6 | 4 | 4 | 4 | 4 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | 0 | 0 | 0 |
|  |  |  |  | 6 | 4 | 0 | 0 | 0 |

Table 8: Rx-Tx error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-3, -13, -13) dB.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | 16 | 16 | 16 |
|  |  |  |  | 6 | 4 | 8 | 8 | 8 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | 8 | 8 | 8 |
|  |  |  |  | 6 | 4 | 4 | 4 | 4 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | 0 | 0 | 0 |
|  |  |  |  | 6 | 4 | 0 | 0 | 0 |

Table 9: Rx-Tx error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (0, -6, -6) dB.

TDL-A Channel

The UE Rx-Tx errors in Tc for AWGN channel, derived from the 90%-ile of the CDF, for Es/IoT of three cells (--3, -13, -13) dB and (0, -6, -6) dB are listed in Table 8 and Table 9, respectively.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | -176 | -176 | -176 |
|  |  |  |  | 6 | 4 | -176 | -176 | -176 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | -88 | -88 | -72 |
|  |  |  |  | 6 | 4 | -84 | -84 | -84 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | -80 | -80 | -80 |
|  |  |  |  | 6 | 4 | -80 | -84 | -82 |

Table 8: Rx-Tx error for TDL-A channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-3, -13, -13) dB.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | -176 | -176 | -176 |
|  |  |  |  | 6 | 4 | -176 | -176 | -176 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | -88 | -88 | -72 |
|  |  |  |  | 6 | 4 | -84 | -84 | -84 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | -80 | -80 | -80 |
|  |  |  |  | 6 | 4 | -80 | -82 | -80 |

Table 9: Rx-Tx error for TDL-A channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (0, -6, -6) dB.

FR1 UE Rx-Tx Results, 1 Rx

AWGN Channel

The UE Rx-Tx errors in Tc for AWGN channel, derived from the 90%-ile of the CDF, for Es/IoT of three cells (-3, -13, -13) dB and (0, -6, -6) dB are listed in Table 8 and Table 9, respectively.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | 16 | 16 | 16 |
|  |  |  |  | 6 | 4 | 8 | 8 | 8 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | 8 | 8 | 8 |
|  |  |  |  | 6 | 4 | 4 | 4 | 4 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | 0 | 0 | 0 |
|  |  |  |  | 6 | 4 | 0 | 0 | 0 |

Table 8: Rx-Tx error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-3, -13, -13) dB.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | 16 | 16 | 16 |
|  |  |  |  | 6 | 4 | 8 | 8 | 8 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | 8 | 8 | 8 |
|  |  |  |  | 6 | 4 | 4 | 4 | 4 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | 0 | 0 | 0 |
|  |  |  |  | 6 | 4 | 0 | 0 | 0 |

Table 9: Rx-Tx error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (0, -6, -6) dB.

TDL-A Channel

The UE Rx-Tx errors in Tc for AWGN channel, derived from the 90%-ile of the CDF, for Es/IoT of three cells (-3, -13, -13) dB and (0, -6, -6) dB are listed in Table 8 and Table 9, respectively.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | -176 | -176 | -176 |
|  |  |  |  | 6 | 4 | -176 | -168 | -176 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | -72 | -72 | -72 |
|  |  |  |  | 6 | 4 | -76 | -76 | -80 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | -80 | -80 | -80 |
|  |  |  |  | 6 | 4 | -80 | -80 | -82 |

Table 8: Rx-Tx error for TDL-A channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-3, -13, -13) dB.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 GHz / 15 kHz | 272 | 32 | 4 / 4 | 6 | 1 | -176 | -176 | -176 |
|  |  |  |  | 6 | 4 | -176 | -168 | -168 |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | -72 | -72 | -72 |
|  |  |  |  | 6 | 4 | -76 | -76 | -80 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | -80 | -80 | -80 |
|  |  |  |  | 6 | 4 | -80 | -80 | -80 |

Table 9: Rx-Tx error for TDL-A channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (0, -6, -6) dB.

FR2 UE Rx-Tx Results, 2 Rx

AWGN Channel

The UE Rx-Tx errors in Tc for AWGN channel, derived from the 90%-ile of the CDF, for Es/IoT of three cells (-3, -13, -13) dB and (0, -6, -6) dB are listed in Table 8 and Table 9, respectively.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 GHz / 60 kHz | 272 | 16 | 4 / 4 | 6 | 1 | 4 | 4 | 4 |
|  |  |  |  | 6 | 4 | 2 | 2 | 2 |
| 40 GHz / 120 kHz | 132 | 16 | 4 / 4 | 6 | 1 | 2 | 2 | 2 |
|  |  |  |  | 6 | 4 | 1 | 1 | 1 |

Table 8: Rx-Tx error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-3, -13, -13) dB.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 GHz / 60 kHz | 272 | 16 | 4 / 4 | 6 | 1 | 4 | 4 | 4 |
|  |  |  |  | 6 | 4 | 2 | 2 | 2 |
| 40 GHz / 120 kHz | 132 | 16 | 4 / 4 | 6 | 1 | 2 | 2 | 2 |
|  |  |  |  | 6 | 4 | 1 | 1 | 1 |

Table 9: Rx-Tx error for AWGN channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (0, -6, -6) dB.

TDL-A Channel

The UE Rx-Tx errors in Tc for AWGN channel, derived from the 90%-ile of the CDF, for Es/IoT of three cells (-3, -13, -13) dB and (0, -6, -6) dB are listed in Table 8 and Table 9, respectively.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | -28 | -28 | -28 |
|  |  |  |  | 6 | 4 | -28 | -28 | -28 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | -2 | -2 | -2 |
|  |  |  |  | 6 | 4 | -1 | -1 | -1 |

Table 8: Rx-Tx error for TDL-A channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (-3, -13, -13) dB.

| Carrier frequency / SCS | PRS BW in RB | sampling rate (Tc) | Num Symbols/ Comb Size | Rep | Number of samples | 90%-ile Cell 1 | 90%-ile Cell 2 | 90%-ile Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 GHz / 30 kHz | 272 | 16 | 4 / 4 | 6 | 1 | -28 | -28 | -28 |
|  |  |  |  | 6 | 4 | -28 | -28 | -28 |
| 4 GHz / 60 kHz | 132 | 16 | 4 / 4 | 6 | 1 | -2 | -2 | -2 |
|  |  |  |  | 6 | 4 | -1 | -1 | -1 |

Table 9: Rx-Tx error for TDL-A channel in Tc, Es/IoT (cell-1, cell-2, cell-3) = (0, -6, -6) dB.

Discussion

From the RSTD and UE Rx-Tx results reported above for the AWGN and TDL-A channels using 1 and 2 UE receive antennas with overlapping PRBs between frequency hops and without RF/BB impairments, we observed:

When AWGN and TDL-A channel are used, for a given bandwidth, the measurement accuracy improves with the increasing number of measurement samples.

Increasing the UE total measurement bandwidth after all hops from 50 MHz (SCS 15 kHz) to 100 MHz (SCS 30 and 60 kHz) improves the measurement accuracy when AWGN or TDL-A channels are used.

For the AWGN channel, Es/IoT of (-6, -13, -13) yields the same results as (-3, -6, -6) indicating that, based on the current simulation setup, the Es/IoT of (-6, -13, -13) is still enough to successfully detect the first path.

The simulation results show that the performance changes depending on the number of measurement samples in all cases of the different SNRs with AWGN and TDL-A channel models.

For the TDL-A channel, the simulation RSTD results show that the performance is improved when 2 Rx antennas are used at the UE. Other cases showed a similar performance (for RSTD and UE Rx-Tx results) when 1 or 2 Rx antennas are used at the UE.

Using 4 measurement samples should be the baseline to define the performance requirements.

For the alignment of simulation results, RAN4 should discuss the modeling of the frequency impairments due to phase shifts between frequency hops and include the PRB overlap as a parameter in the simulation assumptions.

RAN4 to discuss the modeling of the frequency impairments due to phase shifts between frequency hops and include the PRB overlap as a parameter in the simulation assumptions.

Conclusion

In this contribution we have provided first simulation results for DL RSTD and UE Rx-Tx error without RF/BB impairments based on simulation assumptions in [4].

Following observations and proposals are made.

- When AWGN and TDL-A channel are used, for a given bandwidth, the measurement accuracy improves with the increasing number of measurement samples.

- Increasing the UE total measurement bandwidth after all hops from 50 MHz (SCS 15 kHz) to 100 MHz (SCS 30 and 60 kHz) improves the measurement accuracy when AWGN or TDL-A channels are used.

- For the AWGN channel, Es/IoT of (-6, -13, -13) yields the same results as (-3, -6, -6) indicating that, based on the current simulation setup, the Es/IoT of (-6, -13, -13) is still enough to successfully detect the first path.

- The simulation results show that the performance changes depending on the number of measurement samples in all cases of the different SNRs with AWGN and TDL-A channel models.

- For the TDL-A channel, the simulation RSTD results show that the performance is improved when 2 Rx antennas are used at the UE. Other cases showed a similar performance (for RSTD and UE Rx-Tx results) when 1 or 2 Rx antennas are used at the UE.

- Using 4 measurement samples should be the baseline to define the performance requirements.

- RAN4 to discuss the modeling of the frequency impairments due to phase shifts between frequency hops and include the PRB overlap as a parameter in the simulation assumptions.

- References

| [1] | RP-232670, Revised WID on Expanded and Improved NR positioning, 3GPP TSG RAN#101 |
| --- | --- |
| [2] | TR 38.859, "Study on Expanded and Improved NR Positioning (Release 18)," V18.0.0 |
| [3] | R4-2314353, "WF on R18 NR positioning – RedCap positioning and PRS/SRS BW aggregation," Ericsson |
| [4] | R4-2314460, " Simulation assumptions for RedCap positioning with FH," Ericsson |
| [5] | R4-2320856, " Simulation Results for RedCap Positioning with Frequency Hopping," Ericsson, RAN4#109. |