3GPP TSG-RAN WG4 Meeting #110 	R4-2402804

Athens, GR, 26 February – 01 March, 2024

Agenda Item:	8.3.2.3

Source: 	Ericsson

Title:	On RLM and beam management for multi-rx

Document for:	Discussion

# Introduction

The agreements for RLM, CBD, and BFD from RAN4#109 are captured in [24]. All agreements on RLM, CBD, and BFD are listed in Annex A below.

# Discussion

## Measurement restrictions and scheduling availability for CBD

- Observation 1: For cell-specific CBD under multi-rx operation, measurement restrictions requirements were introduced in RAN4#109, but not scheduling availability enhancements which were missing in the endorsed big CR R4-2321636.

- Proposal 1: Cell-specific CBD enhancements for scheduling availability in FR2-1 are captured in section 8.5.8.3 (missing in the Big CR from RAN4#109).

- Observation 2: For TRP-specific CBD under multi-rx operation, measurement restrictions requirements were introduced in RAN4#109, but not scheduling availability enhancements which were missing in the endorsed big CR R4-2321636.

- Proposal 2: TRP-specific CBD enhancements for scheduling availability in FR2-1 are captured in section 8.18.9.3 (missing in the Big CR from RAN4#109).

# Summary

The following have been observed and proposed in the current contribution:

- Observation 1: For cell-specific CBD under multi-rx operation, measurement restrictions requirements were introduced in RAN4#109, but not scheduling availability enhancements which were missing in the endorsed big CR R4-2321636.

- Proposal 1: Cell-specific CBD enhancements for scheduling availability in FR2-1 are captured in section 8.5.8.3 (missing in the Big CR from RAN4#109).

- Observation 2: For TRP-specific CBD under multi-rx operation, measurement restrictions requirements were introduced in RAN4#109, but not scheduling availability enhancements which were missing in the endorsed big CR R4-2321636.

- Proposal 2: TRP-specific CBD enhancements for scheduling availability in FR2-1 are captured in section 8.18.9.3 (missing in the Big CR from RAN4#109).

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2214344, WF on FR2 multi-Rx RRM, vivo, Aug. 2022.

[3] R4-2217246, WF on RRM impacts and general aspects for multi-Rx, vivo, Oct. 2022.

[4] R4-2217587, WF on L1 measurements, beam sweeping factors and simultaneous reception, Qualcomm, Oct. 2022.

[5] R4-2217248, WF on TCI state switching for multi-RX chain DL reception, Ericsson, Oct. 2022.

[6] R4-2220742, WF on NR FR2 multi-Rx chain DL reception – Part 1, Nov. 2022.

[7] R4-2220743, WF on NR FR2 multi-Rx chain DL reception – Part 2, Nov. 2022.

[8] R4-2220402, WF on NR FR2 multi-Rx chain DL reception – Part 3, Nov. 2022.

[9] R4-2303193, Ad-hoc minutes for NR FR2 multi-Rx chain DL reception RRM, vivo, March 2023.

[10] R4-2303302, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, March 2023.

[11] R4-2303317, WF on NR FR2 multi-Rx chain DL reception RRM requirements(part2), Qualcomm, March 2023.

[12] R4-2306318, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, Apr. 2023.

[13] R4- 2306319, WF on FR2_multiRx_part2, Ericsson, Apr. 2023.

[14] R4-2306392, Topic summary for [106bis-e][202] FR2_multiRx_part1, vivo, Apr. 2023.

[15] R4-2306234, Topic summary for [106bis-e][203] FR2_multiRx_part2, Ericsson, Apr. 2023.

[16] R4-2310046, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, May 2023.

[17] R4-2310047, WF on FR2_multiRx_part2, Ericsson, May 2023.

[18] R4-2309952, Topic summary for [106][209] FR2_multiRx _part1, vivo, May 2023.

[19] R4-2309953, Topic summary for [107][208] FR2_multiRx_part2, Ericsson, May 2023.

[20] R4-2314477, WF on NR FR2 multi-RX operation RRM requirements (part 1), vivo, Aug. 2023.

[21] R4-2314283, WF on FR2 multi-Rx chain DL reception (part2), Ericsson, Aug. 2023.

[22] R4-2317332, WF on NR FR2 multi-RX operation RRM requirements (part 1), vivo, Oct. 2023.

[23] R4-2317427, WF on FR2_multiRx_part2, Ericsson, Oct. 2023.

[24] R4-2321602, WF on core maintenance and performance part for multi-Rx WI, vivo, Ericsson, Nov. 2023.

# Annex A: Agreements on RLM, BFD, and CBD

The following was agreed for RLM in RAN4#108bis [22]:

- Issue 2-2-1: Updated conditions for TRP specific BFD requirements enhancement for multi-Rx

  - Both CSI-RSs are not in any CSI-RS resource set with repetition ON

  - The two CSI-RS resources in the two sets q ̅_0,0 and q ̅_0,1 for beam failure detection [and both PDSCHs] are overlapped on the same OFDM symbol.

  - [The CSI-RS in set q ̅_0,0 has same QCL source as the active TCI state of one PDSCH, and the CSI-RS in set q ̅_0,1 has same QCL source as the active TCI state of the other PDSCH]

  - Resources of the active TCI states for the two PDCCHs, or two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

  - [FFS how to capture UE is activated with multi-Rx operation]

The following was agreed for RLM and BM in RAN4#108:

- Issue 2-2-1: TRP specific BFD requirements enhancement for multi-Rx

PTRP=1 for CSI-RS based TRP specific BFD requirements for multi-Rx operation under following conditions,

  - Both CSI-RS are not in any CSI-RS resource set with repetition ON

  - UE is activated with multi-Rx operation

  - Resources of the active TCI states for the two PDCCHs or PDSCHs and one PDCCH and PDSCH have been reported via GBBR in a pair

  - Each CSI-RS has same QCL source as the active TCI state of the PDCCH/PDSCH [from the same TRP]

  - The two CSI-RS resources in the two sets q ̅_0,0 and q ̅_0,1for beam failure detection is transmitted through different [TRPs or QCL sources] at the same time

  - [Both of the CSI-RSs and both of the PDCCH/PDSCHs are transmitted at the same time]

  - Note: additional conditions are not precluded

The following was agreed for RLM and BM in RAN4#107:

- Issue 2-1-1: Beam sweeping factor for cell specific RLM and BFD/CBD for multi-Rx

  - Faster beam sweeping is not applicable for CSI-RS based RLM and BFD measurements, due to N=1 in the existing requirements.

RAN4#106bis-e:

- Cell specific RLM and BFD/CBD

  - No further discussion on TRP specific RLM as there is no such procedure in existing specification..

- Issue 2-1-2: Number of samples

  - For RLM with multi-Rx chain operation, the same number of samples Mout and Min are used as in Rel-17.

  - For link recovery with multi-Rx chain operation, the same number of samples MBFD and MCBD are used as in Rel-17.

- Issue 2-1-3: PDCCH configuration

  - For RLM with multi-Rx chain operation, the same PDCCH transmission configuration is used as in Rel-17.

  - For link recovery with multi-Rx chain operation, the same PDCCH transmission configuration is used as in Rel-17.

RAN4#106:

- Baseline L1 measurement requirements for potential multi-Rx enhancement

  - For cell specific RLM: section 8.1

  - For cell specific link recovery: section 8.5

  - For TRP specific link recovery: section 8.18

  - For L1-RSRP measurement: section 9.5.

RAN4#105:

- Overall handling of L1 measurements

  - Discussion on Rx beam sweeping scaling factor, simultaneous reception of DL data and RS, and other multi-Rx related topics can be done jointly for L1-RSRP, RLM, L1-SINR and CBD/BFD.

  - Note: issues specific to each of the above measurements can be discussed separately.