3GPP TSG-RAN WG4 Meeting #110		R4-2402928

# Athens, Greece, 26th February 2024 – 1st March 2024

Title:	Reply LS on the system parameters for NTN above 10 GHz

Response to:	LS R1-2312553 System parameters for NTN above 10 GHz from RAN1

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	3GPP RAN4

To:	RAN1

Cc:

Contact person:	Dorin Panaitopol

dorin.panaitopol@thalesgroup.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

3GPP RAN4 would like to thank RAN1 for the LS (R1-2312553) on clarifications for Non-Terrestrial Networks in above 10 GHz, asking RAN4 to provide the timing accuracy requirements for supporting NR over NTN in bands defined by FR2-NTN.

RAN4 would like to inform RAN1 with respect to the following agreement at RAN4#109:

  - UL 60kHz SCS:

    - 13 Ts for all cases with 120kHz/240kHz SSB

  - UL 120kHz SCS:

    - Case 1 and case 2: 7.5 Ts

      - FFS for the applicable side condition on case 2

    - Case 3: Higher than 7.5 Ts, FFS for the exact value

RAN4#109 has therefore discussed independently the following cases, and reached the following conclusions:

| Cases | SCS of SSB | Te_NTN [Ts] for 60 kHz of UL SCS | Te_NTN [Ts] for 120 kHz of UL SCS |
| --- | --- | --- | --- |
| Case-1: Stationary UE for GSO | 120kHz | 13 Ts | 7.5 Ts |
|  | 240kHz |  |  |
| Case-2: Stationary UE for LEO | 120kHz | 13 Ts | 7.5 Ts |
|  | 240kHz |  |  |
| Case-3: Mobile UE for GSO | 120kHz | 13 Ts | Higher than 7.5 Ts |
|  | 240kHz |  |  |

For more information, RAN1 is kindly requested to check the big draft CR R4-2321642 for RRM TS 38.133 endorsed at RAN4#109 with the following included information:

Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Editor’s Note: FFS on the applicable side condition for fixed VSAT is served by NGSO

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | [Higher value than 7.5]*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | [Higher value than 7.5]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Editor’s Note: FFS on the definition or how to differentiate fixed VSAT and mobile VSAT

Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq_NTN | Tp_NTN |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | N/A | N/A |
| [FR2-NTN] | 60 | 2.5*Tc | 2.5*Tc |
|  | 120 | 2.5*Tc | 2.5*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.3C.2.2-2: UE Timing Advance adjustment accuracy for VSAT UE in FR2-NTN

| UL Sub Carrier Spacing(kHz) | 60 | 120 |
| --- | --- | --- |
| UE Timing Advance adjustment accuracy | 128 Tc | 32 Tc |
| NOTE 1: VSAT UE are defined in TS 38.xxx |  |  |

# 2	Actions

To: RAN1

ACTION: 	RAN4 respectfully asks TSG RAN1 to take the above conclusions into account.

# 3	Dates of next TSG RAN WG 4 meetings

RAN WG4 Meeting #110-bis	April 15 – April 19, 2024 			Changsha, China

RAN WG4 Meeting #111		May 20 – May 24, 2024 			Fukuoka, Japan