3GPP TSG-RAN4 Meeting #110		R4-2403337

# Athens, Greece, 26 Feb – 1 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4196 | rev | 1 | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_redcap-Perf) Correction of requirements and parameters for RedCap testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Perf |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Ês/Noc and Ês/Ioot are incorrect for Inter-RAT Measurement test cases - In test case A.17.6.2.1 the legacy requirements in 9.3.4 are used instead of RedCap 9.3B.4. - In test case A.17.6.2.3, the wording used in test requirements may suggest that there is more than 1 sub-test. - In test A.17.6.2.4, two sub-test for 2 different DRX configurations are specified. However, sub-test 2 is missing in the test requirements sub-clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Ês/Noc and Ês/Ioot are corrected to match the legacy test cases. - The requirements pointer is corrected to 9.3B.4 in A.17.6.2.1. - Editorial correction in A.17.6.2.3. - The sub-test 2 has been specified in the test requirements sub-clause of A.17.6.2.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases might not be correctly implemented by RAN5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.17.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE 1----------------------------

### A.16.6.3	Inter-RAT Measurements

#### A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE

##### A.16.6.3.1.1	Test purpose and Environment

The purpose of this set of tests is to verify that the 1 Rx redcap UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in Clauses 9.4A.2 and 9.4A.3.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively.  Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1 and has detected Cell 2 at least for the 3.84 seconds. Cell 2 becomes undetectable during T1, and becomes detectable again during T2.

Supported test configurations are shown in table A.16.6.3.1.1-1. General test parameters are provided in Table A.16.6.3.1.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.16.6.3.1.1-3 and A.16.6.3.1.1-4, respectively.

Table A.16.6.3.1.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.16.6.3.1.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 1 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.16.6.3.1.1-3 and A.16.6.3.1.1-4. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 1 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 3 |  |
| Note 1: Values are defined in Table A.16.6.3.1.1-3 |  |  |  |

Table A.16.6.3.1.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 3 | FDD |  |
|  |  |  |  | 2, 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52(TDD) |  |
|  |  |  |  | 3, 6 | 20: NRB,c = 51 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 RedCap FR1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.1 RedCap FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | TDL-C 300ns 100Hz |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x1 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.16.6.3.1.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -76.22+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x1 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.16.6.3.1.2	Test Requirements

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 0.96s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE

##### A.16.6.3.2.1	Test purpose and Environment

The purpose of this set of tests is to verify that the 2 Rx redcap UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in Clauses 9.4A.2 and 9.4A.3.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1 and has detected Cell 2 at least for the 3.84 seconds. Cell 2 becomes undetectable during T1, and becomes detectable again during T2.

Supported test configurations are shown in table A.16.6.3.2.1-1. General test parameters are provided in Table A.16.6.3.2.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.16.6.3.2.1-3 and A.16.6.3.2.1-4, respectively.

Table A.16.6.3.2.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1 for 1 Rx UE

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.16.6.3.2.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 1 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.16.6.3.2.1-3 and A.16.6.3.2.1-4. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 1 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 3 |  |
| Note 1: Values are defined in Table A.16.6.3.2.1-3 |  |  |  |

Table A.16.6.3.1.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 3 | FDD |  |
|  |  |  |  | 2, 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52(TDD) |  |
|  |  |  |  | 3, 6 | 20: NRB,c = 51 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 RedCap FR1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.1 RedCap FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | TDL-C 300ns 100Hz |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.16.6.3.2.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -76.22+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.16.6.3.2.2	Test Requirements

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 0.48s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE

##### A.16.6.3.3.1	Test purpose and Environment

The purpose of this set of tests is to verify that the 1 Rx redcap UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used. This test shall partly verify the cell search and measurement requirements in Clauses 9.4A.2 and 9.4A.3. There are two test cases. In test 1 the UE shall be configured with DRX cycle of 40 ms. In test 2 the UE shall be configured with DRX cycle of 640 ms.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1 and has detected Cell 2 at least for the 3.84 seconds for test 1 or 12.8 seconds for test 2. Cell 2 becomes undetectable during T1, and becomes detectable again during T2.

In each test the UE shall be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore the UE shall be allocated with PUSCH resource at every DRX cycle.

Supported test configurations are shown in table A.16.6.3.3.1-1. General test parameters are provided in Table A.16.6.3.3.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.16.6.3.3.1-3 and A.16.6.3.3.1-4, respectively.

Table A.16.6.3.3.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.16.6.3.3.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | 1 |  | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.16.6.3.3.1-3 and A.16.6.3.3.1-4. |  |  |
| Active cell |  | Cell 1 |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP |  | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP |  | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 |  | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | DRX.1 | DRX.7 | DRX cycle configurations DRX.1 and DRX. 7 are defined in Table A.3.3.1-1 and Table A.3.3.7-1 respectively. |
| T1 | s | 5 |  |  |
| T2 | s | 3 | 10 |  |
| Note 1: Values are defined in Table A.16.6.3.3.1-3 |  |  |  |  |

Table A.16.6.3.3.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 3 | FDD |  |
|  |  |  |  | 2, 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52(TDD) |  |
|  |  |  |  | 3, 6 | 20: NRB,c = 51 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 RedCap FR1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.1 RedCap FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | TDL-C 300ns 100Hz |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x1 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.16.6.3.3.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -76.22+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x1 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.16.6.3.3.2	Test Requirements

In test 1, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 0.96s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

In test 2, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 6.4s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE

##### A.16.6.3.4.1	Test purpose and Environment

The purpose of this set of tests is to verify that the 2 Rx redcap UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used. This test shall partly verify the cell search and measurement requirements in Clauses 9.4A.2 and 9.4A.3. There are two test cases. In test 1 the UE shall be configured with DRX cycle of 40 ms. In test 2 the UE shall be configured with DRX cycle of 640 ms.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1 and has detected Cell 2 at least for the 3.84 seconds for test 1 or 12.8 seconds for test 2. Cell 2 becomes undetectable during T1, and becomes detectable again during T2.

In each test the UE shall be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore the UE shall be allocated with PUSCH resource at every DRX cycle.

Supported test configurations are shown in table A.16.6.3.4.1-1. General test parameters are provided in Table A.16.6.3.4.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.16.6.3.4.1-3 and A.16.6.3.4.1-4, respectively.

Table A.16.6.3.4.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.16.6.3.4.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | 1 |  | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 1 |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.16.6.3.4.1-3 and A.16.6.3.4.1-4. |  |  |
| Active cell |  | Cell 1 |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP |  | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP |  | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 |  | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | DRX.1 | DRX.7 | DRX cycle configurations DRX.1 and DRX. 7 are defined in Table A.3.3.1-1 and Table A.3.3.7-1 respectively. |
| T1 | s | 5 |  |  |
| T2 | s | 3 | 5 |  |
| Note 1: Values are defined in Table A.16.6.3.4.1-3 |  |  |  |  |

Table A.16.6.3.4.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 3 | FDD |  |
|  |  |  |  | 2, 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52(TDD) |  |
|  |  |  |  | 3, 6 | 20: NRB,c = 51 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 RedCap FR1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.1 RedCap FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 |
|  |  |  |  | 3, 6 | -85 | -101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | TDL-C 300ns 100Hz |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.16.6.3.4.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -76.22+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.16.6.3.4.2	Test Requirements

In test 1, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 0.48s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

In test 2, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.2s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

----------------------END OF CHANGE 1----------------------------

----------------------START OF CHANGE 2----------------------------

### A.17.6.2	Inter-frequency Measurements

#### A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)

##### A.17.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3B.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.17.6.2.1.1-1, A.17.6.2.1.1-2, and A.17.6.2.1.1-3.

Measurement gap pattern configuration # 13 as defined in Table A.17.6.2.1.1-2 is provided for UE that does not support per-FR gap and for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.17.6.2.1.1-1.

Table A.17.6.2.1.1-1 SA event triggered reporting tests without SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.17.6.2.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-CD-SSB parameters |  | Config 1 | SSB.2 RedCap FR2 | As specified in clause A.3.10.2 |
| NCD-SSB parameters |  | Config 1 | SSB.3 RedCap FR2 |  |
| offsetMO | dB | Config 1 | 16 | Applied to NR Cell 2 measurement object |
| A3-Offset | dB | Config 1 | -11 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 5.2 for PC1; 3.5 for other PC |  |

Table A.17.6.2.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.3.15 |  |  |  |  |
|  |  |  |  | AoA1 |  | AoA2 |  |  |
| Beam AssumptionNote 7 |  |  | 1,2 | Rough |  | Rough |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  |
|  | Dedicated DL BWP configuration |  |  | DLBWP.1.3 RedCap Note 9 |  | N/A |  |  |
|  | Dedicated UL BWP configuration |  |  | ULBWP.1.3 RedCap Note 10 |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 for CD-SSB |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |  |
| SMTC configuration for NCD-SSB |  |  | Config 1 | SMTC.2 RedCap |  | SMTC.2 RedCap |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -87 | -87 | -Infinity |  | -87 |
| SSBRP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 | -Infinity |  | -87 |
| BB Note 8 |  | dB | Config 1 | 1.89 | 1.89 | -Infinity |  | 1.89 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 | -Infinity |  | -58.01 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: VoidNote 3: SSBRP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. Note 9: The starting PRB index for dedicated DL BWP1 corresponding to CD-SSB PRB index; the starting PRB index for dediacted DL BWP2 corresponding to NCD-SSB PRB index; Note 10: The starting PRB index for dedicated UL BWP1 is the same as the starting PRB index for dedicated DL BWP1; the starting PRB index for dedicated UL BWP2 is the same as the starting PRB index for dedicated DL BWP2. |  |  |  |  |  |  |  |  |

##### A.17.6.2.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

The  UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)

##### A.17.6.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that RedCap UE makes correct reporting of an event in FR2. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3B.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2. The test parameters and configurations are given in Tables A.17.6.2.2.1-1, A.17.6.2.2.1-2, and A.17.6.2.2.1-3.

In test 1&2 measurement gap pattern configuration # 13 as defined in Table A.17.6.2.2.1-2 is provided for RedCap UE that does not support per-FR gap and for RedCap UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.17.6.2.2.1-1.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.17.6.2.2.1-1: SA event triggered reporting tests without SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.17.6.2.2.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC |  |

Table A.17.6.2.2.1-3: Cell specific test parameters for CA inter-frequency event triggered reporting without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -104.7 |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -59.7 | -59.7 | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | Config 1 | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.17.6.2.2.2	Test Requirements

In test 1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

7680 for UE supporting power class 1, or

4800 for UE supporting other power class.

In test 2  the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

81920 for UE supporting power class 1, or

51200 for UE supporting other power class.

In test 1 and 2 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)

##### A.17.6.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3B.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.17.6.2.3.1-1, A.17.6.2.3.1-2, and A.17.6.2.3.1-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.17.6.2.3.1-1.

Table A.17.6.2.3.1-1: SA event triggered reporting tests with SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.17.6.2.3.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1 | 16 | Applied to NR Cell 2 measurement object |
| A3-Offset | dB | Config 1 | -11 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 7 for PC1; 4.5 for other PC |  |

Table A.17.6.2.3.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.3.15 |  |  |  |  |
|  |  |  |  | AoA1 |  | AoA2 |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1 | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -87 | -87 | -Infinity |  | -87 |
| SSBRP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 | -Infinity |  | -87 |
| BB Note 8 |  | dB | Config 1 | 1.89 | 1.89 | -Infinity |  | 1.89 |
| Io Note3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 | -Infinity |  | -58.01 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SBRP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

##### A.17.6.2.3.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

6720 for UE supporting power class 1, or

4160 for UE supporting other power class.

The UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE

##### A.17.6.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3B.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.17.6.2.4.1-1, A.17.6.2.4.1-2, and A.17.6.2.4.1-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.17.6.2.4.1-1.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.17.6.2.4.1-1: SA event triggered reporting tests with SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.17.6.2.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC |  |

Table A.17.6.2.4.1-3: Cell specific test parameters for CA inter-frequency event triggered reporting with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -104.7 |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -59.7 | -59.7 | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.17.6.2.4.2	Test Requirements

In test 1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

10080 for UE supporting power class 1, or

6240 for UE supporting other power class.

In test 1 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

107520 for UE supporting power class 1, or

66560 for UE supporting other power class.

In test 1 and 2 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

----------------------END OF CHANGE 2----------------------------