3GPP TSG-RAN WG4 Meeting #110bis	R4-2404382

# Changsha, China, April 15-19, 2024

Source:	T-Mobile USA

Title:	TP for TR38.850: FDD PC2 for n25 with DL CA_n25A-n41A

Agenda item:	5.18.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 n25 for DL CA_n25A-n41A.

There is no MSD for PC3 CA_n25A-n41A but At RAN4#110 it was suggested that there might be UL4/DL3 Harmonic Mixing MSD and cross-band isolation MSD for n25 PC2. This contribution considers Harmonic Mixing MSD and cross-band isolation MSD for CA_n25A-n41A with UL n25 PC2.

- Discussion

For PC3, in similar frequency range, CA_n39A-n41A with n39 PC3 has the following. Note, square brackets were added for the MSD at RAN4#110:

| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | [8.1] | NOTE 11 | UL4/DL3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | [3.3] | NOTE 11 | UL4/DL3 |

Since there is usually MB-HB diplexing, and the filtering margin along with the PA TX filter response should be enough to mitigate the effect of power class 2 for UL4. Because there is no MSD for PC3, we propose 0.1 dB MSD PC2.

For cross-band isolation MSD, we will assume 0.2 dB MSD for PC3.

- Reference

[1]

- Text Proposal

<Start of text proposal>

## 5.x	CA_ n25A-n41A

### 5.x.1	UE maximum output power

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration |  | Uplink CA configuration or single uplink carrier10 |  | NR Band |  | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A |  | n418,9 n25A8 CA_n25A-n41A8, 13,14 |  | n25 |  | 5, 10, 15, 20 |  | 0 |  |
|  |  |  |  | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n25 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |  |
|  |  |  |  |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  |  |  |  |  | n25 |  | See n25 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  |  |  |  | n41 |  | See n41 channel bandwidths in Table 5.3.5-1 |  |  |  |

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

### 5.x.2	Harmonic Mixing Reference sensitivity requirements

#### 5.x.2.0	General

For PC3, CA_ n25A-n41A has no harmonic MSD for UL n25. This section will examine the existing PC3 MSD and propose MSD for PC2 FDD.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n25 without TxD

For CA_n25-n41, there is no MSD for UL n25 with PC3. For n25 PC2, UL4/DL3 harmonic mixing MSD is considered.

In a similar frequency range, CA_n39A-n41A has harmonic mixing MSD for PC3, although in square brackets:

| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | [8.1] | NOTE 11 | UL4/DL3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | [3.3] | NOTE 11 | UL4/DL3 |

Since n39 is a TDD band and n25 is an FDD band, there will be attenuation of the n25 uplink 4th harmonic, so the impact should be considerably less than for n39. Also, because there is usually MB-HB diplexing, and the filtering margin along with the TX filter response should be enough to mitigate the effect of power class for UL4.

We propose 0.1 dB MSD for CA_n25A-n41A with UL n25 PC2.

Table 7.3A.4-4a: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n25 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | 0.1 | NOTE 11 | UL4/DL3 |
| n25 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 0.1 | NOTE 11 | UL4/DL3 |

#### 5.x.2.2	Harmonic Mixing eference sensitivity requirements with PC2 on n25 with TxD

[TBD].

### 5.x.3	Cross-band isolation reference sensitivity requirements

#### 5.x.3.0	General

For PC3, CA_ n25A-n41A has no cross band isolation MSD for UL n25. This section proposes MSD for PC2 FDD.

#### 5.x.3.1	Reference sensitivity requirements with PC2 on n25 without TxD

For CA_ n25A-n41A, this would be the configuration and MSD for UL n25 with PC3

Table 7.3A.6-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n25 | n41 | 1895 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 0.2 | >ACLR2 |

For PC3 MSD we have N+IPC3. For PC2 MSD we have N+IPC2. So, for PC2 compared to PC3, I increases by 3 dB. For our other PC2 and PC1.5 MSD analysis we have been using the following approach:

MSD due to interference power I is given by:

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have:

If I is increased by X dB, then MSD(X) is given by

Using that approach, the following is proposed as a the PC2 MSD:

Table 7.3A.6-1a: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n25 | n41 | 1760 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 0.4 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |

#### 5.x.3.2	Cross-band isolation reference sensitivity requirements with PC2 on n25 with TxD

[TBD]