3GPP TSG-RAN WG4 Meeting # 110bis	R4-2405160

# Changsha, China, April 15th – 19th , 2024

Title: 	Discussions on UE performance requirements for Rel-18 eRedCap

Source: 	Huawei, HiSilicon

Agenda item:	6.21.3.1

Document for:	Discussion

Background

In last meeting, WF on [110][125] NR_redcap_enh_demod was agreed, in the contribution we provide our views on open issues .

- Discussions

FFS whether to define PDSCH demodulation requirements with 64QAM/256QAM.

One issue is whether to introduce 64QAM and 256QAM requirements for UE supporting or not supporting eRedCapNotReducedBB-BW-r18, our understanding is that higher modulation is an important feature which is widely used in the real deployment. Meanwhile, eRedCap UE is a reduced capability UE with smaller buffer size, but there is no limitation for supported modulation order. Therefore, same as logic of requirements definition for legacy UE type, QPSK/16QAM is not adequate and it’s necessary to test higher modulation order.

Proposal 1: RAN4 to introduce 64QAM and 256QAM requirements for UE supporting or not supporting eRedCapNotReducedBB-BW-r18

Same as Rel-17 RedCap requirements definition, higher MIMO layers should be verified for 2Rx which can be selected with 64QAM since Rank2+256QAM will exceed the capability.

We propose the tests listed in Table 2-1 for UE supporting or not supporting eRedCapNotReducedBB-BW-r18

Proposal 2: Use tests listed in Table 2-1 for UE supporting or not supporting eRedCapNotReducedBB-BW-r18

Table 2-1: Proposed test cases for UE supporting or not support eRedCapNotReducedBB-BW-r18

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64QAM 1Rx | 1Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 25 | 9992 | TDLA30-10 |
| 64QAM 2Rx | 2Rx | MCS19 in Table 1 | 0.50 | Rank 2 | 12 | 9480 | TDLA30-10 |
| 256QAM 1Rx | 1Rx | MCS20 in Table 2 | 0.67 | Rank 1 | 7 | 4864 | TDLA30-10 |
| 256QAM 2Rx | 2Rx | MCS24 in Table 2 | 0.82 | Rank 1 | 5 | 4352 | TDLA30-10 |

CSI requirements

For CSI requirements, it’s agreed to reuse Rel-17 test setup, which means full RB allocation is configured for CSI-RS, however, it’s also agreed to use partial allocation for PDSCH (For CQI test, 15RBs for FDD and 7RBs for TDD). This configuration will lead to the situation that the CSI derived from CSI-RS can’t reflect the channel characteristics of PDSCH. Our understanding is that it is better to reduce the RB allocation difference between PDSCH and CSI-RS to avoid performance degradation.  Considering the limitation that the smallest CSI-RS RB size is min(24, BWP size), and it should be multiple of 4. We prefer to reduce the CSI-RS PRB size to the values in Table 2-2 given the BWP size is 51PRBs for TDD and 52PRBs for FDD:

Table 2-2: Proposed CSI-RS configuration.

|  | CQI test (static/fading) | PMI test |
| --- | --- | --- |
| FDD | BWP:52 RBs CSI-RS: 24RBs PDSCH:15 RBs | BWP:52 RBs CSI-RS: 28RBs PDSCH: 25RBs |
| TDD | BWP:52 RBs CSI-RS: 24RBs PDSCH:7RBs | BWP:52 RBs CSI-RS: 24RBs PDSCH: 15RBs |

Proposal 3: Use CSI-RS configuration in Table 2-2 for CSI test

- Conclusion

In this contribution we provide our views on open issues for Rel-18 eRedCap. The observations and proposals are:

Proposal 1: RAN4 to introduce 64QAM and 256QAM requirements for UE supporting or not supporting eRedCapNotReducedBB-BW-r18

Proposal 2: Use tests listed in Table 2-1 for UE supporting or not supporting eRedCapNotReducedBB-BW-r18

Proposal 3: Use CSI-RS configuration in Table 2-2 for CSI test

- Reference

[1]    RP-2402870 Way forward on [110][325] NR_redcap_enh_demod. Ericsson