# 3GPP TSG-RAN WG4 Meeting # 110bis                                                     R4-2405353

# Changsha, China, 15-19 April , 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS38.101-3 addition of the missing NE-DC requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R18_xBLTE_yBNR_zDL2UL-Core |  |  |  |  |  |  | Date: |  | 2024-04-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NE-DC band combinations have been added to the specifications, but some of their requirements are not yet specified. With this draftCR we plan to add those requirements. NOTE: This draftCR is not for block approval |  |  |  |  |  |  |  |  |
| Summary of change: |  | addition of the missing NE-DC requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NE-DC band combinations cannot be implemented or tested properly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B, 7.3B, 7.6B, 7.7B,7.8B, 7.9B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

## << Start of change >>

#### 6.2B.1.5a	Inter-band NE-DC including both FR1 and FR2

UE maximum output power requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.2 and 6.2.2A of TS 36.101 [4] and for NR single carrier specified in clause 6.2.1 of TS 38.101-1 [2] and for NR single carrier, CA operation and UL-MIMO specified in clause 6.2.1, 6.2A.1, and 6.2D.1 of TS 38.101-2 [3] apply. When uplink is NE-DC mode within FR1 only then UE maximum output power requirement is specified in clause 6.2B.1.3a of this specification.

## << Next change >>

#### 6.2B.2.5a	Inter-band NE-DC including both FR1 and FR2

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.3 and 6.2.3A of TS 36.101 [4] and for NR single carrier specified in clause 6.2.2 of TS 38.101-1 [2] and for NR single carrier, CA operation and UL-MIMO specified in clause 6.2.2, 6.2A.2 , and 6.2D.2 of TS 38.101-2 [3] apply.

## << Next change >>

### 6.2B.3	UE additional maximum output power reduction for EN-DC and NE-DC

## << Next change >>

#### 6.2B.3.5a	Inter-band NE-DC including both FR1 and FR2

UE additional maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.4 and 6.2.4A of TS 36.101 [4] and for NR single carrier specified in clause 6.2.3 of TS 38.101-1 [2] and for NR single carrier, CA operation and UL-MIMO specified in clause 6.2.3, 6.2A.3 and 6.2D.3 of TS 38.101-2 [3] apply.

## << Next change >>

##### 6.2B.4.1.5a	Inter-band NE-DC including both FR1 and FR2

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, with both CGs configured in FR1, the requirements specified in clause 6.2B.4.1.3a apply.

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, with NR configured in FR2, the requirements specified in clause 6.2B.4.1.4a apply.

For inter-band dual connectivity with one uplink serving cell in first CG on E-UTRA and two uplink serving cells in second CG on NR FR1 and NR FR2 respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) , i = 1,2,3 with i=1 for E-UTRA, i=2 for NR FR1 and i=3 for NR FR2.

–	For serving cell on FR2, the requirements specified in clause 6.2.4 in TS 38.101-2 [3] apply to the UE maximum configured power PCMAX,c(3),3 and the measured maximum configured power.

–	For remaining inter-band dual connectivity involving CG1 and CG2, the requirements specified in clause 6.2B.4.1.3a apply.

## << Next change >>

##### 6.2B.4.2.5a	Inter-band NE-DC including both FR1 and FR2

###### 6.2B.4.2.5a.1	ΔTIB,c for NE-DC three bands

Unless otherwise stated, for inter-band NE-DC configurations defined in table 5.5B.6a.2-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band NE-DC configuration without the FR2 bands specified in 6.2B.4.2.3a.

###### 6.2B.4.2.5a.2	ΔTIB,c for NE-DC four bands

Unless otherwise stated, for inter-band NE-DC configurations defined in table 5.5B.6a.3-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band NE-DC configuration without the FR2 bands specified in 6.2B.4.2.3a.

###### 6.2B.4.2.5a.3	ΔTIB,c for NE-DC five bands

Unless otherwise stated, for inter-band NE-DC configurations defined in table 5.5B.6a.4-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands   is the same as those for the corresponding inter band NE-DC configuration without the FR2 bands specified in 6.2B.4.2.3a.

###### 6.2B.4.2.5a.4	ΔTIB,c for NE-DC six bands

Unless otherwise stated, for inter-band NE-DC configurations defined in table 5.5B.6a.5-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band NE-DC configuration without the FR2 bands specified in 6.2B.4.2.3a.

## << Next change >>

#### 7.3B.2.4a	Inter-band NE-DC including FR2

##### 7.3B.2.4a.1	Void

## << Next change >>

#### 7.3B.2.5a	Inter-band NE-DC including both FR1 and FR2

##### 7.3B.2.5a.1	Reference sensitivity exceptions due to UL harmonic interference for NE-DC including both FR1 and FR2

For inter-band NE-DC of E-UTRA and NR in both FR1 and FR2, the UE is allowed to apply each sensitivity degradation for NE-DC in FR1 specified in clause 7.3B.2.3a TS 38.101-3 and for NE-DC including FR2 specified in clause 7.3B.2.3a of TS 38.101-3 independently.

## << Next change >>

#### 7.3B.3.1a	Intra-band contiguous NE-DC within FR1

Unless ΔRIB,c is specified in this clause, the value of ΔRIB,c for the correspondingly specified EN-DC configuration in clause 7.3B.3.1 is applicable.

## << Next change >>

#### 7.3B.3.5a	Inter-band NE-DC including both FR1 and FR2

Unless ΔRIB,c is specified in this clause, the value of ΔRIB,c for the correspondingly specified EN-DC configuration in clause 7.3B.3.5 is applicable.

## << Next change >>

#### 7.6B.2.5a	Inter-band NE-DC including both FR1 and FR2

In-band blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.1.1 and 7.6.1.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.2, 7.6A.2 and 7.6B.2 of TS 38.101-1 [2] and TS 38.101-2 [3] apply.

## << Next change >>

#### 7.6B.3.5a	Inter-band NE-DC including both FR1 and FR2

Out-of-band blocking requirements specified for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] apply for lowest level NE-DC fall-backs (three bands) in clause 5.5B.6a.2 with only E-UTRA UL with output power as in TS 36.101 [4] (4 dB below PCMAX_L).

## << Next change >>

#### 7.6B.4.5a	Inter-band NE-DC including both FR1 and FR2

Narrow band blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.3.1 and 7.6.3.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.4 and 7.6A.4 of TS 38.101-1 [2] apply.

## << Next change >>

### 7.7B.5a	Inter-band NE-DC including both FR1 and FR2

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.7 and 7.7A of TS 38.101-1 [2] apply for lowest level NE-DC fall-backs (three bands) in clause 5.5B.6a.2 with only E-UTRA UL with output power as in TS 36.101 [4] (4 dB below PCMAX_L).

## << Next change >>

#### 7.8B.2.5a	Inter-band NE-DC including both FR1 and FR2

Wide band Intermodulation requirement for E-UTRA single carrier and CA operation specified in clauses 7.8.1 and 7.8.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.8.2 and 7.8A.2 of TS 38.101-1 [2] apply.

## << Next change >>

### 7.9B.5a	Inter-band NE-DC including both FR1 and FR2

Spurious emissions requirement for E-UTRA single carrier and CA operation specified in clauses 7.9.1 and 7.9.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.9 and 7.9A of TS 38.101-1 [2] and TS 38.101-2 [3] apply.

## << End change >>